TSG-RAN WG1 #99	R1-1913346

Reno, NV, USA, November 18 – 22, 2019

Source:	Ericsson

Title:	Outcome of offline discussion on remaining open issues on 7.1.3 resource allocation and DCI content

Agenda Item:	7.1.3

Document for:	Discussion and Decision

# Slot configuration

The draft CR in R1-1913125 was discussed during the online session today with the conclusion

R1-1913125	Correction on slot configurations	Huawei, HiSilicon

Offline discussion on how to clarify the case of partial overlapping

In the draft CR, three approaches were listed on the cover page and option 1 used as the basis for the draft CR. However, this option changes the technical behavior of existing UEs are is therefore not acceptable. Option 2 is in essence rate matching or puncturing to handle the colliding symbols and option 3 is that the UE drops the whole channel when collations occur.

Is the following text suitable as a basis for updating 38.213 (instead of the text in R1-1913125)?

For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with in the set of symbols of the slot.

For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with in the set of symbols of the slot.

Outcome: Prepare a draft for the alignment CR based on the TP above.

# N_info

The current specification text in 38.214, clause 5.1.3.2 states

2)	Intermediate number of information bits (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

From the equation it is clear that N_info is a floating point variable, while the text preceding the equation can be read as N_info being an integer. This was discussed online with the conclusion

R1-1913153	Discussion on Ninfo for TBS determination	Huawei, HiSilicon

Continue discussion offline – particularly w.r.t. whether N_info is limited to an integer (in this case, using a floor(.) operation) or not.

Hence, we need to pick one of the two alternatives

- N_info is a floating point number. Revise the specification text to

Intermediate number of information bits (Ninfo) TBStemp) is obtained by The quantityTBStemp= NRE*R*Qm*ʋ.

- N_info is an integer. Revise the specification text to

Intermediate number of information bits (Ninfo) TBStemp) is obtained by Ninfo=NRERQmυTBStemp= NRE*R*Qm*ʋ.

Which of the two alternatives should we select as the basis for updating 38.214?TBStemp= NRE*R*Qm*ʋ

Outcome: Alt 1 to be adopted for the 38.214 alignment CR, clause 5.1.3.2.