3GPP TSG RAN WG1 #107-e		R1-2111061

e-Meeting, November 11th – 19th, 2021

Title:	Support of 2 Tx codebook configuration to 4Tx capable UE in UL

Agenda Item:  8.17

Source:       vivo, ZTE, CMCC, Samsung, Qualcomm

Document for: Discussion and Decision

- Discussion

Rel-15 NR specified 4Tx UL MIMO transmission while supporting various UE implementations. Depending on hardware implementation, UE Tx chains could be fully-coherent, partially-coherent or non-coherent and hence corresponding codebook subsets are specified. Rel-15 also supports coherent or non-coherent codebook subsets for 2Tx UL MIMO corresponding to coherent or non-coherent Tx chains capability. If an UE is capable of coherent 4Tx chains supporting coherent codebook subset for 4Tx UL MIMO, it can be straight forward to assume that the same UE can support coherent codebook subset for 2Tx UL MIMO from 2 out of 4 antennas; similarly UE supporting non-coherent 4Tx codebook subset can support non-coherent 2Tx codebook subset. However, for an UE capable of partial-coherent 4 Tx chains, it could support either coherent 2Tx codebook subset with 2 coherent Tx chains or non-coherent 2Tx codebook subset with 2 non-coherent Tx chains.

In 38.214, following is specified,

“A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent'.”

The intention of the above statement is to prevent gNB configuring 4Tx full-coherent codebook subset to an UE capable of 4Tx partial-coherent chains. It is not clear from the current spec whether 2Tx coherent codebook subset can be configured for an UE supporting 4Tx partial codebook subset.

Motivation and proposals:

gNB may configure 2-port SRS for an UE supporting 4Tx in UL for different reasons; it could be for UE power saving purpose, gNB may configure fewer number of SRS ports than max number of ports UE supported in different BWPs, or it could be due to overall SRS overhead in the cell.

For 4Tx partial-coherent UE not supporting Rel-16 UL full power transmission, if configured with 2-port SRS, maximum deliverable output power could be different with coherent codebook subset and non-coherent codebook subset. Let’s assume PC3 UE, 2Tx non-coherent codebook subset contains only antenna selection TPMIs, that means the maximum output power for rank=1 transmission is scaled by 1/4, if 2Tx coherent codebook subset can be configured then the maximum output power for rank=1 transmission with non-antenna selection TPMIs is scaled by 2/4 since there are 2 non-zero PUSCH ports, which means 3dB more power.

For example, as shown in figure 1 below, for 4Tx partial-coherent UE (with 17dBm PAs), by virtualizing 2 antennas it can operate as 2Tx coherent or non-coherent UE. If it is assumed 2Tx non-coherent UE after virtualization, due to power scaling mechanism, for rank=1 transmission the maximum output power is 1/4 of Pc_max, i.e. 17dBm for PC3 UE and if assuming 2Tx coherent UE after virtualization, the non-antenna selection TPMIs can deliver 1/2 of Pc_max, i.e. 20dBm for PC3 UE. On the other hand, if such an UE chooses two coherent antenna pair without antenna virtualization for 2Tx operation, the non-antenna selection TPMIs can also deliver 1/2 of Pc_max.

Figure 1, 4Tx partial-coherent UE operating as 2Tx UE

Hence, following proposal is made.

Proposal 1:

- For 4Tx partial-coherent capable UE, 2Tx coherent codebook subset is supported when the network configures 2-port SRS (for codebook) and SRS resource set includes 1 SRS resource or configured with same number of ports for all resources.

  - Introduce a new UE capability which is per FS

Conclusion:

In this contribution, we discussed the motivation and use case of configuring fewer number of SRS ports for an UE supporting larger number of Tx chains. For a 4Tx partial-coherent UE not supporting Rel-16 UL full power, 2 port SRS and 2Tx coherent codebook subset can be configured, hence following is proposed.

Proposal 1:

- For 4Tx partial-coherent capable UE, 2Tx coherent codebook subset is supported when the network configures 2-port SRS (for codebook) and SRS resource set includes 1 SRS resource or configured with same number of ports for all resources.

  - Introduce a new UE capability which is per FS

Annex:

TP for 38.214 section 6.1.1.1

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent', except for the case when a UE supporting 4 antenna ports is configured with 2 antenna port codebook depending on UE reported capability  'fullyAndPartialAndNonCoherent' can be configured by either codebookSubset or codebookSubsetForDCI-Format0-2.