3GPP TSG RAN WG1 #107-e                                                                                                    R1-2111204

Meeting, 11th Nov – 19th Nov., 2021

Source:               TCL Communication

Title:                    Discussion on TBoMS PUSCH

Agenda item:      8.8.1.2

Document for:    Discussion and Decision

- Introduction

In the RAN1#106bis-e meeting, the discussions were focused on a single TBoMS repetition, rate matching, time resource and TBS determination, the following agreements and working assumptions for TBoMS were achieved [1].

Agreement - For transmission power determination of TBoMS transmission in Rel-17, RAN1 to down-select one of the following two options: - Option 1: The transmission power determination of TBoMS should be based on all the REs allocated in one available slot for the TBoMS transmission, excluding the overhead of reference signals - Option 2: The transmission power determination of TBoMS should be based on all the REs allocated in the N available slots for the TBoMS transmission, excluding the overhead of reference signals. - FFS: details on BPRE Agreement The number of MIMO layers (rank) for TBoMS transmission in Rel-17 is limited to 1. Agreement For a single TBoMS transmission and TBoMS repetitions in Rel-17, at least the legacy Rel-15/16 inter-slot frequency hopping framework used in PUSCH repetition Type A is supported. - FFS: other frequency hopping schemes. Agreement - The number N of allocated slots for TBoMS is indicated via a new column added to the TDRA table configured via PUSCH-TimeDomainAllocationList. The existing column for configuring the number of repetitions in the TDRA for Rel-17 PUSCH repetition Type A, i.e., numberOfRepetitions, is used for indicating the number of repetitions M of a single TBoMS, when TBoMS transmission is enabled. - FFS: supported values of N and M. - FFS: how to enable the TBoMS transmission - FFS: details of retransmission of TBoMS Agreement For the repetition of a single TBoMS transmission, redundancy versions (RVs) are cycled across the TBoMS repetitions. The legacy Rel-15/16 RV sequences and RV index indication are reused. Conclusion Values 1<K<N for the scaling factor to calculate N_info for TBS determination for TBoMS transmission in Rel-17 are not supported. Agreement At least the following values are supported in Rel-17 for the number N of allocated slots for the single TBoMS: FFS: whether N=1 is also supported depends on how TBoMS transmission feature is enabled (or disabled) FFS: other values, if any. FFS: further constraints on N*M Agreement The following values are supported in Rel-17 for the number M of repetitions of the single TBoMS: FFS: further constraints on N*M, e.g., N*M is a valid value according to agreements in AI 8.8.1.1 Agreement BPRE for TBOMS is calculated as where N is the number of slots allocated for a single TBOMS and is the number of allocated REs in one allocated slot of a single TBOMS. Note: How this equation or its equivalent is captured in the specification is left to the editor Agreement For a single TBoMS transmission and TBoMS repetitions in Rel-17, the legacy Rel-15/16 intra-slot frequency hopping framework used in PUSCH repetition Type A is supported. - FFS: other frequency hopping schemes. Working Assumption For TBoMS in Rel-17, the following is supported: - Bit interleaving is performed per slot. - The index of the starting coded bit for each transmitted slot is predetermined prior to the start of the TBoMS transmission. - Transmission is limited to one CB only. - FFS: whether UCI multiplexing bits or cancellation/dropping of coded bits, if any, have to be known prior to the determination of the index of the starting coded bit for each transmitted slot or not - FFS: Performance with UCI multiplexing on single and multiple slots of a single TBoMS Note: How UCI multiplexing and cancellation/dropping of coded bits influence the sequence of coded bits transmitted in each slot of a single TBOMS is to be further discussed. Some knowledge on UCI to be multiplexed or cancellation/dropping of coded bits in each slot of a single TBOMS may be known prior to the start of a single TBOMS transmission. How this is to be handled is to be discussed further. Agreement For the bit selection for each transmitted slot for TBoMS, one of the following is to be down selected in RAN1 #107-e for determining the index of the starting coded bit in the circular buffer: - Option B: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot. - Option C: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot, regardless of whether UCI multiplexing occurred in the previous allocated slot or not. FFS: whether the index of the starting coded bit for each transmitted slot is expressed as a multiple integer of the lifting size Zc Note: Dropping/cancellation rules are not considered for the starting bit position determination in both Option B and Option C. Agreement For TBoMS transmission in Rel-17: - TBoMS feature is enabled (or disabled) by configuring (or not) the number of allocated slots for a single TBoMS (N) in a row of the TDRA table. - TBoMS transmission is enabled when N>1, where N is the number of allocated slots for a single TBoMS. - Single-slot PUSCH transmission is enabled when N=1. - Supported combinations of N and M that can be configured in the TDRA table, these combinations are constrained by retransmission are to be further discussed

In this contribution, we further analyze the potential enhancements and provide our views on TBoMS.

- Discussion

2.1 Handling of collision for TBoMS

In Rel-15/16, when a PUSCH transmission is overlapped with other transmission with high priority or PUCCH repetition, etc., the PUSCH transmission need to be discarded. When TBoMS is enabled, if the similar method in Rel-15/16 is reused, then the whole TBoMS transmission is discarded. However, in this way, much resources will be wasted when only part of resource within a TBoMS is collided. The dropping rules based on overlapped slot(s) could be considered.

Proposal 1: Only dropping the overlapped slot(s) should be considered for TBoMS transmission when collision happen.

2.2 Re-transmission for TBoMS

One of the issue related to TBoMS is retransmission. When TBoMS is enabled, a TB can occupy multiple slots, then each slots (or a set of slots) of the TBoMS transmit only part bits of a TB. One straightforward way is re-transmitted the whole TB of the TBoMS when the TB is error. However, in this way, it’s not friendly for using of resources when only a part of slots within the TBoMS have bad channel condition. An enhanced mechanism should be considered. In NR Rel-15/16, code block groups based (CBG-based) PUSCH transmission was specified by grouping code blocks of a TB into code block groups, each code block groups include several code blocks. The main purposes of introducing CBG based transmission mechanism which is to reduce the re-transmission resource and further improve the capacity of the CBG. Similar mechanism could be reused for Rel-17 TBoMS retransmission. Unfortunately, in last RAN 1 e-meeting, a working assumption of transmission is limited to one CB only for Rel-17 TBoMS was achieved, CBG based mechanism can’t be reused for TBoMS retransmission directly. Some modified will be needed. A slot or a set of slots retransmission for TBoMS could be supported.

Proposal 2. A slot or a set of slots retransmission for TBoMS should be supported.

2.3 Maximum TBS limitation for TBoMS

In RAN1#104-e, it has been agreed that the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers. Unfortunately, due to time limitation, the issue has been not discussed yet. Anyway, the issue should be determined. In current specification, the data rate constraint conditions is defined in clause 6.1.4 of [3]. As show below:

For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to 'enable', or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, or if it is an actual repetition for PUSCH repetition Type B, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PUSCH

-	M is the number of TB in the PUSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PUSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i)

-	each actual repetition for PUSCH repetition type B is treated as one PUSCH.

It’s has been agreed that the Ninfo  calculation for TBS determination, at least the scaling factor value K=N is supported, where N is the number of allocated slots for a single TBoMS. A simple way to limit the TBS of TBoMS is to redefine the L as the number of symbols allocated to a TBoMS within a slot.

Proposal 3: The maximum TBS for TBoMS should be limited based on DataRateCC.

2.4 FDRA for TBoMS

One of the main reason for bottleneck of PUSCH coverage is the limitation of transmission power, boosting the transmission power with the narrow bandwidth for better coverage is a feasible way. When TBoMS transmission is enabled, there is no need to occupy a lager frequency domain resources to achieve even lower code rate thanks to the increased time domain resources from multiple slots. Thus, the maximum number of PRB in the frequency domain can be limited. As a result, some bits of FDRA field in DCI can be saved.  Some companies argued that the PRBs limitation for TBoMS should be done by gNB scheduling, however, in this way, saving some bits in FDRA field will be disappeared. How to define the maximum number of PRBs, it could be further studied, e.g., depending on the number of slots for TBoMS could be considered.

Proposal 4: The maximum number of PRBs can be limited when TBoMS is enabled.

2.5 UCI multiplexing

In the current specification, when a single PUCCH and PUSCH transmissions of same priority index overlaps with each other, the UCI is multiplexed on the PUSCH in the overlapped slot and the PUCCH transmission is cancelled if the timeline condition is met. When TBoMS transmission is enabled, a TB would be occupied multiple of slots, the legacy per-slot UCI multiplexing mechanism is not applicable.

When the overlapping of PUCCH and PUSCH is realized by UE before the start of TBoMS PUSCH preparation, the UE can determine the amount of REs for the TBoMS PUSCH in consideration of UCI transmission resources when performing rate-matching, but not vice versa. UCI puncturing can be an option when the determination time is later than the start of the PUSCH preparation, as shown in Figure 1.

Figure 1: UCI multiplexing mechanism

Proposal 5: UCI multiplexing is performed by puncturing or rate-matching depending on the determination time is before or latter the starting time of PUSCH preparation.

We know the timelines are different for the cases with and without multiplexing. The reason for the difference is that UCI multiplexing performed by rate-matching in current specification, so the impact of processing time of other type messages to be considered, and then the S0 is introduced which is the first symbol of the earliest PUSCH. But if UCI multiplexing performed by puncturing, the impact on PUSCH transmission will be ignored, and S0 should be defined independently of PUSCH transmission and probably not the first symbol of PUSCH

Proposal 6: If UCI multiplexing is performed by puncturing，S0  may differ from rate-matching for UCI multiplexing.

In addition, considering that when TBoMS is scheduled, UE is generally located at the edge of the cell with poor coverage. If UCI multiplexing occurs at this time, it means that UCI coverage is also poor. UCI coverage needs to be improved, which can be addressed by increasing UCI resources with UCI repetition.

Proposal 7: If UCI multiplexing in TBoMS is supported, UCI repetition should be considered.

2.6 Frequency hopping

Frequency hopping can obtain the gain of frequency diversity, which can effectively improve the performance of PUSCH transmission. In the last meeting, it has been agreed for a single TBoMS transmission and TBoMS repetitions in Rel-17, the legacy Rel-15/16 intra-slot frequency hopping framework used in PUSCH repetition Type A is supported. In our views, inter-slot frequency hopping and joint channel estimation need also to be supported in the design of TBoMS transmission.

Figure 2: Inter-slot frequency hopping with inter-slot bundling

Proposal 8: The bundling of inter-slot frequency hopping should be supported for TBoMS.

- Conclusion

In this contribution, the following proposals and observations have been made:

Proposal 1: Only dropping the overlapped slot(s) should be considered for TBoMS transmission when collision happen.

Proposal 2. A slot or a set of slots retransmission for TBoMS should be supported.

Proposal 3: The maximum TBS for TBoMS should be limited based on DataRateCC.

Proposal 4: The maximum number of PRBs can be limited when TBoMS is enabled.

Proposal 5: UCI multiplexing is performed by puncturing or rate-matching depending on the determination time is before or latter the starting time of PUSCH preparation.

Proposal 6: If UCI multiplexing is performed by puncturing，S0  may differ from rate-matching for UCI multiplexing.

Proposal 7: If UCI multiplexing in TBoMS is supported, UCI repetition should be considered.

Proposal 8: The bundling of inter-slot frequency hopping should be supported for TBoMS.

References

[1] 3GPP RAN1, Chair’s Notes RAN1#106bis-e

[2] R1-2110530, “Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)”

[3] 3GPP TS 38.214, “NR, Physical layer procedures for data”