3GPP TSG RAN WG1#107-e			R1-2111723

e-Meeting, 11th – 19th November, 2021

Agenda item:	8.1.4

Source: 	Samsung

Title: 	Views on Rel-17 CSI enhancements

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s view on the two Rel. 17 CSI enhancement items included in the WID [1]: CSI enhancements for NC-JT transmission, and CSI enhancements for FDD reciprocity.

NC-JT CSI enhancements

  - RI restriction

In RAN1#106b-e [6], the following agreement was made regarding to RI restriction for Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting.

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting - Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses. - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. FFS: Whether there can be multiple candidate values of X and Y

According to the above agreement two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all the Single-TRP measurement hypotheses and the other is applied to all NCJT measurement hypothesis. Moreover, it was agreed that if a rank restriction of (X, Y) is configured then the reported rank is X for all single TRP hypothesis and reported rank Y, for NCJT measurement hypothesis, which is mapped to one of the four possible rank combination {1,1},{1,2},{2,1} and {2,2}. Furthermore, whether multiple values of X and Y can be configured is left for further study. The legacy RI restrictions in Rel. 15 and 16 are based on bitmap indication with the length of the bitmap vector is set to the largest rank the configured codebook supports. Additionally, the Rel. 15 and 16 codebook configurations allow multiple candidate ranks to be configured, i.e., ranks corresponding to bits in the bitmap vector set to ‘1’ and UE is allowed to report one of them via RI. This allows the UE to flexibly select one of the configured candidates ranks values and indicate it with RI. The same design principle can be considered for Rel. 17 NCJT CSI. Hence, multiple candidate values of X and Y can be configured where RI(s) for Single-TRP measurement hypotheses indicate one of the candidate values of X while the RI for NCJT measurement hypothesis is mapped to one of the candidate values of Y.

Proposal 1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting settings support multiple RI candidate values X and Y for Single-TRP and Multi-TRP measurement hypotheses, respectively.

  - Discussions on CBSR configuration

In RAN1#106b-e [6], the following agreement was made regarding to codebook subset restriction (CBSR) for Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting.

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, down-select one alternative from the following in RAN1 107: - Alt 1: One CBSR can be configured per CodebookConfig, whereas CBSR is applied to all CMRs regardless measurement hypotheses or CMR groups. - Alt 2: Two CBSRs can be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

According to the above agreement a down selection will be made in RAN1#107 between Alt 1: a single CBSR to be applied to all the CMRs regardless of measurement hypotheses or CMR groups and Alt 2: two CBSRs are configured and a CBSR is applied per TRP. Alt1 renders lower flexibility as compared to Alt2 especially when there are few antenna ports. To show this with a simple example, if the possible codebook indices (beams) are given as {b0,b1,b2,b3, b4,b5} and if gNB wants to restrict { b0,b1} based on other-cell UEs experiencing interference from TRP1 and { b5,b6} based on other-cell UEs experiencing interference from TRP2, then Alt1 restricts all but only two codebook indices. Even if Rel. 15 DPS configures a single CBSR that is applied to all CMR resources in a CMR resource set wherein the CMRs may correspond to multiple TRPs, Rel. 17 NCJT CSI should strive for more flexible design and better performance by configuring CBSR per each TRP.

Proposal 2: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting settings support two CBSRs to be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

  - Discussions on Inter-CMR distance in a CMR Pair

In RAN1#106-e [4] and RAN1#106b-e [6], the following agreements were made on whether to restrict the two CMRs within a same CMR pair configured for NCJT measurement hypothesis to be within the same downlink slot or CDRX active time.

Agreement For CSI measurement associated with a CSI-ReportingConfig for NC-JT, study following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis: - FFS: two resources are restricted within the same DL slot - FFS: two resources are restricted with the same CDRX active time Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting - Alt 1: CBSR is supported and can be applied for both single-TRP and Multi-TRP measurement hypotheses. - FFS detailed CBSR signalling configured for Multi-TRP Agreement For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs - X=1, 2 - whereas X=1 implying the same slot and X=2 implying two adjacent slots - FFS other restrictions for FR2 - FFS whether UE capability is needed for X=2

It was agreed to restrict two CMRs within a CMR pair configured for NCJT measurement hypothesis to be restricted within X=1,2 continuous slot(s) without DL/UL switching between the two CMRs. In RAN1#106-bis various reasons were raised for this restriction including CSI accuracy and UE’s memory requirement to buffer the estimated channel. The remaining issue is whether there is a need for UE capability for X=2. In Rel. 15 and 16, a CMR and IMR could be in two adjacent sots without a need for UE capability. Similarly, since in Rel. 17 NCJT CSI a CMR from one TRP serve in a similar way as an IMR to the other TRP, a new capability should not be introduced. Besides, too many UE capabilities have already been introduced in Rel. 17 FeMIMO. Given these two observations, it is preferable if a UE capability is avoided for X=2 and the effective restriction is set to X=2 than X=1,2.

Proposal 3: For two CMRs within a same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs:

- Do not support UE capability for X=2

  - Discussions on PMI sharing method

In RAN1#104bis-e [7], it was agreed to support a 2-part CSI report in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis as below:

Agreement A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications: - Within CSI part 1 - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details - FFS whether RI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead - FFS whether additional field is needed, at least for Option 2 - Within CSI part 2: - FFS further compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses

Further, it was also agreed to study whether to support PMI/RI sharing mechanisms between NC-JT and sTRP CSI in RAN1#105-e [3]. It was also agreed in RAN1#103-e at least ‘typeI-SinglePanel’ codebook is supported. Nevertheless, other codebook types are not precluded. In the following, we discuss the possible compression/omission/sharing of PMI among single-TRP and NCJT hypotheses by giving an example based on the Type II codebook while the same concept can be applied to ‘typeI-SinglePanel’ codebook.

Agreement For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s): - Enabling/Disabling PMI, RI sharing via higher-layer configuration - Dynamic indication of PMI, RI sharing in the CSI report - FFS: other details - FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

Consider an NCJT operation wherein  and  denote the received signal at UE’s antenna panel 1 and 2 corresponding to TRP1 and TRP2, respectively, is given as

- (1)

where   and denote the precoding matrix and data vector, respectively, corresponding to the  TRP. Whereas, the channel matrix   represents the channel between the i-th TRP and j-th receive antenna panel at the UE. Moreover, denotes a noise vector associated with the j-th UE panel.

Similarly, a UE may operate in a single-TRP mode when the channel of a particular TRP is significantly better than the other or based on other conditions such as traffic loads at the TRPs. In this case, the received signal from the i-th TRP can be modeled as

(2)

where ,  and  denote the channel matrix, precoding matrix and data vector, respectively, corresponding to the   TRP. From (1) and (2), it is clear that the main difference in the precoders selection for NCJT (, ) and sTRP hypotheses (, ) is the consideration of the cross-link interference channels, i.e.,  and , in the NCJT case. In other words, when the channels corresponding to the two TRPs are nearly orthogonal, i.e.,  for  wherein  denotes Frobenius norm, the PMI for sTRP hypotheses can be reused for NCJT hypothesis too. This can be considered as a full compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses.

Moreover, considering the codebook format of type II codebook as an example, i.e., , the same beam-group selector matrix  can be reported for both NCJT and single-TRP hypotheses while mitigating cross-link interference by a separately reporting for the second part (). As  corresponds to the dominant angles wherein the received power is concentrated, it doesn’t make much sense to report different PMI components for  corresponding to NCJT and single-TRP hypotheses. The above concept can also be extended to other codebook types and can be considered as partial compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses. The PMI components that could be shared by PMI reporting for single-TRP and NCJT hypotheses can be configured to the UE by higher layer configuration. This way supports flexible trade-off between CSI overhead and precoding performance. In fact, it is reasonable for the UE to dynamically decide which PMI components to be omitted/shared among single-TRP and NCJT hypotheses based on the channel and inter-TRP interference measurement. In other words, upon reporting PMIs for single-TRP and NCJT hypotheses, the UE may dynamically decide on the PMI reporting overhead.

Proposal 4: Support full and/or partial compression/omission/Sharing of PMI among single-TRP and NCJT hypotheses.

Proposal 5: Support the dynamic variation on the level of compression/omission/Sharing of PMI and the associated payload of PMI for single-TRP and NCJT hypotheses.

  - Discussion on UCI structure

The following agreements were made in RAN1#103-e [2] and RAN1#106-e [4], respectively.

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4 - FFS: Maximal transmission layers larger than 4 - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE - FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT - FFS: whether/how to support CRI(s) to be reported in a CSI - FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs - FFS: whether/how to support non-PMI based port-selection - FFS: whether/how to support single value of reported LI Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN Agreement To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis: - Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214. - Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214. - Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

Since the UCI payloads for NCJT and sTRP CSI reports are different, it is preferable if the CSI for NCJT and sTRP are reported separately as distinct reports. Moreover, for option 1 with X=2, the UE has to reports a CSI for NCJT and 2 additional CSIs for sTRP. Then if these 3 CSIs are carried by a single report, then all CSIs may get dropped. However, if CSIs for different measurement hypotheses are reported by distinct reports partial dropping is possible. Accordingly, the priority equation in Section 5.2.5 in 38.214 can be modified by giving the CSI report for NCJT a higher priority. An example for mapping CSI fields of one CSI report in 38.212, for both NCJT and sTRP CSI, is given in Fig. 6. Akin to Rel. 15/16 CSI report, a report can be divided in two parts. The first part of a CSI report for NCJT may consist of CRI and RI, if they are reported, WB and SB differential CQIs, and an indication for sharing of CRI, RI and PMI with sTRP CSI. Moreover, CSI part 2 for NCJT CSI can be subdivided in two parts, i.e., WB and SB parts, and the corresponding fields as illustrated in Fig. 6 can be reported.

Figure 6. UCI structure for CSI report for NCJT and sTRP CSI

Proposal 6: Report NCJT and sTRP CSIs as separate reports.

- Modify priority equation in Section 5.2.5 in 38.214 by giving the CSI report for NCJT a higher priority.

- Include an indication for sharing of CRI, RI and PMI with sTRP measurement hypotheses in CSI part 1 for NCJT.

  - Discussion on Non-PMI CSI

The following agreements were made in RAN1#104-e [5] and RAN1#106-e [4], respectively.

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, an NCJT CSI hypothesis based on a pair of CMRs assumes to occupy two CPUs, two active NZP CSI-RS resources, and a number of active ports corresponding to both CMRs. - If a NZP CSI-RS resource is referred X times by CMR pairs for NCJT measurement hypothesis and CMR for Single-TRP measurement hypothesis, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times for active resources and active ports. - Note: For above CSI computation, UE assumes PDSCH transmission is single-DCI based multi-TRP scheme(s). FFS: Multi-DCI based multi-TRP scheme Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT measurement hypothesis, study whether to support non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" in Rel-17 - Related details, if needed, are to be discussed in RAN1#106bis. - Interested companies are encouraged to share details and related specification impact if support

In some cases, it is computationally burdensome for UE to compute NCJT CSI. In fact, if the UE is configured with Option 1 and X=2 a UE has to compute and report 4 PMIs. When channel reciprocity is available the CSI can be computed at the gNB and non-PMI based CSI reporting can be utilized. In order to minimize the remaining specification work a non-PMI CSI reporting without higher layer parameter non-PMI-PortIndication can be adopted.

Proposal 7: For NC-JT CSI reporting enhancement, support following

- Non-PMI CSI reporting

- Minimize the remaining specification work by adopting Non-PMI CSI without non-PMI-PortIndication configuration.

CSI enhancements for FDD reciprocity

The following agreements were made in RAN1#105-e [3] and RAN1#106bis-e [6] about the Rel. 17 codebook enhancement based on FDD reciprocity.

RAN1#105-e Agreement For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook - FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2 - FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8 Agreement For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e: - Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed - Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed. - Alt 3: Keep both Wf OFF and Wf ON with Mv=1. - If PMI format is SB, Wf is an all-one vector of length N3 - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e - Note: N3 = NCQISubband*R. - FFS: the case when no SB size is configured.

RAN1#106bis-e Agreement For UCI part II of Rel-17 PS codebook, study the following alternatives and down-select one or more alternatives in RAN1 107 - Alt 1: Report Port indicator, SCI, and FD indicator in Group 0 - Alt 2: Report bitmap in Group 0 or Group 1 without bitmap partition - Alt 3: Three groups of UCI Part 2 for Rel-16 PS codebook is reused for Rel-17 PS codebook enhancement except that the starting position of the FD basis window is not needed Note that other solutions of UCI part II design are not excluded. Agreement With regarding to parameter combinations, following 8 parameter combinations are supported in Rel-17 PS codebook: M; Alpha; Beta. 1; 1; 1. 1; 1; 3/4. 1; 1; 1/2. 1; 3/4; 1/2. 2; 1; 3/4. 2; 1; 1/2. 2; 3/4; 1/2. 2; 1/2; 1/2. FFS: whether further restrictions/dependences for given parameter combination(s) are needed Agreement For the priority of mapping coefficients for Rel17 PS codebook, study the following alternatives and down-select one or more alternatives in RAN1#107-e: - Alt 1: Support mapping coefficients firstly across port indices, secondly across FD basis indices, and thirdly across layers, i.e. priority value is given by the priority value Pril,i,f=K1⋅M⋅l+K1⋅f+i - Alt 2: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅i+l - Alt 3: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l - FFS port permutation function ψ(i) Note that other solutions are not excluded. Agreement In addition to N=2, N=4 is supported when M=2 for rank 1/2 - For rank 3/4, when M=2, N = 2 or 4 is supported and same with the value of N configured for rank 1/2 - FFS how to handle N3=3 case Agreement For Rel-17 PS codebook, support R=2 when M=2 Note that this R is optional, whereas how to support R=2 in Rel-17 UE capability signalling is FFS, e.g. similar with Rel-16 eType II codebook.

In this subsection, we first provide our views related to the remaining issues. In particular, the following open issues have been discussed. The simulation assumptions in this section are according to the agreed EVM in RAN1#102-e, which are also copied in Table 2 in the appendix A. The Rel. 16 PS T2 with paraComb = 1 is considered as reference, unless stated otherwise.

  - M = 1

According to agreement in RAN1#104-e [5], the Wf component of the codebook can be turned OFF by configuring M=1. The length of all-one vector when M=1 is still open, and there are three alternatives (according to the agreement copied above) to down-select from. We provide our view about this issue below.

Agreement (RAN#104-e) For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain

Based on the agreement,

- when Wf is OFF, Wf is an all-one vector

- a single precoding matrix indicated by the PMI for all SBs or for the entire CSI reporting band

Spec related issue:

- N3 is defined as the total number of precoding matrices (based on the cyan text).

- If N3 >1, we can’t say “N3 precoding matrices” for M=1 since there is only one precoding matrix indicated by the PMI.

- We can say one of the following:

  - A single precoding matrix is indicated by the PMI

  - N3 precoding matrices indicated by the PMI, but they are the same when M=1

Section 5.2.2.2.5, 38.214 The parameter R is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows

Observation 1: according to current 38.214 specification, N3 is defined as the total number of precoding matrices, which is incorrect when Wf OFF (M=1) since there is only one precoding matrix.

We therefore propose the following.

Proposal 8: When M=1, the description on N3 includes one of the following:

- Alt1: a single precoding matrix is indicated by the PMI

- Alt2: N3 precoding matrices indicated by the PMI, but they are the same when M=1

  - BWP < 24 PRBs

In the current specification, the CSI reporting is restricted to WB CSI using Rel.15 Type I codebook when the BWP size < 24 PRBs. The reason for supporting WB only CSI is due to the fact there is no SB size supported for such small BWPs. In our view, the WB CSI reporting should be supported for Rel.17 codebook for all BWPs, both BWP < 24 and BWP >= 24 PRBs. Some rationales behind this are as follows:

- Rel.17 CB can be used for any BWP sizes since gNB performs some frequency compression via beamformed CSI-RS so that the beamformed channel is less sensitivity to frequency selectivity when compared with Rel.16 codebook. This is also corroborated by the fact that the performance gains of the Rel. 17 codebook shrink quickly with large M value.

- Rel.17 codebook with M=1 (Wf OFF) essentially corresponds to WB PMI reporting, hence, can be configured for WB CSI reporting for BWP size < 24.

- Performance gain: the UPT vs overhead trade-off comparing Rel.17 codebook and Rel.15 Type I codebook for BWP = 20 PRBs is shown in Figure 1. A large performance gain (up to 16% in avg. UPT) can be observed.

- Reduced capacity or low cost UEs: since the complexity of Rel.17 codebook (especially when Wf is OFF) is small (comparable to Rel.15 Type I), and it achieves large UPT gain, it can be quite attractive for use cases such as low cost UEs, reduced capability UEs, NR railways, or more futuristic use cases.

- Finally, only low-resolution (Type I) codebook is supported for BWP size < 24, it is beneficial from the perspective of the overall system to also support a high-resolution (Type II) codebook for these BWP sizes. The performance gain can be large, and the CSI overhead and UE complexity are reasonable.

Observation 2: for BWP size = 20 PRBs, Rel.17 codebook can achieve up to 16% gain in avg. UPT over the current specification support (i.e. based on Rel.15 Type I codebook)

Proposal 9: support Rel.17 codebook for BWP size < 24 PRBs with the current restriction in the specification, i.e. support only WB CSI implying M=1

Figure 1

  - Further restriction on M = 2

As discussed in past RAN1 meetings, the support of M > 1 should be based on the tradeoff among UPT performance, CSI overhead, and UE complexity, as mentioned in WID [1]. This tradeoff for M=2 is described in Appendix C based on the results from companies during RAN1#104bis-e and RAN1#105-e. It is evident that this tradeoff is attractive for small #CSI-RS ports, but not so for high #CSI-RS ports. In fact, for 32 CSI-RS ports, there is hardly any UPT gain, but the CSI overhead and UE complexity burden are very high. The very small UPT gain is due to the fact that when the number of CSI-RS ports is large (e.g. 32), then with high probability, all dominant SD-FD pairs can be beamformed by the gNB via CSI-RS ports, and hence the resultant beamformed DL channel may not have any strong FD components, hence, the Wf component in the codebook may not bring any performance benefits. Therefore, the UE supporting M > 1 should not be forced to support this feature for all values of number of CSI-RS ports (P). Such a UE should be allowed to provide the information whether it UE support P > t (threshold), where t = 12, for example.

Proposal 10: Regarding M = 2, UE reporting its capability to support M = 2 shall also report whether it supports M=2 for P > 12 CSI-RS ports

  - Wf window size when N3=3

Agreement In addition to N=2, N=4 is supported when M=2 for rank 1/2 - For rank 3/4, when M=2, N = 2 or 4 is supported and same with the value of N configured for rank 1/2 - FFS how to handle N3=3 case

According to agreement in RAN1#106b-e, the additional value of N>2 for the case when N3=3 is FFS. A simple solution for this case can be setting N = N3, which can be achieved by setting the window size as min(N3,N).

Proposal 11: when N3=3, the value of N>2 is N=3, which is achieved by setting the window size as min(N3,N).

  - Parameter combinations

Some of the remaining issues for the codebook parameters are as follows:

- #CSI-RS ports (PCSIRS): the maximum value of PCSIRS has been agreed to be 32. Regarding the minimum PCSIRS value, in our view, PCSIRS=2 should be supported in addition to {4,8,12,16,24,32}. This can be beneficial for scenarios in which the UL-DL reciprocity is strong (including the TDD scenario) and the channel has very few (1 or 2) strong clusters. Note that a 2-port beamformed codebook (Class B, K=1) is supported in LTE.

- Restrictions on parameter combinations: in our view, there is no need for specifying any restrictions on the agreed parameter combinations unless it is an infeasible configuration. Two such infeasible configurations are as follows.

  - α=34: since K1=3 and 9 for PCSIRS=4 and 12, respectively, the value L=K12 is not an integer, implying L port selection is not possible. Such parameter combinations for 4 and 12 CSI-RS ports therefore can’t be configured.

  - PCSIRS=4 case: when α=12, the value of K1=2 implying that only rank 1-2 CSI can be reported. Therefore, α=12 can be restricted to rank 1 or 2 only (i.e. rank 3 or 4 reporting can’t configured in this case).

Proposal 12: Regarding Rel.17 codebook parameters,

- support PCSIRS=2

- specify the following restrictions on the parameter combinations

  - α=34 can’t be configured when PCSIRS=4 or 12

  - allowed rank can’t be 3 or 4 when α=12 and PCSIRS=4

  - UCI design

Re UCI omission, we prefer to reuse the Rel.16 UCI omission, i.e., a two-part UCI comprising UCI part 1 and part 2 is used, UCI part 1 is the same as in Rel. 16, and UCI part 2 is partitioned in three groups G0, G1, and G2, where the three groups are simplified when compared with that in Rel.16. For instance,

- Permutation: since the maximum value of M is 2, which is subject to UE capability, then there is no need for any FD permutation. Re permutation in spatial domain (Alt 3 in RAN1#106b-e agreement on priority of mapping coefficients), we don’t see any need for it; however, a simple permutation can be considered if need and benefit can be justified. One example of simple permutation can be based on even and odd numbering of SD indices.

- Bitmap partition: since the bitmap size is much smaller than that in Rel.16, there is no need and benefit associated with it. We therefore prefer un-partitioned bitmap reported in G0 or G1 (i.e. Alt 2 of RAN1#106b-e agreement on UCI part 2 design).

- FD basis indicator: since SCI(s) indicates both SD and FD indices of the strongest coefficient, it makes sense to include both SD and FD indicators in G0 together with the SCI(s). We therefore can support Alt 1 of RAN1#106b-e agreement on UCI part 2 design).

Proposal 13: for Rel.17 codebook, reuse two-part UCI comprising part 1 and part 2

- UCI part 1 is the same as in Rel. 16

- For UCI part II,

  - support the following from RAN1#106b-e:

    - Alt 1: Report Port indicator, SCI, and FD indicator in Group 0

    - Alt 2: Report bitmap in Group 0 or Group 1 without bitmap partition

  - Re coefficient mapping, support the following from RAN1#106b-e:

    - 1st preference (Rel. 16): Alt 2

      - mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅i+l

    - 2nd preference (if need and benefit can be justified): Alt 3

      - mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l, where the SD permutation ψ(i) is based on even and odd SD indices, i.e. 0, 2, 4, ….., 1, 3, 5,.

Conclusion

In this contribution, the following observations and proposals are made:

NC-JT CSI enhancements

Proposal 1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting settings support multiple RI candidate values X and Y for Single-TRP and Multi-TRP measurement hypotheses, respectively.

Proposal 2: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting settings support two CBSRs to be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

Proposal 3: For two CMRs within a same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs:

- Do not support UE capability for X=2

Proposal 4: Support full and/or partial compression/omission/Sharing of PMI among single-TRP and NCJT hypotheses.

Proposal 5: Support the dynamic variation on the level of compression/omission/Sharing of PMI and the associated payload of PMI for single-TRP and NCJT hypotheses.

Proposal 6: Report NCJT and sTRP CSIs as separate reports.

- Modify priority equation in Section 5.2.5 in 38.214 by giving the CSI report for NCJT a higher priority.

- Include an indication for sharing of CRI, RI and PMI with sTRP measurement hypotheses in CSI part 1 for NCJT.

Proposal 7: For NC-JT CSI reporting enhancement, support following

- Non-PMI CSI reporting

- Minimize the remaining specification work by adopting Non-PMI CSI without non-PMI-PortIndication configuration.

FDD CSI enhancements

Observation 1: according to current 38.214 specification, N3 is defined as the total number of precoding matrices, which is incorrect when Wf OFF (M=1) since there is only one precoding matrix.

Observation 2: for BWP size = 20 PRBs, Rel.17 codebook can achieve up to 16% gain in avg. UPT over the current specification support (i.e. based on Rel.15 Type I codebook)

Proposal 8: When M=1, the description on N3 includes one of the following:

- Alt1: a single precoding matrix is indicated by the PMI

- Alt2: N3 precoding matrices indicated by the PMI, but they are the same when M=1

Proposal 9: support Rel.17 codebook for BWP size < 24 PRBs with the current restriction in the specification, i.e. support only WB CSI implying M=1

Proposal 10: Regarding M = 2, UE reporting its capability to support M = 2 shall also report whether it supports M=2 for P > 12 CSI-RS ports

Proposal 11: when N3=3, the value of N>2 is N=3, which is achieved by setting the window size as min(N3,N).

Proposal 12: Regarding Rel.17 codebook parameters,

- support PCSIRS=2

- specify the following restrictions on the parameter combinations

  - α=34 can’t be configured when PCSIRS=4 or 12

  - allowed rank can’t be 3 or 4 when α=12 and PCSIRS=4

Proposal 13: for Rel.17 codebook, reuse two-part UCI comprising part 1 and part 2

- UCI part 1 is the same as in Rel. 16

- For UCI part II,

  - support Alt 1 and Alt 2 following from RAN1#106b-e:

    - Alt 1: Report Port indicator, SCI, and FD indicator in Group 0

    - Alt 2: Report bitmap in Group 0 or Group 1 without bitmap partition

  - Re coefficient mapping, support the following from RAN1#106b-e:

    - 1st preference (Rel. 16): Alt 2

      - mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅i+l

    - 2nd preference (if need and benefit can be justified): Alt 3

      - mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l, where the SD permutation ψ(i) is based on even and odd SD indices, i.e. 0, 2, 4, ….., 1, 3, 5,.

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- Chairman’s notes, RAN1#103-e

- Chairman’s notes, RAN1#105-e

- Chairman’s notes, RAN1#106-e

- Chairman’s notes, RAN1#104-e

- Chairman’s notes, RAN1#106bis-e

- Chairman’s notes, RAN1#104b-e

Appendix A

Table 2: Simulation assumptions for FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance |  | 200m |
| Reciprocity model |  | Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 44 dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz with 15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO, rank 1 only |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | CSI-RS, DMRS |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% for SU/MU-MIMO, rank 1 only |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | User throughput vs CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular and PS eTypeII codebooks |
| SRS modeling for UL channel estimation |  | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897 with Δ=9 BW: same as CSI-RS Number of ports = 2 Tx power = 23 dBm |
| FDD DL/UL calibration error model at gNB |  | According to R1-144943, with amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant per 4 PRB at frequency. |

Appendix C

WID:

Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

RAN1#104bis-e

Companies: HW, Nokia, CATT, MTK, Fraunhofer, QCM, SS, E///

Results comparing M=2 with Wf OFF (M=1)

| Company | 8 ports | 16 ports | 32 ports |
| --- | --- | --- | --- |
| C1 |  |  | No comparison |
| C2 |  |  | No comparison |
| C3 |  |  | Small gain (1%) |
| C4 |  | Mod gain (2-3%) |  |
| C5 |  | Small gain (~1%) | Very small gain (<0.5%) |
| C6 |  |  | No gain |
| C7 |  | Small gain (1-2%) | No gain |
| C8 | Gain (~4%) | Small gain (~1%) | No gain |
| Performance | Gain | Small gain | No gain |
| Overhead | Not too high | High | Highest |
| Complexity | Not too high | High | Highest |
| Overall trade-off among 3 metrics | Yes | No | No |

RAN1#105-e

Companies: HW, CATT, Fraunhofer, Nokia/NSB, SS, MTK, E///

Results comparing M=2 with Wf OFF (M=1)

| Company | 8 ports | 12 ports | 16 ports | 24 ports | 32 ports |
| --- | --- | --- | --- | --- | --- |
| C1 |  | 3.58% | 3.26% | 2.31% | 0.87% |
| C2 |  |  | <3% |  | <1.3% |
| C3 |  |  | ~1% |  | ~1% |
| C4 |  |  |  |  | No gain in lower overhead regime, very small gain in high overhead regime; New results: gain when there SRS BW < ½ CSIRS BW |
| C5 |  |  | Small gain (1-2%) |  | No gain |
| C6 |  |  | Mod gain (3-4%) |  |  |
| C7 | Mod gain (2-3%) |  | Mod gain (2-3%) |  | No gain |
| C8 |  |  |  |  |  |
| Performance | Gain |  | Small gain |  | No gain, very small gain |
| Overhead | Not too high |  | High |  | Highest |
| Complexity | Not too high |  | High |  | Highest |
| Overall trade-off among 3 metrics | Yes |  | No |  | No |