3GPP TSG RAN WG1 Meeting #107-e			                                   R1- 2111819

e-Meeting, November 11th – 19th, 2021

______________________________________________________________________Agenda item: 8.11.1.2

Source: Moderator (LG Electronics)

Title: Feature lead summary #3 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements

Document for: Discussion and information

- Draft proposals for Thursday’s GTW (November 11th)

  - Scheme 1

FL’s observation: It is observed that companies’ views on the container and signalling contents are still divergent. Considering this is the last RAN1 meeting for this WI, it is proposed to take a straightforward signalling design based on the bitmap without further consideration, e.g., compression of signalling bits. And it is observed that higher layer signalling is suitable for the bitmap because of the size issue.

Draft proposal 1-1:

- For the set of resources in Scheme 1, the following is supported for its indication mechanism:

  - Bitmap indication where each bit indicates whether a pair of sub-channel and slot is included in the set of resources

    - Higher layer signalling (Working assumption: MAC CE) is used as the container of this bitmap

Updated Draft proposal 1-1:

- For Scheme 1, down-select one of following options as the container of inter-UE coordination information transmision:

  - Option 1: Higher layer signalling (Working assumption: MAC CE)

  - Option 2: Physical layer signalling, i.e., SCI

  - Option 3: Higher layer signalling (Working assumption: MAC CE) and physical layer signalling, i.e., SCI

    - FFS: Which one will be used for which case

FL’s observation: Assuming that higher layer signalling is used as the container of inter-UE coordination information, it seems that there is no technical difference between Option 1-1 and Option 1-2. Also for this case, PHY layer doesn’t need to report the intersection of S_A and the preferred resource set to MAC layer. Given that this is the last RAN1 meeting for this WI, this direction seems more preferable than Option 2 which has FFS points that need to be resolved additionally and can be a wayforward to simplify the operation.

Draft proposal 1-2:

- For Option A of Scheme 1 with preferred resource set,

  - S_A report from PHY layer of UE-B is the same as the outcome after Step 7) of Rel-16 TS 38.214 Section 8.1.4

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

FL’s observation: Since Option 2 in the contribution summary is supported by the majority of companies, so it is proposed to adopt it.

Updated draft proposal 1-3:

- For Scheme 1 with non-preferred resource set,

  - Option 2: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

  - FFS: Whether/how to handle the case when the requirement of X⋅Mtotal  as specified in Step 7) of TS 38.214 Section 8.1.4 is not satisfied

  - FFS: Whether/how to determine M_total based on non-preferred resources in step 7)

FL’s observation: Since applying RSRP threshold increase is supported by the majority of companies, so following is proposed.

Draft proposal 1-4:

- For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resoruces preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS38.214 Sectoin 8.1.4.

FL’s observation: Since informing UE-B’s resource selection window from UE-B to UE-A is supported by the majority of companies, so following is proposed.

Draft proposal 1-5:

- For Condition 1-A-1 in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Starting/Ending time locations of resource selection window is provided by signaling from UE-B

    - It replaces the time interval [n+T_1,n+T_2]

FL’s observation: Since employing UE-B’s request to convey the parameters for determining the set of resoruces is supported by the majority of companies, so following is proposed.

Draft proposal 1-6:

- For Scheme 1, at least following parameters are provided by UE-B’s request:

  - Priority value to be used for PSCCH/PSSCH transmission

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

  - Resource reservation interval

  - Starting/Ending time locations of resource selection window

FL’s observation: Since companies’ views are still divergent and it seems that common understanding that which feature combinations are supported in Rel-17 is a separate issue/discussion, so following is proposed.

Draft proposal 1-7:

- The following feature combination(s) can be enabled or disabled or controlled by (pre)configuration

  - Scheme 1 with preferred resource set triggered by an explicit request

  - Scheme 1 with non-preferred resource set triggered by an explicit request

  - Scheme 1 with preferred resource set triggered by a condition rather than request reception

  - Scheme 1 with non-preferred resource set triggered by a condition rather than request reception

  - Scheme 2

  - Note: It is a separate issue/discusison which combinations of features are supported in Rel-17.

  - Scheme 2

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

Note from FL: In the discussion of Scheme 2, ‘UE-B’ and ‘other UE’ mean the UEs scheduling the conflicting TBs as in the working assumption above, where UE-A is a destination UE of at least one of two TBs.

FL’s observation: FL understands that the original Option 1 is for the case where UE-A is a destiantoin of UE-B and in order to incorporate the other case implied by the working assumption, a variant of Option 1 (Option 1’) is also necessary for the case where UE-A is a destination of the other UE.

FL’s observation: Option 1 (and its variant) is supported by the majority of companies, so it is proposed to adopt it as an additional criteria.

Draft proposal 2-1-1:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 1’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

FL’s observation: It is observed that several companies propose to consider additional critieria which in FL’s understanding is compatible with Option 1. It is porposed to adopt Option 4 which is claimed to provide the performance benefit over Option 1 by simulation submitted to this meeting.

Draft proposal 2-1-2:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 4’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - FFS: Whether the threshold depends on priority

FL’s observation: Since UE-B selection among UEs scheduling conflicting TBs based on their priority is supported by the majority of companies, so following is proposed. For better understanding on the draft proposal 2-2, let me provide some example. When UE-1, UE-2, and UE-3 schedule the conflicting TBs, UE-A selects UE(s) with higher priority value as UE-B(s) for each pair of (UE-1, UE-2)-pair, (UE-1, UE-3)-pair, (UE-2, UE-3)-pair. If priority values of UE-1, UE-2, and UE-3 are in increasing order, UE-A finally sends the inter-UE coordination information to UE-2 and UE-3.

Draft proposal 2-2:

- For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value can be UE-B.

FL’s observation: It is observed that the following was claimed as the main motivation of each option in the PSFCH occasion for inter-UE coordination information transmission. No strong majority view was observed.

- Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

  - Reselection can be triggered earlier, so more resources remain for the reselection

- Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

  - When an SCI with conficting TB is transmitted after the PSFCH occastion of Option 1, Option 1 has a limitation in sending the inter-UE coordination information. Option 2 can avoid this problem.

FL’s suggestion:

- For PSFCH occasion for inter-UE coordination information for Scheme 2, collect company input on the main main motivation of two options and make down-selection.

FL’s observation: Since applying different m_CS values for different conditioins is supported by the majority of companies, so following is proposed.

Draft proposal 2-3:

- For PSFCH resource index determination in Scheme 2,

  - m_CS: 0 when UE-A determines Condition 2-A-1 is satisfied

  - m_CS: 6 when UE-A determines Condition 2-A-2 is satisfied

- Draft proposals for Monday’s GTW (November 15th)

  - Scheme 1

Draft proposal 1-1:

- For Scheme 1, down-select one of following options as the container of inter-UE coordination information transmission:

  - Option 1: Higher layer signalling (Working assumption: MAC CE)

  - Option 2: Physical layer signalling, i.e., SCI

  - Option 3: Higher layer signalling (Working assumption: MAC CE) and physical layer signalling, i.e., SCI

FL observation: The views between companies were similarly divided between Option 1 and Optoin 2, and futher email discussion is not meaningful in terms of makin progress. So, it is necessary to make a decision in the GTW session. The proponents of Option 1 commented that there is no critical difference between Option 1 and Option 2 (especially for 2nd SCI) in terms of latency and reliability because MCS is selected by UE and the same resource selection will be applied irrespective of signalling option.

- Option 1: OPPO, NEC, Lenovo, Ericsson (details up to RAN2), Intel, DOCOMO, LGE, Apple (8)

- Option 2: vivo (SCI for non-preferred resource set), Nokia, Huawei, Samsung (2nd SCI), Fraunhofer, Xiaomi, CMCC, CATT (8)

- Option 3: vivo (SCI for non-preferred resource set), Nokia (as compromise), Qualcomm (2nd SCI for preferred resource set and MAC CE for other cases), Futurewei (SCI for preferred and non-preferred resource sets for half duplex, MAC CE for all types of coordination information), Huawei (2nd SCI for preferred resource set and MAC CE for non-preferred resource set), Fraunhofer (6)

Updated draft proposal 1-1:

Alt 1:

- For Scheme 1,

  - Higher layer signalling (Working assumption: MAC CE) is used as the container of inter-UE coordination information transmission.

    - Down-select one of following options for the indication of resource set in RAN1#107-e meeting

      - Option 1: N combinations of TRIV, FRIV, resource reservation period (when the transmission of inter-UE coordination information is triggered by a condition rather than request reception or when the transmission of non-perferred resource set is triggered by an explicit request) as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification:

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - FFS: Value of N.

      - Option 2: Bitmap indication where each bit indicates whether a pair of sub-channel(s) and slot(s) is included in inter-UE coordination information

        - FFS: Granularity in time-and-frequency resources

        - FFS: Other information (if any)

      - Option 3: When the resource set indicated by UE-A is a set of k candidate single-slot resources, the resource set is indicated by

RIV=j=1krjj+i=1LsubCH-1nik

If LsubCH is is known by UE-B, the second sum term is omitted

Alt 2:

- For Scheme 1,

  - 2nd SCI is used as the container of inter-UE coordination information transmission.

    - Down-select one of following options for the indication of resource set in RAN1#107-e meeting

      - Option 1: N combinations of TRIV, FRIV, resource reservation period (when the transmission of inter-UE coordination information is triggered by a condition rather than request reception or when the transmission of non-perferred resource set is triggered by an explicit request) as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification:

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - FFS: Value of N.

      - Option 2: Bitmap indication where each bit indicates whether a pair of sub-channel(s) and slot(s) is included in inter-UE coordination information

        - FFS: Granularity in time-and-frequency resources

        - FFS: Other information (if any)

      - Option 3: When the resource set indicated by UE-A is a set of k candidate single-slot resources, the resource set is indicated by

RIV=j=1krjj+i=1LsubCH-1nik

If LsubCH is is known by UE-B, the second sum term is omitted

Alt 3:

- For Scheme 1,

  - Higher layer signalling (Working assumption: MAC CE) is used as the container of inter-UE coordination information transmission in case of non-perferred resource set. 2nd SCI is used as the container of inter-UE coordination information transmission in case of perferred resource set.

    - Down-select one of following options for the indication of resource set in RAN1#107-e meeting

      - Option 1: N combinations of TRIV, FRIV, resource reservation period (when the transmission of inter-UE coordination information is triggered by a condition rather than request reception or when the transmission of non-perferred resource set is triggered by an explicit request) as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification:

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - FFS: Value of N.

      - Option 2: Bitmap indication where each bit indicates whether a pair of sub-channel(s) and slot(s) is included in inter-UE coordination information

        - FFS: Granularity in time-and-frequency resources

        - FFS: Other information (if any)

      - Option 3: When the resource set indicated by UE-A is a set of k candidate single-slot resources, the resource set is indicated by

RIV=j=1krjj+i=1LsubCH-1nik

If LsubCH is is known by UE-B, the second sum term is omitted

Draft proposal 1-2:

- For Option A of Scheme 1 with preferred resource set,

  - S_A report from PHY layer of UE-B is the same as the outcome after Step 7) of Rel-16 TS 38.214 Section 8.1.4

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

FL observation: Depending on a decision of container used for the inter-UE coordination transmsison, whether or not PHY layer reports the intersection set to MAC layer is differernt.

- Support: vivo (1)

- Not support: Fraunhofer (depending on the outcome of draft proposal 1-1) (1)

Updated draft proposal 1-2:

Alt 1 (when MAC CE is used as the container of inter-UE coordination information transmission):

- For Option A of Scheme 1 with preferred resource set,

  - S_A report from PHY layer of UE-B is the same as the outcome after Step 7) of Rel-16 TS 38.214 Section 8.1.4

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

Alt 2 (when 2nd SCI is used as the container of inter-UE coordination information transmission):

- For Option A of Scheme 1 with preferred resource set,

  - PHY layer at UE-B reports both the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection

  - MAC layer selects resources using S_A and the intersection set

    - MAC layer firstly selects resources for transmissions within the intersection set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

Updated draft proposal 1-3:

- For Scheme 1 with non-preferred resource set,

  - Option 2: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

  - FFS: Whether/how to handle the case when the requirement of X⋅Mtotal  as specified in Step 7) of TS 38.214 Section 8.1.4 is not satisfied

  - FFS: Whether/how to determine M_total based on non-preferred resources in step 7)

FL observation: Seem that there is no critical concern on the current version of draft proposal.

- Support: Huawei (with removing FFF points), Fraunhofer, OPPO, vivo (with keeping two FFS points), CATT (with keeping FFS point of ‘Whether/how to determine M_total based on non-preferred resources in step 7’), Samsung (6)

- Not support:

Updasted draft proposal 1-3:

- For Scheme 1 with non-preferred resource set,

  - Option 2: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

  - FFS: Whether/how to handle the case when the requirement of X⋅Mtotal  as specified in Step 7) of TS 38.214 Section 8.1.4 is not satisfied

  - FFS: Whether/how to determine M_total based on non-preferred resources in step 7)

Draft proposal 1-4:

- For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS38.214 Section 8.1.4.

FL observation: Since applying RSRP threshold increase is supported by the majority of companies, so following is proposed.

- Support: Fraunhofer, CATT, vivo, Samsung (4)

- Not support: OPPO (up to UE implementation) (1)

Draft proposal 1-4 (no change from previous version):

- For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS38.214 Section 8.1.4.

Draft proposal 1-5:

- For Condition 1-A-1 in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Starting/Ending time locations of resource selection window is provided by signaling from UE-B

    - It replaces the time interval [n+T_1,n+T_2]

FL observation: Since informing UE-B’s resource selection window from UE-B to UE-A is supported by the majority of companies, so following is proposed.

- Support: Fraunhofer, Lenovo, CATT, OPPO (4)

- Not support: vivo (1)

Draft proposal 1-5 (no change from previous version):

- For Condition 1-A-1 in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Starting/Ending time locations of resource selection window is provided by signaling from UE-B

    - It replaces the time interval [n+T_1,n+T_2]

Draft proposal 1-6:

- For Scheme 1, at least following parameters are provided by UE-B’s request:

  - Priority value to be used for PSCCH/PSSCH transmission

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

  - Resource reservation interval

  - Starting/Ending time locations of resource selection window

FL observation: Since employing UE-B’s request to convey the parameters for determining the set of resoruces is supported by the majority of companies, so following is proposed.

- Support: Fraunhofer, Lenovo, CATT, OPPO (4)

- Not support:

Draft proposal 1-6 (no change from previous version):

- For Scheme 1, at least following parameters are provided by UE-B’s request:

  - Priority value to be used for PSCCH/PSSCH transmission

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

  - Resource reservation interval

  - Starting/Ending time locations of resource selection window

Draft proposal 1-7:

- The following feature combination(s) can be enabled or disabled or controlled by (pre)configuration

  - Scheme 1 with preferred resource set triggered by an explicit request

  - Scheme 1 with non-preferred resource set triggered by an explicit request

  - Scheme 1 with preferred resource set triggered by a condition rather than request reception

  - Scheme 1 with non-preferred resource set triggered by a condition rather than request reception

  - Scheme 2

  - Note: It is a separate issue/discussion which combinations of features are supported in Rel-17.

FL observation: The views between companies were similarly divided, the previous version of draft proposal was kept. Also it seems that other companies without providing inputs have no concern on the current version of draft proposal.

- Support: Fraunhofer, Lenovo, vivo (3)

- Not support: Huawei (no separation between preferred and non-preferred resource sets), CATT (prefer to remove ‘Scheme 1 with preferred resource set triggered by a condition rather than request reception’), OPPO (no separation between preferred and non-preferred resource sets), Samsung (not supportive of the case where the transmission of inter-UE coordination information is triggered by a condition rather than request reception) (4)

Draft proposal 1-7 (no change from previous version):

- The following feature combination(s) can be enabled or disabled or controlled by (pre)configuration

  - Scheme 1 with preferred resource set triggered by an explicit request

  - Scheme 1 with non-preferred resource set triggered by an explicit request

  - Scheme 1 with preferred resource set triggered by a condition rather than request reception

  - Scheme 1 with non-preferred resource set triggered by a condition rather than request reception

  - Scheme 2

  - Note: It is a separate issue/discusison which combinations of features are supported in Rel-17.

  - Scheme 2

Draft proposal 2-2:

- For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value can be UE-B.

FL observation: Since the majority of companies support with the modified version. So it is proposed to adopt it.

- Support: OPPO (w/ some modification), NEC, DOCOMO, CATT, Intel, LGE, Fraunhofer, Lenovo, vivo (9)

- Not support: Huawei (up to UE implementation), Apple (up to UE implementation) (2)

Updated draft proposal 2-2:

- For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

Draft proposal 2-4:

- Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

- Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

FL observation: Since Option 2 is supported by the majority of companies, so it is proposed to adopt it.

- Option 1: CATT, Huawei, Samsung, Apple, Xiaomi (5)

- Option 2: OPPO, vivo, DOCOMO, Intel, LGE, Fraunhofer, Lenovo, NEC (8)

- Both Option 1 and Option 2: vivo (as compromise) (1)

Updated draft proposal 2-4:

- For PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

  - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least X slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

    - FFS: value of X

Draft proposal 2-1-1:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 1’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

FL observation: Seem that there is no critical concern on the current version of draft proposal.

- Support: Huawei, Lenovo, CATT, OPPO, vivo, Fraunhofer (6)

- Not support:

Draft proposal 2-1-1 (no change from previous version):

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

      - Strive to reuse Rel-16 specification wherever possible

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 1’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

      - Strive to reuse Rel-16 specification wherever possible

Draft proposal 2-1-2:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 4’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - FFS: Whether the threshold depends on priority

FL observation: Since the majority of companies support with Option 4/4’. So it is proposed to adopt it.

- Support: DOCOMO, OPPO (with (pre)configurability between Option 1/1’ and Option 4/4’), vivo, Nokia (with adding ‘MCS, amount of overlapping resources’ in FFS point), Fraunhofer (5)

- Not support: Huawei, Samsung (2)

Draft proposal 2-1-2 (no change from previous version):

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority, MCS, amount of overlapping resources

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 4’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - FFS: Whether the threshold depends on priority, MCS, amount of overlapping resources

Draft proposal 2-3:

- For PSFCH resource index determination in Scheme 2,

  - m_CS: 0 when UE-A determines Condition 2-A-1 is satisfied

  - m_CS: 6 when UE-A determines Condition 2-A-2 is satisfied

FL observation: Since the majority of companies support with this proposal, so it is proposed to adopt it.

- Support: Lenovo, CATT, vivo (only for unicast), Fraunhofer (4)

- Not support: Samsung (1)

Updated draft proposal 2-3 (no change from previous version):

- For PSFCH resource index determination in Scheme 2,

  - m_CS: 0 when UE-A determines Condition 2-A-1 is satisfied

  - m_CS: 6 when UE-A determines Condition 2-A-2 is satisfied

- Draft proposals for Wednesday’s GTW (November 17th)

  - 1st set of draft proposals

    - Scheme 1

Draft proposal 1-1: FL observed that companies’ views are still devided between two alternatives, and having futher email discussion is not meaningful in terms of making progress. So, it is necessary to make a decision in the GTW session.

Alt 1:

Support: Huawei, Futurewei (w/ changing 2nd SCI to SCI), Qualcomm (w/ 1st preference), vivo (w/ 1st preference), NEC, Fraunhofer, CMCC, (7 companies)

- For Scheme 1,

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 3 and UE-A has a TB to be transmitted to UE-B, 2nd SCI is used and transmitted with the TB. Otherwise, MAC CE is used.

Alt 2:

Support: Ericsson, Nokia (w/ adding Option 3), Apple (w/ adding Option 2 for non-preferred resource set), Sharp, DOCOMO, Qualcomm (w/ 2nd preference), Samsung (w/ adding Option 2), vivo (w/ 2nd preference), Fujitsu, OPPO, Intel, (9 companies + 2)

- For Scheme 1,

  - MAC CE is used as the container of inter-UE coordination information transmission.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

Draft proposal 1-2: FL understands that the necessity of adopting the following solution(s) depends on the result of draft proposal 1-1. So, after making a decision on the draft proposal 1-1, my suggestion is to adopt the necessary option(s) below accordingly.

When MAC CE is used as the container of inter-UE coordination information transmission:

Not support: Futurewei (w/ adding option of using the remaining resources outside the intersection but inside preferred resource set), Samsung, Intel (with changing the wording of “until it becomes impossible” to “until it meets pre-configured condition”) (3 companies)

- For Option A of Scheme 1 with preferred resource set,

  - S_A report from PHY layer of UE-B is the same as the outcome after Step 7) of Rel-16 TS 38.214 Section 8.1.4

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

- For Option B of Scheme 1 with preferred resource set,

  - MAC layer selects resources belonging to the received preferred resource set

When 2nd SCI is used as the container of inter-UE coordination information transmission:

Alt 1:

- For Option A of Scheme 1 with preferred resource set,

  - PHY layer at UE-B reports both the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection

  - MAC layer selects resources using S_A and the intersection set

    - MAC layer firstly selects resources for transmissions within the intersection set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

Alt 2:

- For Option A of Scheme 1 with preferred resource set,

  - PHY layer at UE-B reports both the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

- For Option B of Scheme 1 with preferred resource set,

  - PHY layer at UE-B reports the preferred resource set

  - MAC layer selects resources belonging to the received preferred resource set

Draft proposal 1-3: FL observed that a majority of companies support Alt 1. So, my suggestion is to adopt it.

Alt 1:

Support: Ericsson, DOCOMO, Huawei, Qualcomm, Samsung, NEC, Fujitsu, Fraunhofer, CMCC (w/ 2nd preference), Intel, (9 companies + 1)

- For Scheme 1 with non-preferred resource set,

  - Option 2: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

Alt 2:

Support: Futurewei, CMCC (w/ 1st preference), (2 companies)

- For Scheme 1 with non-preferred resource set,

  - Option 3: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 4) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

Draft proposal 1-4: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: Ericsson, DOCOMO, Huawei, Samsung, NEC, Fujitsu, Fraunhofer, CMCC, (8 companies)

Not support: Intel,

- For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

Draft proposal 1-5/1-6: FL observed that a majority of companies support these proposals. So, my suggestion is to adopt them.

Support: Ericsson, DOCOMO, Huawei, NEC, Fujitsu, Fraunhofer, CMCC, Intel, (8 companies)

Not support: Qualcomm, Samsung, vivo, (3 companies)

Draft proposal 1-5:

- For Condition 1-A-1 in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Starting/Ending time locations of resource selection window is provided by signaling from UE-B

    - It replaces the time interval [n+T_1,n+T_2]

Draft proposal 1-6:

- For Scheme 1, at least following parameters are provided by UE-B’s request:

  - Priority value to be used for PSCCH/PSSCH transmission

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

  - Resource reservation interval

  - Starting/Ending time locations of resource selection window

    - Scheme 2

Draft proposal 2-4: FL observed that as it would be difficult to make a concensus on this issue by the email discussion, the down-selection between two options is necessary at the GTW session.

Support Option 1: Ericsson, Qualcomm, Samsung, (3 companies)

Support Option 2: Sharp, DOCOMO, NEC, Fujitsu, OPPO, Fraunhofer, Intel, (7 companies)

Not support: Futurewei (w/ adding option of having multiple PSFCH occasions)

- Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

  - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

  - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

- Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

  - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

  - The PSFCH resources for inter-UE coordination information is at least T_proc,0 slots after a latest SCI scheduling the conflicting TB

Draft proposal 2-2: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: Ericsson, DOCOMO, Huawei, NEC, Fujitsu, Fraunhofer, Intel, (7 companies)

Not support: Futurewei,

- For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when ‘a non-destination UE of a TB transmitted by UE-B can be UE-A’ is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

  - In the above procedure, at least a destination UE of a TB transmitted by one of the paired UEs is UE-A, and the paired UEs are limited to UEs scheduling the conflicting TBs for which a PSFCH occasion for conflict indication reception has not passed.

  - Note: It is assumed that the UEs scheduling the conflicting TBs are capable of Scheme 2.

Draft proposal 2-1-1/2-1-2: FL observed that based on companies’ inputs so far, it is not possible to agree both proposals at the same time. To make progress, my suggestion is to adopt the draft proposal 2-1-1 first that is supported by the majority of companies, and then discuss further whether to adopt the draft proposal 2-1-2 within the remaining time of this meeting. If this direction fails, I would like to suggest a conclusion of “No consensus to define additional criteria to determine resource(s) where expected/potential resource conflict occurs for Condition 2-A-1 of Scheme 2”. This is because I think that we have already had enough discussion time, and having additional discussion time will not lead to the progress within this meeting.

Support Draft proposal 2-1-1: Ericsson, DOCOMO, Huawei, Samsung, Fujitsu, Fraunhofer, (6 companies)

Support Draft proposal 2-1-2: Futurewei, DOCOMO, Fujitsu, Intel, (4 companies)

Draft proposal 2-1-1:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 1’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

Draft proposal 2-1-2:

- For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

  - For the case when UE-A is a destination UE of a TB transmitted by UE-B

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

  - For the case when UE-A is a destination UE of a TB transmitted by another UE

    - Option 4’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

Draft proposal 2-3: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: Ericsson, Futurewei, DOCOMO, Qualcomm, Samsung, NEC, Fujitsu, Fraunhofer, (8 companies)

Not support: vivo, Intel, (2 companies)

- For PSFCH resource index determination in Scheme 2,

  - m_CS: 0 when UE-A determines Condition 2-A-1 or Condition 2-A-2 is satisfied

  - 2nd set of draft proposals

    - Scheme 1

Draft proposal 3-1: FL observed that a clear majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Fujitsu, Xiaomi (if this is a majority view), Qualcomm, LGE, (10 companies)

- For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

  - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

Updated draft proposal 3-2: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Fujitsu, Xiaomi, Qualcomm, LGE, (10 companies)

Not support: OPPO,

- For inter-UE coordination information triggered by an explicit request in Scheme 1,

  - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

Updated draft proposal 3-3: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Huawei, Fujitsu, Xiaomi, Qualcomm, LGE, (9 companies)

Not support: Samsung,

- For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

  - UE-A uses a resource pool used for UE-A’s inter-UE coordination information transmission to determine the set of resources

Updated draft proposal 3-5: FL formulated this proposal to cover both the camp in which the priority value of inter-UE coordination information is (pre)configured and the camp in which the priority value of inter-UE coordination information is the same as indicated by UE-B’s request. So, I think that this could be a compromise and suggest to adopt it.

Support: NEC, Ericsson, Huawei (If the proposal does not introduce new issues), Fujitsu, Xiaomi, LGE, (6 companies)

Not support: Fraunhofer (same as the value indicated by UE-B’s request), OPPO, Samsung (only with (pre)configuration), Qualcomm, (4 companies)

- For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s request.

  - Note: For the case when inter-UE coordination information is transmitted together with other data (if supported), how to set a priority value indicated by the 1st SCI is a separate issue.

Updated draft proposal 3-6: FL formulated this proposal to cover both the camp in which the priority value of request is (pre)configured and the camp in which the priority value of request is the same as that of a TB to be transmitted by UE-B. So, I think that this could be a compromise and suggest to adopt it.

Support: NEC, Ericsson, Huawei (If the proposal does not introduce new issues), Fujitsu, LGE, (5 companies)

Not support: Fraunhofer (same as the value of a TB to be transmitted by UE-B), OPPO, Samsung (only with (pre)configuration), Xiaomi (having discussion after deciding a container of request), Qualcomm, (5 companies)

- For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

  - Note: For the case when request is transmitted together with other data (if supported), how to set a priority value indicated by the 1st SCI is a separate issue.

Updated draft conclusion 3-8: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Fujitsu, (7 companies)

Not support: Xiaomi, Qualcomm, (2 companies)

- Source ID and destination ID for inter-UE coordination information and its request in Scheme 1 is up to RAN2 decision.

Updated draft proposal 3-15: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Samsung, Huawei, Fujitsu, Xiaomi, LGE, (8 companies)

Not support: Fraunhofer (fine only with 1st sbu-bullet), Qualcomm, (2 companies)

- For Scheme 1,

  - UE-A performs its resource (re)selection procedure in the same way according to Rel-16 TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - UE-B performs its resource (re)selection procedure in the same way according to Rel-16 TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B supports sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

    - Scheme 2

Draft proposal 4-1: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Xiaomi, (7 companies)

Not support: Fujitsu (only okay for UE-B), Sharp, (2 companies)

- UE-A and UE-B assumes the priority value of PSFCH transmission/reception for Scheme 2 is the same as indicated by UE-B’s SCI

Updated draft proposal 4-2: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, DOCOMO, Fraunhofer, vivo, Fujitsu, Xiaomi, LGE, (7 companies)

Not support: Samsung, Huawei (always prioritizing SL-HARQ feedback over PSFCH TX/RX for Scheme 2), Sharp, (3 companies)

- When PSFCH TX/RX for Scheme 2 is overlapping with PSFCH TX/RX for SL HARQ-ACK feedback in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.2

  - (Pre)configuration can enable that PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for Scheme 2

Draft proposal 4-3: FL observed that a clear majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Fujitsu, Xiaomi, LGE, (9 companies)

- When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

Draft conclusion 4-4: FL observed that a clear majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Samsung, Huawei, Fujitsu, Xiaomi, Qualcomm, LGE, (10 companies)

- For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

  - Period of PSFCH resources (sl-PSFCH-Period)

  - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

  - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

Draft proposal 4-6: FL observed that a majority of companies support this proposal. So, my suggestion is to adopt it.

Support: NEC, Ericsson, DOCOMO, Fraunhofer, Huawei, Fujitsu, Xiaomi, Qualcomm, LGE, (9 companies)

Not support: Samsung,

- For Scheme 2, PRB and m_0 for PSFCH transmission/reception is derived by PSFCH resource index in the same way according to Rel-16 TS 38.213 Section 16.3

- Summary of contributions

- Details on supported conditions to determine inter-UE coordination information

  - Condition 1-A-1

    - If inter-UE coordination information is triggered by an explicit request

      - Signaling from UE-B which provides prio_TX, L_subCH, P_rsvp_TX

        - UE-B’s explicit request for inter-UE cooridnatoin information

          - Supported by [Huawei,1] [vivo,4] [CATT,7] [OPPO,8] [Zhejiang Lab,9] [Fraunhofer,14] [ZTE,16] [NEC,17] [Samsung,18] [LGE,19] [InterDigital,20] [Apple,22] (12)

      - Additional parameters provided by UE-B’s expclicit request

        - Starting and/or ending time position of resource selection window

          - Supported by [Huawei,1] [CATT,7] [OPPO,8] [Intel,11] [Xiaomi,12] [CMCC,13] [LGE,19] [Apple,22] [Sharp,25] [Lenovo,29] (10)

            - With respect to the time location of UE-B’s request signaling [OPPO,8] [LGE,19] (2)

        - Resource set type

          - Supported by [Nokia,2] [CATT,7] [ZTE,16] [InterDigital,20] [Apple,22] [Convida,26] [Lenovo,29] (7)

        - Remaining PDB (UE-A determines resource selection window)

          - Supported by [vivo,4] [CMCC,13](if resource selection window is absent) [Fraunhofer,14] [ZTE,16] [NEC,17] [Samsung,18] (6)

        - C_resel

          - Supported by [OPPO,8] [LGE,19] (2)

        - Message size

          - Supported by [Nokia,2]

        - Number of resoruces to be reported

          - Supported by [Nokia,2]

        - X%

          - Supported by [Fujitsu,6]

        - Resoruces to be used for inter-UE coordination information signaling

          - Supported by [Fujitsu,6]

        - Number of time resrouces for a TB [Apple,22]

    - If inter-UE coordination information is triggered by a condition rather than request reception

      - Setting of prio_TX, L_subCH, P_rsvp_TX, resource selection window

        - Indicated by a (pre)configuration

          - Supported by [Futurewei,3] [vivo,4] [Intel,11](except for resource selection window) [LGE,19] [Lenovo,29] (5)

        - Indicated by UE-B’s prior SCI

          - Supported by [Futurewei,3] [OPPO,8] [Fraunhofer,14] (3)

        - Determined by UE-A’s implementation, and they are included in UE-A’s inter-UE coordination information

          - Supported by [Huawei,1] [vivo,4] [Intel,11](resrouce selection window) (3)

        - Indicated by PC5-RRC

          - Supported by [Futurewei,3]

    - RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Sectoin 8.1.4

      - Supported by [CATT,7] [Samsung,18] [LGE,19] [Apple,22] [Ericsson,32] (5)

      - Objected by [ZTE,16]

  - Condition 1-A-2

    - Removal of the possibility of disabling this feature

      - Supported by [CATT,7] [Lenovo,29] (2)

    - Consider it in Step 6) of TS 38.214 Section 8.1.4

      - Supported by [LGE,19]

  - Condition 1-B-1

    - Support Option 1 only: [Spreadtrum,5] [Fraunhofer,14] [ZTE,16] [InterDigital,20] (4)

    - Removal of RSRP part in option 2: [CATT,7]

    - Support both with separate labeling: [LGE,19]

    - Add “measured RSRP of UE-B’s reserved reserouces in option 2: [MediaTek,31]

  - Condition 1-B-2

    - Removal of “due to half duplex operation”: [vivo,4]

  - Condition 2-A-1

    - Additional criteria

      - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

        - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

        - Strive to reuse Rel-16 specification wherever possible

        - Supported by [Huawei,1] [Spreadtrum,5] [CATT,7] [Fraunhofer,14] [ZTE,16] [Samsung,18] [LGE,19] [Panasonic,23] [ETRI,24] [Convida,26] [Ericsson,32] (11)

          - For the case when UE-A is a destination UE of UE-B’s TB [LGE,19] [Panasonic,23] (2)

          - UE expects symmetric configuration of RSRP thresholds based on priorities [Ericssson,32]

      - Option 1’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource(s) is larger than a RSRP threshold according to the priorities included in the SCI:

        - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

        - Supported by [LGE,19] [Panasonic,23] (2)

          - For the case when UE-A is a destination UE of other UE’s TB [LGE,19] [Panasonic,23] (2)

      - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - FFS: Whether the threshold depends on priority

        - Supported by [Futurewei,3] [Intel,11] [Qualcomm,30] [MediaTek,31] (4)

          - For the case when UE-A is a destination UE of other UE’s TB [Futurewei,3]

      - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

        - Supported by [Intel,11] [Fraunhofer,14] [Panasonic,23] (3)

      - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - FFS: Whether the threshold depends on priority

        - Supported by [Nokia,2] [Futurewei,3] [LGE,19] [InterDigital,20] [DCM,28] (5)

          - For the case when UE-A is a destination UE of UE-B’s TB [Nokia,2] [Futurewei,3] [LGE,19] [DCM,28] (4)

      - Option 4’: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resoruce is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of other UE’s reserved resource.

        - FFS: Whether the threshold depends on priority

        - Supported by [Nokia,2] [Futurewei,3] [LGE,19] [DCM,28] (4)

          - For the case when UE-A is a destination UE of other UE’s TB [Nokia,2] [Futurewei,3] [LGE,19] [DCM,28] (4)

      - Alt 3: No additional criteria is supported

        - Supported by [OPPO,8] [NEC,17] (2)

    - UE-B selection

      - UE-B is a UE transiting a TB with lower or equal priority value among the conflicting TBs

        - Supported by [Intel,11] [Samsung,18] [LGE,19] [DCM,28] [Qualcomm,30] (5)

          - Further consider the arrival time of reservation [Qualcomm,30]

  - Condition 2-A-2

    - UE-A does not indicate a resource conflict if UE-B’s transmission has higher priority than UE-A’s [Nokia,2]

  - How to align TX resource pool between UE-A and UE-B

    - In scheme 1

      - If inter-UE coordination information is triggered by an explicit request

        - TX resource set where a slot of UE-A’s inter-UE coordination information singaling belongs to

          - Supported by [OPPO,8] [Zhejiang Lab,9] (2)

        - TX resource pool ID is indicated by UE-B’s request

          - Supported by [Intel,11] [ZTE,16] (2)

        - UE-B’s Tx/Rx resource pool is assumed to be aligned with UE-A’s Tx/Rx resource pool via implementation

          - Supported by [Huawei,1]

        - TX resource set where a slot of UE-B’s explicit request singaling belongs to

          - Supported by [LGE,19]

      - If inter-UE coordination information is triggered by a condition rather than request reception

        - TX resource set where a slot of UE-A’s inter-UE coordination information singaling belongs to

          - Supported by [OPPO,8] [Zhejiang Lab,9] [Intel,11] [LGE,19] (4)

        - UE-B’s Tx/Rx resource pool is assumed to be aligned with UE-A’s Tx/Rx resource pool via implementation

          - Supported by [Huawei,1]

        - TX resource pool ID indicated by UE-A’s inter-UE coordination information

          - Supported by [Intel,11]

    - In scheme 2

      - UE-B’s Tx/Rx resource pool is assumed to be aligned with UE-A’s Tx/Rx resource pool via implementation

        - Supported by [Huawei,1] [LGE,19] (2)

      - UE-A receives other UE’s SCI and UE-B’s SCI in slot(s) belonging to the same TX resource pool

        - Supported by [LGE,19]

- Contents of inter-UE coordination information and its request

  - Contents of the inter-UE coordination information in Scheme 1

    - Set of resources

      - Indication mechanism

        - Option 1: N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification:

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

          - FFS: Value of N.

          - Supported by [Fraunhofer,14] [LGE,19] [Apple,22] [ETRI,24](optional) [Sharp,25] [DCM,28] [Qualcomm,30] (7)

            - Removal of indicating number of sub-channels [Sharp,25]

        - Option 2: Bitmap indication where each bit indicates whether a pair of sub-channel(s) and slot(s) is included in inter-UE coordination information

          - FFS: Granularity in time-and-frequency resources

          - FFS: other information (if any) e.g. periodicity

          - Supported by [CATT,7] [OPPO,8] [Zhejiang Lab,9] [Fraunhofer,14] [Samsung,18] [Apple,22](for non-preferred resource set) [ETRI,24](1st priority) [Sharp,25] (8)

        - Option 3: Reuse a single combination of TRIV and FRIV as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification:

          - For TRIV, window size of 32 slots is replaced with the value corresponding to the resource selection window

          - For FRIV, only combinations of starting sub-channels are indicated

          - For a pair of TRIV and FRIV, more than 2 additional resources can be indicated

          - Supported by [Nokia,2]

        - Option 4: 2-dimensional resource indicator value

          - Each value is associated with a pair of sub-channel(s) and slot(s) is included in inter-UE coordination information

          - Supported by [Huawei,1] [Nokia,2] (2)

            - Not allowing resoruces overlapping in time [Huawei,1]

            - Allowing resoruces overlapping in time [Nokia,2]

    - Identifier to identify a UE receiving this coordination information [Huawei,1] [Fujitsu,6]  [Intel,11] [Samsung,18] [LGE,19] (5)

    - Identifier to identify a UE transmitting this coordination information [Huawei,1] [Fujitsu,6] [Samsung,18] [LGE,19] (4)

    - Resource set type [Huawei,1] [Zhejiang Lab,9] [Fraunhofer,14] [LGE,19] (4)

    - TX parameters to be used to determine the set of resoruces

      - TX Priority [Huawei,1] [InterDigital,20] [ASUSTek,27] (3)

      - Resoruce selection window [Huawei,1] [Intel,11] [InterDigital,20] (3)

      - Number of sub-channels [Huawei,1] [InterDigital,20] (2)

      - Resource reservation period [Huawei,1] [InterDigital,20] (2)

    - Condition type for non-preferred resource set [Intel,11] [LGE,19] (2)

    - RSRP measurement of reserved resources [Apple,22] [ASUSTek,27] [Ericsson,32] (3)

    - RX priority value of non-preferred resources [LGE,19] [Apple,22] (2)

    - Zone ID and communication range requirement [Samsung,18] [InterDigital,20] (2)

    - ID(s) of UE-B’s intended receiver(s) [Nokia,2]

    - ID(s) used by UE-B [Nokia,2]

    - Source ID of other UE’s reserved resources [Intel,11]

    - Feedback timestamp [Intel,11]

    - Indicator to indicate whether coordination information is assistance type or scheduling type [Convida,26]

- Container  of inter-UE coordination information and its request

  - Container of the inter-UE coordination information in Scheme 1

    - SCI format 1-A [Futurwei,3] [vivo,4](non-preferred resource set) [Fujitsu,6] [CMCC,13] [Sharp,25] [MediaTek,31] (6)

      - Stand-alone PSCCH [Futurewei,3]

    - New 2nd-stage SCI format  [Huawei,1] [Futurewei,3] [vivo,4](preferred resource set) [Spreadtrum,5] [Fujitsu,6] [CATT,7] [OPPO,8] [Zhejiang Lab,9] [Sony,10] [Xiaomi,12] [CMCC,13] [Fraunhofer,14] [Samsung,18] [Mitsubishi,21] [Apple,22] (for preferred resource set) [Qualcomm,30](for preferred resource set) [MediaTek,31] (17)

      - Possibility of having 2nd SCI without TB scheduling  [Huawei,1] [Futurewei,3] [Sony,10] [Xiaomi,12] [Fraunhofer,14] [Samsung,18] (6)

      - with scheduling TB containing remaining L2 source/destination ID [LGE,19]

      - Keep Rel-16 SCI format size budget [LGE,19]

    - MAC CE [vivo,4](preferred resource set, for Condition 1-B-1 optoin 2) [Spreadtrum,5] [Fujitsu,6]  [Intel,11] [ZTE,16] [LGE,19] [InterDigital,20] [Mitsubishi,21] [Apple,22](for non-preferred resource set) [Panasonic,23] [DCM,28] [Qualcomm,30](for non-preferred resource set) [MediaTek,31] [Ericsson,32] (14)

      - With the possibility of multiplexing with other data [Intel,11] [LGE,19] [Qualcomm,30] (3)

        - Destination ID are always the same [LGE,19]

        - Destination ID can be different [Intel,11]

      - Without multiplexing with other data [Futurewei,3]

    - PC5-RRC [Intel,11] [InterDigital,20] [MediaTek,31] [Ericsson,32] (4)

  - Container of the explicit request in Scheme 1

    - New 2nd-stage SCI format [Huawei,1] [Nokia,2] [Futurewei,3] [vivo,4] [Spreadtrum,5] [Fujitsu,6] [CATT,7] [Zhejiang Lab,9] [Sony,10] [Xiaomi,12] [CMCC,13] [CAICT,15] [Samsung,18] [Apple,22] [Lenovo,29] (15)

      - without TB scheduling [Huawei,1] [Samsung,18] (2)

    - MAC CE [Nokia,2] [vivo,4] [Fujitsu,6] [Sony,10] [Fraunhofer,14] [ZTE,16] [LGE,19] [Apple,22] [Panasonic,23] [Lenovo,29] (10)

      - With the possibility of multiplexing with data [Intel,11] [LGE,19] (2)

        - Destination ID are always the same [LGE,19]

        - Destination ID can be different [Intel,11]

    - PC5-RRC [Qualcomm,30]

    - PSFCH [MediaTek,31]

  - Other details for scheme 1

    - Inter-UE coordination triggered by UE-B’s request

      - Cast type

        - Unicast [Huawei,1] [Nokia,2] [Futurewei,3] [Spreadtrum,5] [CATT,7] [Fraunhofer,14] [Mitsubishi,21] [Panasonic,23] [Ericsson,32] (9)

        - Groupcast [Nokia,2] [Fraunhofer,14] (2)

      - Source ID

        - Destination ID of UE-B’s request signaling [LGE,19]

      - Destinatoin ID

        - Source ID of UE-B’s request signaling [CATT,7] [Intel,11] [LGE,19] (3)

        - Broadcast destination ID [Intel,11](standalone mode)

        - Groupcast desitnation ID of UE-B’s transmission [Intel,11]

      - Priority value

        - Indicated by UE-B’s request [Intel,11]

        - (pre)configured [LGE,19]

    - Request signaling

      - Cast type

        - Unicast [Huawei,1] [Nokia,2] [Futurewei,3] [Intel,11] [Fraunhofer,14] [Mitsubishi,21] [Panasonic,23] [Ericsson,32] (8)

        - Groupcast [Nokia,2] [Futurewei,3] [Fraunhofer,14] [Mitsubishi,21] (4)

      - Source ID

        - Source ID to be used for UE-B’s transmission [LGE,19]

      - Destinatoin ID

        - Destinatoin ID to be used for UE-B’s transmission [LGE,19]

      - Priority value

        - (pre)configured [LGE,19]

    - Inter-UE coordination triggered by a condition rather than request reception

      - Cast type

        - Unicast [Huawei,1] [Futurewei,3] [Spreadtrum,5] [Mitsubishi,21] [Ericsson,32] (5)

        - Groupcast [Futurewei,3] [Panasonic,23] (2)

        - Broadcast [Panasonic,23]

      - Source ID

        - Target destination ID to be used for UE-B’s transmission [LGE,19]

      - Destinatoin ID

        - Broadcast destination ID [Intel,11]

        - (pre)configured [LGE,19]

      - Priority value

        - (Pre)configured value [Intel,11] [LGE,19] (2)

        - Highest priority between multiplexed data and inter-UE coordination information [Intel,11]

  - Container of the inter-UE coordination in Scheme 2

    - PSFCH resource set

      - Period of PSFCH resources (sl-PSFCH-Period)

        - Indicated by a separate (pre)configuration

          - Supported by [Zhejiang Lab,9] [Intel,11] (2)

        - Same as that of SL HARQ-ACK feedback in the same resource pool

          - Supported by [Qualcomm,30]

      - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

        - Indicated by a separate (pre)configuration

          - Supported by [CATT,7] [Zhejiang Lab,9] (2)

        - Same as that of SL HARQ-ACK feedback in the same resource pool

          - Supported by

      - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

        - Indicated by a separate (pre)configuration

          - Supported by [CATT,7] [Zhejiang Lab,9] (2)

        - Starting sub-channel index

          - Supported by [Intel,11]

        - Same as that of SL HARQ-ACK feedback in the same resource pool

          - Supported by

      - Scrambling ID for sequence hopping of PSFCH (sl-PSFCH-HopID)

        - Indicated by a separate (pre)configuration

          - Supported by [Intel,11]

        - Same as that of SL HARQ-ACK feedback in the same resource pool

          - Supported by [Qualcomm,30]

    - Prioritization rule

      - PSFCH TX/TX and TX/RX prioritization rule

        - Based on highest priority of conflicting TBs [Fujitsu,6] [Lenovo,29] (2)

        - PSFCH for SL HARQ-ACK feedback is prioritized over PSFCH for Scheme 2 [vivo,4] [Ericsson,32] (2)

      - PSFCH and UL/LTE SL prioritization rule

        - Based on highest priority of conflicting TBs [vivo,4] [Fujitsu,6] [Lenovo,29] (3)

    - PSFCH resource determination

      - Timing of the PSFCH transmission

        - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

          - Supported by [Huawei,1] [CATT,7] [Xiaomi,12] [ZTE,16] [Samsung,18] [Apple,22] [Convida,26] [Qualcomm,30] [Ericsson,32] (9)

        - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

          - Supported by [Futurewei,3] [vivo,4] [OPPO,8] [Intel,11] [LGE,19] [InterDigital,20] [Panasonic,23] [ETRI,24] [ASUSTek,27] [DCM,28] (10)

            - Latest PSFCH slot for Scheme 2 T_proc,x slots before the resource with resource conflict [Futurewei,3] [vivo,4] [Intel,11] [LGE,19] [DCM,28] (5)

              - T_proc,x is a function of T_3 and/or T_prep [vivo,4] [Intel,11] [LGE,19] (3)

      - Frequency and code domain resources derived by

        - m_CS

          - Option 1:

            - 0

            - Supported by [ZTE,16] [Samsung,18] [Apple,22] [Sharp,25] [Ericsson,32] (5)

          - Option 2:

            - 0 for Condition 2-A-1,

            - 6 for Condition 2-A-2

            - Supported by [Nokia,2] [Futurewei,3] [vivo,4](for option 2) [Spreadtrum,5] [CATT,7] [Intel,11] [LGE,19] [InterDigital,20] [DCM,28] (9)

          - Option 3:

            - 0 for 2nd reserved resource,

            - 6 for 3rd reserved resource

            - Supported by [vivo,4](for option 1) [Qualcomm,30] (2)

          - Option 4:

            - 0 for 2nd reserved resoruce

            - 2 for 3rd reserved resoruce

            - 4 for both 2nd and 3rd reserved resource

            - 6 for the case when no UE transmitted SCI with periodic reservation on the non-moniotred slot of UE-B

            - 8 for Condition 2-A-2

            - Supported by [Huawei,1]

          - Optoin 5:

            - (pre)configured

            - Supported by [Panasonic,23]

        - m_0 determination based on PSFCH resource index

          - In the same way as specified in TS 38.213 Section 16.3

            - Supported by [LGE,19] [DCM,28] [Ericsson,32] (3)

          - Update to indicate the time location of expected/potential resrouce conflict

            - Supported by [CATT,7] [Samsung,18] (2)

  - Dedicated resource pool is (pre)configured for inter-UE coordination information transmission in Scheme 1

    - Supported by [Nokia,2] [Qualcomm,30] (2)

- Details on how UE-B uses or skip the received inter-UE coordination in its resource (re)selection

  - Scheme 1 with preferred resource set Option A

    - Option 1-1:

      - Physical layer at UE-B reports both the intersection set between the preferred resource set  and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection

      - Higher layer at UE-B first uses the candidate single-slot resource(s) belonging to the intersection set, and then further uses the remaining S_A outside the intersection in its resource (re-)selection if necessary

      - Supported by [Huawei,1] [vivo,4] [Apple,22] [DCM,28] [Ericsson,32] (5)

        - [DCM,28]: If MAC CE is used as a container, physical layer at UE-B reports only S_A.

    - Option 1-2:

      - Physical layer at UE-B reports both the preferred resource set (if necessary) and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 to higher layer for its resource (re-)selection

      - Higher layer at UE-B first uses the candidate single-slot resource(s) belonging to the intersection set between the preferred resource set and S_A, and then it is up to UE-B’s implementation to further uses the remaining S_A or remaining preferred resources outside the intersection in its resource (re-)selection if necessary

      - Supported by [Futurewei,3] [CMCC,13] (2)

    - Option 2:

      - If the number of candidate single-slot resources belonging to the intersection between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 satisfies the requirement of X⋅M_"total" as specified in Step 7) of TS 38.214 Section 8.1.4

        - Physical layer at UE-B reports the intersection set instead of S_A to higher layer for its resource (re-)selection

      - Otherwise,

        - Physical layer at UE-B reports the set(s) determined by the intersection set as defined above and the S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 to higher layer for its resource (re-)selection.

          - FFS: how to determine the set(s) based on the intersection set and S_A

      - Supported by [Fujitsu,6] [Intel,11] [Xiaomi,12] [CMCC,13] [Fraunhofer,14] [Samsung,18] [ETRI,24] (7)

        - The set is the intersection set plus preferred resources excluded in Step 5) [Fujitsu,6] [CMCC,13] [Samsung,18] (3)

        - The set is all or a subset of S_A [Intel,11] [Xiaomi,12] [Fraunhofer,14] (3)

    - Other option

      - [CATT,7]: Apply the set of resrouces in Step 4)

  - Scheme 1 with preferred resource set Option B

    - Applicable scenario

      - It is applied if UE-B’s transmission is on a TX resource pool (pre)configured with random selection only

        - Supported by [OPPO,8] [LGE,19] [DCM,28] (3)

      - It is applied if UE-B is not capable of performing sensing

        - Supported by [vivo,4] [DCM,28] [Ericsson,32] (3)

      - UE-B selects no to perform sensing [vivo,4] [Fraunhofer,14] (2)

      - It is applied up to UE-B’s implementation [Lenovo,29]

    - Alt 1:

      - Physical layer at UE-B reports preferred resource set to higher layer for its resource (re-)selection

      - Higher layer at UE-B uses the candidate single-slot resource(s) belonging to the preferred resource set in its resource (re-)selection

      - Supported by [Huawei,1] [LGE,19] (2)

    - Alt 2:

      - If the number of candidate single-slot resources belonging to the preferred resource set satisfies the requirement of X⋅M_"total"   as specified in Step 7) of TS 38.214 Section 8.1.4

        - Physical layer at UE-B reports the preferred resource set to higher layer for its resource (re-)selection

      - Otherwise,

        - Physical layer at UE-B reports the S_A obtained after Step 4) of Rel-16 TS 38.214 Section 8.1.4 to higher layer for its resource (re-)selection.

      - Supported by [Intel,11]

    - Alt 3:

      - If the number of candidate single-slot resources belonging to the preferred resource set satisfies the requirement of X⋅M_"total"   as specified in Step 7) of TS 38.214 Section 8.1.4

        - Physical layer at UE-B reports the preferred resource set to higher layer for its resource (re-)selection

      - Otherwise,

        - Physical layer at UE-B reports both the preferred resource set and the S_A obtained after Step 4) of Rel-16 TS 38.214 Section 8.1.4 to higher layer for its resource (re-)selection.

        - Higher layer at UE-B first uses the candidate single-slot resource(s) belonging to the preferred resource set, and then further uses the remaining S_A in its resource (re-)selection if necessary

      - Supported by [Apple,22]

  - Scheme 1 with non-preferred resource set

    - Option 1: Physical layer at UE-B excludes candidate single-slot resource(s) overlapping with the non-preferred resource set from S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4. It reports the updated S_A to higher layer for its resource (re)selection.

      - Supported by [Huawei,1] [Fujitsu,6] [Xiaomi,12] (3)

    - Option 2: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

      - Supported by [vivo,4] [Fujitsu,6] [Fraunhofer,14] [Samsung,18] [LGE,19] [Apple,22] [ETRI,24] [DCM,28] [Qualcomm,30] [Ericsson,32] (10)

    - Option 3: Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 4) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

      - Supported by [Futurewei,3] [CATT,7] [Xiaomi,12] [CMCC,13] [Fraunhofer,14] (5)

    - M_total update

      - Number of candidate single-slot resoruces overlapping with non-prefered resources is not counted for M_total [vivo,4] [CATT,7] [CMCC,13] [Qualcomm,30] (4)

      - Number of candidate single-slot resoruces overlapping with non-prefered resources associated with Condition 1-B-2 is not counted for M_total [LGE,19]

  - Validity check for scheme 1

    - Based on priority level used for determining the set of resrouces [Intel,11] [Fraunhofer,14] [LGE,19](for Condition 1-B-1 with option 2) [InterDigital,20] [Lenovo,29] (5)

    - Based on radio/geographical distance [Intel,11] [Fraunhofer,14] [Samsung,18] [Mitsubishi,21] (4)

    - Destinatoin ID of inter-UE coordination information [Fraunhofer,14] [LGE,19] (2)

    - Source ID of inter-UE coordination information [Intel,11] [Samsung,18] (2)

    - Aging time condition expires [Intel,11]

    - Remaining PDB is smaller than a threshold [Apple,22]

  - Scheme 2

    - Condition when UE-B does not perform re-selection upon the reception of the inter-UE coordination

      - Priority of UE-B’s transmission is high [Nokia,2] [Fraunhofer,14] (2)

      - For Condition 2-A-2, the destination of a PSCCH/PSSCH to be transmitted by UE-B is not UE-A [Fraunhofer,14] [LGE,19] (2)

      - Time gap between UE-B’s SCI and its reserved resource is smaller than a threshold [CATT,7] [Apple,22] (2)

      - Remaining PDB of UE-B’s transmission is smaller than a threshold [Nokia,2]

      - UE-B reselects its reserved resource where resource conflict occurs [Fujitsu,6]

      - There is not enough resources for its transmission [Ericsson,32]

    - Skip Step 5) if UE-A informs that there is periodic reservation from other UEs on non-monitored slots of UE-B [Huawei,1]

    - UE-B excludes the resources corresponding to the collision indication in its resource reselection [vivo,4] [LGE,19] [DCM,28] (3)

- Details on a condition to trigger inter-UE coordination information

  - Condition(s) when UE-A transmits the inter-UE coordination information to UE-B in Scheme 1

    - Potential/expected resource conflict is detected on the resources reserved by UE-B  [Futurewei,3] [OPPO,8] [Xiaomi,12] [Fraunhofer,14] [Mitsubishi,21] (5)

    - Up to UE’s implementation [Futurewei,3] [LGE,19] [Apple,22] [DCM,28] (4)

    - Reception at an intended destination UE of an SCI indicating reserved resources for its reception [Nokia,2] [Qualcomm,30] (2)

    - UE-A completes its resource selection [vivo,4] [Qualcomm,30] (2)

    - UE has data to UE-B which is multiplexed with feedback payload [Intel,11] [DCM,28] (2)

    - Number of failure of TB decoding at UE-A side is larger than a threshold [Mitsubishi,21] [Lenovo,29] (2)

    - Change in resource to be sent via inter-UE coordination [Nokia,2]

    - UE-A transmits resource reservation signaling [vivo,4]

    - Feedback was not transmitted for a certain amount of time [Intel,11]

    - Distance between UE-A and UE-B is smaller than a threshold [Xiaomi,12]

    - Distance between UE-A and UE-B is larger than a threshold [Xiaomi,12]

    - CBR is higher than a threshold [Apple,22]

    - CBR is lower than (pre)configured threshold [DCM,28]

    - UE-A detects a resource re-selection is to be performed by UE-B [Ericsson,32]

  - Condition(s) when UE-B transmits the request for the inter-UE coordination information in Scheme 1

    - Up to UE’s implementation [CATT,7] [ZTE,16] [LGE,19] [DCM,28] (4)

    - Resource re-selection is triggered by UE-B [OPPO,8] [Intel,11] [Xiaomi,12] (3)

    - Resource re-selection is expected to be performed by UE-B  [Intel,11] [Ericsson,32] (2)

    - UE-B has data/TB for transmission that can be multiplexed with request to UE-A [Intel,11] [DCM,28] (2)

    - TB arrives at physical layer of UE-B [vivo,4]

    - Priority index of UE-B’s packet is smaller than a threshold [OPPO,8]

    - Remainig PDB of UE-B’s packet is larger than a threshold [OPPO,8]

    - Number of resoruces within the set S_A is larger than a threshold [OPPO,8]

    - UE-B does not have valid inter-UE coordination information [Intel,11]

    - Elapsed time from the previous inter-UE coordination feedback request exceeds pre-configured value [Intel,11]

    - UE-B supports inter-UE coordination scheme 1 [Intel, 11]

    - UE-B does not have valid inter-UE coordination information from nay destination UE [Intel,11]

  - whether/how UE-A knows that UE-B is capable of receiving inter-UE coordination information and taking into account it in its resource re-selection

    - Option 1: UE-B’s SCI indicates whether UE-B has such a capability or not [CATT,7] [OPPO,8] [Intel,11] [InterDigital,20] [Apple,22] (5)

    - Option 2: UE-A is provided with whether UE-B has such a capability or not via PC5-RRC signaling. [CAICT,15]

- Details on a (pre)configuration to enable or disable or control feature of the inter-UE coordination

  - Alt 1: [Qualcomm,30]

    - Scheme 1 with preferred-resource indication

    - Scheme 1 with non-preferred resource indication

    - Scheme 2

  - Alt 2: [CATT,7]

    - Scheme 1 with explicit request-based manner

    - Scheme 1 with condition-based manner

    - Scheme 2

  - Alt 3: [ZTE,16]

    - Scheme 1

    - Scheme 2

  - Alt 4: [LGE,19]

    - Scheme 1

      - Preferred resource vs non-preferred resource

      - Expclicit request-based manner vs condition-based manner

    - Scheme 2

      - Whehter UE-A is non-destinatoin of a TB transmitted by UE-B or not

- Others

  - Futher consideration on skipping inter-UE coordination information signaling when a condition is met [vivo,4] [Fujitsu,6] [Apple,22]

  - Further consideration on specifying details for Condition 1-A-2/1-B-2/2-A-2 [vivo,4] [DCM,28] [Qualcomm,30]

  - Further consideration on supporting additional feature or condition of determing the set of resources [Nokia,2] [CATT,7] [OPPO,8] [Intel,11]

  - Further consideration on restricting combinations of features to be supported in Scheme 1 [Fujitsu,6] [CATT,7] [Xiaomi,12] [Fraunhofer,14] [ZTE,16] [Samsung,18] [LGE,19] [Apple,22] [DCM,28] [Qualcomm,30]

  - Further consideration on additional criteria for determining the set of resoruces in Scheme 1 based on RSRP distance between UE-A and UE-B [Fujitsu,6] [Intel,11] [Ericsson,32]

  - Further restrict or expand on the condition to be UE-A and/or UE-B [Huawei,1] [Nokia,2] [vivo,4] [Sony,10] [Fraunhofer,14] [ZTE,16] [Samsung,18] [InterDigital,20] [Mitsubishi,21] [DCM,28] [Qualcomm,30] [Ericsson,32]

  - Furuther consideration on latency bound for the inter-UE coordination information singaling [CATT,7] [Samsung,18] [Lenovo,29]

  - Further consideration of repetition of inter-UE coordination information signaling [BOSCH,33]

- Reference

- R1-2110845	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2110862	Inter-UE coordination for Mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2110887	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2111037	Remaining issues on mode-2 enhancements	vivo

- R1-2111112	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2111151	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2111229	Remaining issues  on Inter-UE coordination for Mode 2 enhancements	CATT, GOHIGH

- R1-2111301	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2111354	Inter-UE coordination for mode 2 enhancements	Zhejiang Lab

- R1-2111407	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

- R1-2111515	Design of Inter-UE Coordination Solutions for Sidelink Communication	Intel Corporation

- R1-2111547	Discussion on inter-UE coordination	Xiaomi

- R1-2111626	Discussion on inter-UE coordination for mode 2 enhancement	CMCC

- R1-2111650	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2111657	Considerations on mode 2 enhancements	CAICT

- R1-2111667	Discussion on inter-UE coordination	ZTE

- R1-2111700	Discussion on mode 2 enhencements	NEC

- R1-2111759	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2111816	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2111825	On inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

- R1-2111827	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2111895	Inter-UE Coordination	Apple

- R1-2111967	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2111998	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

- R1-2112025	Discussion on inter-UE coordination for mode 2 enhancements	Sharp

- R1-2112034	Inter-UE Coordination for NR SL Mode 2 Enhancement 	Convida Wireless

- R1-2112043	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2112127	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2112165	Inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2112238	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2112318	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2112352	Details on mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2112396	Remaining details on mode 2 inter-UE coordination	ROBERT BOSCH GmbH

- Appendix

  - Conclusions made in RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

  - Conclusions made in RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

  - Agreements made in RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Agreements made in RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

  - Agreements made in RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured