3GPP TSG RAN WG1 #107-e                                                                  R1-2112730

e-Meeting, November 11th – 19th, 2021

Agenda item:	7.2.4

Source:	Moderator (Sharp)

Title:	Summary of [107-e-NR-5G_V2X-06]

Document for:	Discussion and Decision

Introduction

The document is to collect companies’ views and provide a summary for the email discussion thread:

[107-e-NR-5G_V2X-06] Discussion on the alignment of priority values in the specifications (R1-2111298, R1-2112010) by Nov 16.

Discussion

1st change in R1-2111298

In the power control procedure for PSFCH transmission, i.e. 16.2.3 in TS38.213, it was specified that “UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order…”. The intention is to select PSFCH transmission(s) with higher priority(s) and it is noted that priority with value ‘1’ is the highest priority. As pointed out in [1], the UE should select PSFCH transmission(s) based on the decreasing priority order instead, i.e. ascending order of corresponding priority field values. Hence, the following change was proposed,

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥1and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

Round#1 discussion

Please provide your views on the change in the table below.

Question 1-1: Do you agree that the change should be adopted? If no, please provide the reasons and suggestions, if any.

| Company | Views |
| --- | --- |
| Intel | Agree |
| NTT DOCOMO | OK |
| LGE | OK |
| CATT,GOHIGH | Agree |
| OPPO | Agree |
| Vivo | Fine with the intention, but please note that the meaning of “priority field value” and “priority” is also confusing and being discussed in the next section. We prefer not to use the “priority filed value”. Instead, considering that “priority value” is already used for sidelink, such as 16.2.4, we propose to modify as below (one example): PSFCH transmissions with ascending priority order of priority value as described in clause 16.2.4.2 |
| Samsung | Agree |
| Nokia, NSB | Agree, but prefer vivo’s proposal of using “priority value” |
| Huawei, HiSilicon | We think this is not an essential issue, current wording seems not introduce ambiguity on UE implementation. |
| Ericsson | Agree |
| Lenovo/Motorola Mobility | Agree |
| Qualcomm | OK |
| ZTE,Sanechips | OK |

Round#2 discussion

Based on the input from companies in the 1st round, it was clear majority of views that some updates of specs are needed for clause 16.2.3 of TS38.213.

Regarding suggestion from vivo and Nokia, in moderator’s views, with the aligned understanding of priority values in the following section 2.2, either “priority field value” or “priority value” is fine. It is noted that the related specs refer to clause 16.2.4.2, where “priority field value” is used for determination of NTx,PSFCH.

#### 16.2.4.2 Simultaneous PSFCH transmission/reception […] If a UE would transmit Nsch,Tx,PSFCH PSFCHs in a PSFCH transmission occasion, the UE transmits NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values indicated in all SCI formats 1-A associated with the PSFCH transmission occasion.

Furthermore, in current clause 16.2.3, it is noted that “priority field value” is used already, shown as follows.

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as […] - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise […]

Accordingly, the moderator suggests to adopt the change from [1].

Proposal 1: Adopt the 1st change in R1-2111298.

Question 1-2: Do you agree with Proposal 1 above and why?

| Company | Views |
| --- | --- |
| Apple | Agree |
| Sharp | Agree |
| LGE | OK |
| OPPO | Agree |
| CATT,GOHIGH | Agree |
| NTT DOCOMO | OK |
| Samsung | Agree |
| vivo | We also agree that with the alignment in 2.2, there may not be big difference between “priority field value” and “priority value”. But it may be confusing if the spec using different terms in different places. If 2.2 is agreed, is it better to have a common word by clearing the spec? |
| Ericsson | Agree with Proposal 1. It provides a better understanding of the text. |
| Nokia, NSB | Agree. Otherwise the ordering is unclear |

2nd change in R1-2111298, change in R1-2112010

As discussed in [1] and [2], the value of “Priority” field with 3bits in SCI format 1-A ranges from 0 to 7, while they hold different opinions on the range of the priority value, which is widely used throughout TS38.213 and TS38.214 for NR V2X.

It is noted that a clear mapping between “Priority” field and priority value is specified for LTE V2X in 14.2.1 of TS36.213. The related specs are hereby copied as follows,

- 14.2.1 UE procedure for transmitting the PSCCH […] - The UE shall set the contents of the SCI format 1 as follows: - the UE shall set the Modulation and coding scheme as indicated by higher layers. - the UE shall set the "Priority" field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block. Priority field ‘000’ corresponds to priority ‘1’, priority field ‘001’ corresponds to priority ‘2’, and so on. […]

Specifically, in [1], it is stated that priority value in physical layer, as indicated in SCI format 1-A, starts from 0. When UE determines NTx,PSFCH for power control of PSFCH transmission, the priority value i starting from 1 is used in current TS38.213. Thus, the following change was proposed,

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=0KMii=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=0KMii=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 0, 1, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=0KMii=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=0KMii=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 0, 1, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 such that NTx,PSFCH≥1and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

In [2], the priority value is treated as ranging from 1 to 8. In congestion control for mode 2, CR(i) is the CR evaluated with ‘Priority’ field in the SCI set to i. Thus, range of i is [0,7]. For the parameter CRLimit(k), it corresponds to the higher layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range. Thus, range of k is [1,8]. The parameters i and  k are directly compared in the item “i≥kCR(i)”. Thus, the following change was proposed,

8.1.6 Sidelink congestion control in sidelink resource allocation mode 2 If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k; i≥kCRi≤CRLimitk where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with 'Priority' field in the SCI set to (i-1), and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time. The congestion control processing time N is based on µ of Table 8.1.6-1 and Table 8.1.6-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH is to be transmitted. A UE shall only apply a single processing time capability in sidelink congestion control. <unchanged part omitted>

Round#1 discussion

Please provide your views regarding the following questions in the table below.

Question 2-1: Is the range of priority value as [0, 7] or [1, 8] used in physical layer specs and why? For the range of priority value as [0, 7], do you agree that 2nd change in R1-2111298 should be adopted? For the range as [1, 8], do you agree that the change in R1-2112010 should be adopted?

| Company | View |
| --- | --- |
| Intel | We don’t have strong preference. What matters is the aligned assumption throughout the specs. The interpretation which leads to a smaller number of corrections should be selected. |
| NTT DOCOMO | Either is fine, but it seems that appropriate update is necessary for the selected direction between [0, 7] and [1, 8]. If we go with [1, 8], an update as discussed in 2-2 below would be necessary, and above updates of [1][2] becomes unnecessary. If we go with [0, 7], the updates of [1][2] would be OK. |
| LGE | We think it is reasonable to consider the priority value range as [1, 8] as the text copied from 14.2.1 of TS 36.213 is defining how the bit field in SCI is set. In this sense we understand that the priority value signaled via SCI having the priority field set to ‘000’ is 1, not 0. So the changes in [1] or [2] are not necessary, and, if necessary, we may consider the following clarification in the congestion control to say that the index i corresponds to the priority value indicated by the SCI, not the field itself: where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with 'Priority' field in the SCI indicating set to priority value i, To make this intention clear, we agree that the text in 14.2.1 of TS 36.213 needs to appear in NR spec as well. |
| CATT,GOHIGH | No strong preference. However, in physical layer, indication value range of priority field in SCI is from '000' to '111'. It is better to treat the value range as [0,7]. |
| OPPO | After double check 38.213 and 38.331, we think the value range [1, 8] is more reasonable. For 16.2.4.3.1 in 38.213: For 38.331 in RP configuration If the SL priority value range is [1, 8], then if the priority thd is set to [2, 8], then SL priority can be higher or lower than UL. If the priority thd is set to 9, SL is always prioritized over UL. If the priority thd is set to 1, UL is always prioritized over SL. That is the logic for the prioritization comparison between SL and UL. While if the SL priority value range is [0, 7], the value 9 for the priority thd is meaningless. Since if the priority thd is set to 8, SL is always prioritized over UL. There is no necessary to set the value to 9. Furthermore, following the same logic in LTE, we think it is preferred to clarify the priority range is [1, 8] in SCI. |
| vivo | Either works. But considering the required spec changes and the compatibility/coexistence with LTE, we prefer to use [1, 8] instead of [0, 7]. If we migrate the corresponding text from LTE to NR for clarifying the ‘priority’, the other changes may not be needed (which may actually also due to a copy from LTE …). |
| Samsung | Either works. But for the consistency between NR and LTE specifications, and between PHY and higher layer specifications, we think clarifying priority range as [1, 8] is better way. |
| Nokia, NSB | We think it is clear the priority is [1,8]: - 38.212 refers to higher layer specifications, these use [1,8], as can be seen from the ASN.1 in 38.331. - For in-device coexistence, we discussed prioritization between LTE SL and NR SL transmission. This is based on direct comparison of priority. For LTE V2X sidelink, the priority was explicitly stated as [1,8]. |
| Huawei, HiSilicon | We think the priority range is [1, 8]. There are many places in both high layer and PHY layer in addition other companies listed to demonstrate the priority value ranges. Priority value in SCI refers to TS23.287, where it specifies that the priority value in NR-V is same as in LTE-V. Thus the value range is [1, 8]. #### 5.4.3.3 Priority Level The Priority Level for NR PC5 has the same format and meaning as the Priority value of the ProSe Per-Packet Priority (PPPP) for LTE PC5 defined in TS 23.285 [8]. For LTE PC5, the PPPP value also reflects the latency requirement and the PDB derivation is according to TS 23.285 [8], i.e. the low PDB is mapped to the high priority PPPP value. On the other hand, for NR PC5, the PDB is derived from the PQI table as defined in clause 5.4.4.. In TS 38.331, it specifies the highest priority level for a QoS flow is 1 and the lowest is 8. sl-PriorityLevel Indicates the Priority Level for a QoS flow. Values ordered in decreasing order of priority, i.e. with 1 as the highest priority and 8 as the lowest priority.. In TS 38.214, clause 8.1.4 3) The internal parameter Th(pi,pj) is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-Thres-RSRP-List, where i=pi+pj-1*8.. The priority pi,pj cannot be equal to 0 (if they are equal to 0, the value of i could be 0 or negative, which is not reasonable) and imply the priority range should be [1, 8], rather than [0, 7] Thus 2nd change in R1-2111298 is not needed. As the explanation from R1-2112010, we think the description of “value of the xxx field” is commonly used in specs, which does not refer to the binary value itself, instead. Because SCI binary value is internally mapped to the priority value, i.e. 1-8, before being used in procedure after SCI decoding. Thus we do not think change in R1-2112010 is not needed. |
| Ericsson | We do not have strong preference of which approach to take as long as the priority fields along the specifications are consistent. |
| Lenovo/Motorola Mobility | No strong preference for this issue |
| Qualcomm | We share the view that the preferred option is the one that leads to consistent specifications with the fewest changes. |
| ZTE,Sanechips | Share the view that less spec. change is preferred |

Question 2-2: For proponents of the range of priority value as [1, 8], do you think a similar part as LTE V2X for mapping between “Priority” field and priority value  as cited above should be added in NR V2X and why?

| Company | View |
| --- | --- |
| NTT DOCOMO | Yes for [1, 8]; otherwise, it seems that correspondence between priority field and priority value is unclear. |
| LGE | Yes |
| OPPO | Yes. |
| vivo | Yes, as explained above. |
| Samsung | Yes |
| Nokia, NSB | Yes, to avoid doubts about interpretation of the priority field in SCI format 1-A and for consistency with the LTE specification. |
| Huawei, HiSilicon | LTE-V value is reused which is specified in TS23.287 (see our comments in Q2-1), it seems no strong need to capture it again in other specs. |
| Qualcomm | We would be ok with this update for [1, 8]. |
| ZTE,Sanechips | OK with [1,8] |

Round#2 discussion

For the inputs on Question 2-1, as commented from several companies, “Priority value” in current specs ranges from 1 to 8. In this sense, the 2nd change in [1] seems not needed as the proponent implied in the 1st round discussion.

Regarding whether the change in [2] is needed, Huawei raised a comment that “the description of value of the xxx field is commonly used in specs, which does not refer to the binary value itself”. In moderator’s understanding, in NR L1 specs, e.g. for “Bandwidth part indicator” field in DCI format 0_1, it is clearly specified in TS38.212, clause 7.3.1.1.2, which configured BWP the binary value of the field corresponds to (i.e. Table 7.3.1.1.2-1). For the change in [2], it is not clear enough whether the parameter i refers to the priority field value (0-7) or priority value (1-8 as majority views). Thus, we propose the following TP for further clarification.

TP1

8.1.6 Sidelink congestion control in sidelink resource allocation mode 2 If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k; i≥kCRi≤CRLimitk where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with 'Priority' field in the SCI set to priority value i, and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time. The congestion control processing time N is based on µ of Table 8.1.6-1 and Table 8.1.6-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH is to be transmitted. A UE shall only apply a single processing time capability in sidelink congestion control. <unchanged part omitted>

Question 2-3: Do you agree with TP1 above? If not, please indicate any concern or suggestion.

| Company | View |
| --- | --- |
| Apple | Agree |
| LGE | OK |
| OPPO | Agree |
| CATT,GOHIGH | Agree |
| NTT DOCOMO | OK |
| Samsung | Agree |
| vivo | OK |
| Ericsson | We do not think the clarification is needed. The specification text is clear. |
| Nokia, NSB | Not needed, especially if the next TP for 38.212 is agreed. |

For the inputs on Question 2-2, companies except for Huawei agreed to introduce a mapping between priority field value and priority value, similar as LTE-V. Note that in LTE-V, interpretation of “Priority” field in SCI format 1 refers to SA2 specs TS23.285 and the mapping is specified in the section for PSCCH transmission. For NR V2X, corresponding interpretation of “Priority” field in SCI format 1-A refers to TS23.287 and the mapping is not specified yet. Thus, we propose the following TP.

TP2

#### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: - Priority – 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value ‘000’ of Priority field corresponds to priority value ‘1’, value ‘001’ of Priority field corresponds to priority value ‘2’, and so on. <unchanged part omitted>

Question 2-4: Do you agree with TP2 above? If not, please indicate any concern or suggestion.

| Company | View |
| --- | --- |
| Apple | Agree |
| LGE | Agree |
| OPPO | Agree |
| CATT,GOHIGH | Agree |
| NTT DOCOMO | OK |
| Samsung | Agree |
| vivo | OK |
| Ericsson | OK |
| Nokia, NSB | OK |

3rd change in R1-2111298

In [1], for power control of PSFCH transmission, it was stated that if dl-P0-PSFCH is not provided, the number of PSFCH transmissions NTx,PSFCH determined by the UE should be upper bounded by Nmax,PSFCH, since UE is capable of transmitting a maximum of Nmax,PSFCH PSFCHs. Hence, the following change is proposed,

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in clause 16.2.4.2 such that NTx,PSFCH≥1, NTx,PSFCH≤Nmax,PSFCH and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

Round#1 discussion

Please provide your views regarding the question in the table below.

Question 3-1: Do you agree that the change should be fixed? If no, please provide the reasons and suggestions, if any.

| Company | View |
| --- | --- |
| Intel | Agree |
| LGE | OK |
| CATT,GOHIGH | Agree |
| OPPO | Agree |
| vivo | We don’t think the change is essential: anyway the UE will never autonomously select a value beyond upper bound that exceeds its capabilities. |
| Samsung | Agree |
| Nokia, NSB | No strong view, but we don’t think that is essential since it should be obvious. |
| Huawei, HiSilicon | We think UE implementation, for example how many PSFCH are transmitted simultaneously, are always subject to the UE capability specified in TS 38.306, not matter whether the PHY spec clearly states the capability or not. So the necessity to have this CR is less. |
| Lenovo/Motorola Mobility | No, it is not essential correction and it is part of the UE capability and it is captured. |
| Qualcomm | While we’re ok with the principle, we don’t think the change is essential. The UE would behave in this way given current specifications and capability. |
| ZTE,Sanechips | Prefer to capture a reference to 38.331 or 38.306 on the parameter psfch-TxNumber representing N_{psfch,max} |

Round#2 discussion

Based on the inputs from companies, it seems several companies think the change is not necessary, since UE implementation is always subject to UE capability. Thus, we propose the following conclusion with no specs change.

Proposed conclusion: It is RAN1 understanding that the determined number NTx,PSFCH of simultaneous PSFCH transmissions is subject to UE capability of transmitting a maximum of Nmax,PSFCH PSFCHs.

Question 3-2: Do you agree with the proposed conclusion and why?

| Company | Views |
| --- | --- |
| Apple | Fine with the conclusion. No specification change is needed. |
| LGE | Okay with the conclusion |
| OPPO | We are fine with the conclusion. |
| CATT,GOHIGH | Fine with the conclusion. |
| NTT DOCOMO | OK with this conclusion. |
| Samsung | OK with the conclusion. |
| vivo | OK with the conclusion. |
| Huawei, HiSilicon | There is no need to require a conclusion to every question. The conclusion is a statement of the simple meaning of a capability! Such things are not needed in the late maintenance phase. Issue needs to be closed without further steps. |
| Ericsson | We are OK with the conclusion or simply close the discussion on this issue without any further action. |
| Nokia, NSB | We don’t think this is needed since it should be clear that all UE behaviour is subject to UE capability. However, we can accept this conclusion if other companies feel this is absolutely necessary. |

Conclusion

The outcome of [107-e-NR-5G_V2X-06] is as following,

Agreement

Correction on alignment of priority values in is endorsed in R1-21xxxxx mapping between priority field value and priority value is endorsed in R1-211XXXX (TS38.212, Rel-16, CR#0XXX, Cat. F).

Correction on alignment of priority values in is endorsed in R1-21xxxxx priority order in power control of PSFCH is endorsed in R1-211XXXX (TS38.213, Rel-16, CR#0XXX, Cat. F).

For the 2nd change in R1-2111298, no specification update is needed. For the 3rd change in R1-2111298, the change in R1-2112010, no consensus was reached.

Reference

- R1-2111298, “Corrections for PSFCH power control in TS 38.213”, OPPO, RAN1#107-e.

- R1-2112010, “Correction on priority field value in congestion control for mode 2”, Sharp, RAN1#107-e.