3GPP TSG-RAN WG1 Meeting #107-e			R1-21xxxxxx

E-Meeting, November 11–19, 2021

Title:	Draft reply LS on PDCCH Blind Detection in CA

Response to:	R1-2110757(R2-2109168),

Release:	Rel-16

Work Item:	NR_L1enh_URLLC-Core

Source:	Huawei [TSG RAN WG1]

To:	RAN2

Contact Person:

#### Name:	Cheng Yan

E-mail Address:	chengyan.cheng@huawei.com

1. Overall Description:

RAN1 thanks RAN2 for the LS on PDCCH Blind Detection in CA. RAN1 discussed the questions in the LS and would like to provide answers as below:

Question 1: How many combinations for FG 11-2c and FG 11-2g can be reported at most from RAN1 perspective?

Answer to Question 1: RAN1 confirms that multiple combinations of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for FG 11-2c and FG 11-2g can be reported.

- The maximum number of reported combinations is a trade-off between configuration flexibility and the potential signaling overhead. From RAN1 perspective, reporting a maximum number of 8 combinations is sufficient but RAN1 leaves it to RAN2’s decision if more combinations should be supported.

- Note: UE can report any number of combinations that is smaller or equal to the maximum number of combinations.

Question 2: Whether the “supported span arrangement for CA” should be reported for each of the combinations or reported only once for FG 11-2c?

Answer to Question 2: The “supported span arrangement for CA” is reported only once for all reported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for FG 11-2c.

Question 3: RAN2 wonders whether more than one combination should be supported for FG 11-2e as well? If the answer is yes, how many combinations for FG 11-2e can be reported at most from RAN1 perspective?

Answer to Question 3: RAN1 confirms that multiple combinations of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) for FG 11-2e can be reported.

- The maximum number of reported combinations is the same as that for combinations of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for FG 11 2-c.

- Note 1: UE can report any number of combinations that is smaller or equal to the maximum number of combinations.

- Note 2: One combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) reported by a UE for FG 11-2e corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) reported by the UE for FG 11-2c or FG 11-2g, as defined in clause 10 in TS 38.213. It is up to RAN2 on where/how to capture the note.

In addition, during the discussion of the questions from RAN2, the following note 3 and note 4 are also agreed in RAN1:

- Note 3: Only one from FG 11-2c and FG 11-2g will be reported by UE if reported. It is up to RAN2 on where/how to capture the note.

- Note 4: Only one from FG 11-2a and FG 11-2f will be reported by UE if reported. It is up to RAN2 on where/how to capture the note.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107bis-e	17 – 25 January 2022			E-Meeting

TSG-RAN WG1 Meeting #108-e	21 February – 03 March 2022	E-Meeting