3GPP TSG-RAN WG1 Meeting #107-e	R1-211xxxx

# e-Meeting, November 11–19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Multicast and Broadcast Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS |  |  |  |  |  | Date: |  |  | 2021-11-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Inclusion of Rel-17 NR Multicast and Broadcast Services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-17 NR Multicast and Broadcast Services: - Section 3.3: Add abbreviation for MBS - Section 5.4.2.1: Update to support limited buffer rate matching for multicast and broadcast. - Section 7.3.1.5 & 7.3.1.5.1 & 7.3.1.5.2: Define DCI formats for scheduling of MBS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR Multicast and Broadcast Services in Rel-17 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.4.2.1, 7.3.1, 7.3.1.5(new), 7.3.1.5.1(new), 7.3.1.5.2(new), 7.3.1.5.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Dedicated demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MBS	Multicast broadcast services

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

SL-SCH	Sidelink shared channel

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

### 5.4.2	Rate matching for LDPC code

The rate matching for LDPC code is defined per coded block and consists of bit selection and bit interleaving. The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as .

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

- if the PDSCH is scheduled by DCI format 4_1/4_2,

- maximum number of layers is given by X, where

- if the higher layer parameter maxMIMO-Layers-Multicast of PDSCH-Config-Multicast is configured,  X is given by that parameter;

- Otherwise, X equals to 1;

- if the higher layer parameter mcs-Table given by a PDSCH-Config-Multicast for at least one CFR is set to  'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

- if the PDSCH is scheduled by DCI format 4_0,

- maximum number of layers is 1;

- if the higher layer parameter mcs-Table given by a PDSCH-Config-MCCH is set to  'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

- if the higher layer parameter mcs-Table given by a PDSCH-Config-MTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the CFR if only one CFR is configured to the UE;

-	maximum coding rate of 948/1024;

-	NRE=156∙nPRB;

-	C is the number of code blocks of the transport block determined according to Clause 5.2.2.

For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4-0/4_1/4_2,

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell

-	if the higher layer parameter mcs-Table or mcs-TableDCI-1-2 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise a maximum modulation order  is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2 or mcs-TableTransformPrecoderDCI-0-2 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for UL-SCH; otherwise a maximum modulation order  is assumed for UL-SCH

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial downlink bandwidth part if there is no other downlink bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Clause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured DL BWPs and UL BWPs of a carrier for DL-SCH and UL-SCH, respectively, or Maximum number of PRBs across all CFRs of a carrier for DL-SCH with PDSCH scheduled by DCI format 4_1/4_2 |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

Denoting by  the rate matching output sequence length for the -th coded block, where the value of  is determined as follows:

Set

for  to

if the -th coded block is not scheduled for transmission as indicated by CBGTI according to Clause 5.1.7.2 for DL-SCH and 6.1.5.2 for UL-SCH in [6, TS 38.214]

;

else

if

;

else

;

end if

;

end if

end for

where

-	 is the number of transmission layers that the transport block is mapped onto;

-	 is the modulation order;

-	 is the total number of coded bits available for transmission of the transport block;

-	 if CBGTI is not present in the DCI scheduling the transport block and  is the number of scheduled code blocks of the transport block if CBGTI is present in the DCI scheduling the transport block.

Denote by  the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where  is given by Table 5.4.2.1-2 according to the value of  and LDPC base graph:

;

;

while

if

;

;

end if

;

end while

Table 5.4.2.1-2: Starting position of different redundancy versions,

|  | LDPC base graph 1 | LDPC base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

< Unchanged parts are omitted >

7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI) |
| 0_2 | Scheduling of PUSCH in one cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback |
| 1_2 | Scheduling of PDSCH in one cell |
| 2_0 | Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |
| 2_4 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE |
| 2_5 | Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473] |
| 2_6 | Notifying the power saving information outside DRX Active Time for one or more UEs |
| 3_0 | Scheduling of NR sidelink in one cell |
| 3_1 | Scheduling of LTE sidelink in one cell |
| 4_0 | Schedulng of PDSCH with CRC scrambled by MCCH-RNTI/G-RNTI for broadcast |
| 4_1 | Schedulng of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast |
| 4_2 | Schedulng of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast |

The fields defined in the DCI formats below are mapped to the information bits  to  as follows.

Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit  and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to .

If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12.

The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

If a UE is configured with pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16 in this clause.

< Unchanged parts are omitted >

7.3.1.5	DCI formats for scheduling of MBS

7.3.1.5.1	Format 4_0

DCI format 4_0 is used for the scheduling of PDSCH for broadcast in DL cell.

The following information is transmitted by means of the DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI configured by MBS-SessionInfo:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2 bits where NRBDL,CFR equals to  NRBDL,BWP as given by clause 7.3.1.0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	MCCH change notification – 2 bits as defined in Clause x.x.x of [8, TS38.321] if the CRC of the DCI format 4_0 is scrambled by MCCH-RNTI. Otherwise, this bit field is reserved.

-	Padding bits, if required

Zeros shall be appended to DCI format 4_0 until the payload size equals that of DCI format 1_0 monitored in common search space in the same serving cell.

7.3.1.5.2	Format 4_1

DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_1 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2 bits where NRBDL,CFR equals to NRBDL,BWP as given by clause 7.3.1.0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	Reserved bits –  3 bits

-	Padding bits, if required

Zeros shall be appended to DCI format 4_1 until the payload size equals that of DCI format 1_0 monitored in common search space in the same serving cell.

7.3.1.5.3	Format 4_2

DCI format 4_2 is used for the scheduling of PDSCH in DL cell.

The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI:

-	Frequency domain resource assignment – number of bits determined by the following, where NRBDL,CFR is the size of the common frequency resource as configured by higher layer parameter locationAndBandwidth-Multicast:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	log2(NRBDL,CFR(NRBDL,CFR+1)2 bits if only resource allocation type 1 is configured, or

-	maxlog2(NRBDL,CFR(NRBDL,CFR+1)2,NRBG+1 bits if resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch'.

-	If resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2(NRBDL,CFR(NRBDL,CFR+1)2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if vrb-ToPRB-Interleaver in PDSCH-Config-Multicast is not configured;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured in PDSCH-Config-Multicast or is set to 'staticBundling', or 1 bit if the higher layer parameter prb-BundlingType in PDSCH-Config-Multicast is set to 'dynamicBundling' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2 in PDSCH-Config-Multicast, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if more than one serving cell are configured in the DL for multicast and the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL for multicast and the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 bits are the counter DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-1 is configured in PDSCH-Config-Multicast, if the bit width of the Downlink assignment index in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config-Multicast1 if configured or PUCCH-Config-Multicast2 if configured; otherwise, I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config.

If higher layer parameter priorityIndicatorDCI-1-1 is configured in PDSCH-Config-Multicast, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports {p0,…,pv-1} shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals max⁡{xA,xB}, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI in PDCCH-Config-Multicast is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

-	DMRS sequence initialization – 1 bit.

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-1-1 is not configured in PDSCH-Config-Multicast; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Enabling/disabling HARQ-ACK feedback indication –1 bit if higher layer parameter harq-FeedbackEnabler-Multicast indicates dci-enabler; 0 bit, otherwise.

< Unchanged parts are omitted >