3GPP TSG RAN WG1 Meeting #85		                                                       R1-164257

Nanjing, P.R.China, 23rd - 27th May 2016

Source:	CATT

Title:	Text Proposal on the purpose of link-level channel modes for 38.900

Agenda Item:	7.2

Document for:	Discussion and Decision

Introduction

The attached TP is to provide an update to TR36.900. The changes are incorporated in Section 7.7 of the attached draft.

---------------------------------- Start of Text Proposal ------------------------------------------------------

7.7 	Channel Models for Link-Level Evaluations

Link level simulations alone will not be used for algorithm comparison because they reflect only one snapshot of the channel behaviour. Furthermore, they do not account for system attributes such as scheduling and HARQ. For these reasons, link level simulations do not allow any conclusions about the typical behaviour of the system. Only system level simulations can achieve that. Therefore this document targets system level simulations for the final algorithm comparison.

Link level simulations will not be used to compare performance of different algorithms. Rather, they will be used only for calibration, which is the comparison of performance results from different implementations of a given algorithm.

7.7.1 	Clustered Delay Line (CDL) Models

The CDL models are defined for the full frequency range from 0.5 GHz to 100 GHz with a maximum bandwidth of 2 GHz. CDL models can be implemented by e.g., Step 10 in Section 7.5. Three CDL models, namely CDL-A, CDL-B and CDL-C are constructed to represent three different channel profiles, the parameters of which can be found respectively in Table 7.7.1-1, Table 7.7.1-2 and Table 7.7.1-3. It is noted that the delay values are normalized so that RMS delay spread is 1 sec.

Each CDL model can be scaled in delay so that the model achieves a desired RMS delay spread, according to the procedure described in 7.7.3.

For modeling effect of beamforming in a simplified way, a brick-wall window can be applied to a delay-scaled CDL model. The power shall be normalized after applying the window. A TDL model for simplified evaluations can be obtained from the CDL model, according to this method. Angle scaling and details of this method are FFS.

Table 7.7.1-1. CDL-A

| Clusters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cluster | Normalized delay |  | Power |  | AoD |  | AoA |  | ZoD |  | ZoA |
| # |  |  | dB |  | º |  | º |  | º |  | º |
| 1 | 0 |  | -18.8 |  | -178.1 |  | 51.3 |  | 50.2 |  | 125.4 |
| 2 | 0.3816 |  | -5.4 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 3 | 0.4023 |  | -7.6 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 4 | 0.5864 |  | -9.4 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 5 | 0.4608 |  | -11.4 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 6 | 0.5372 |  | -13.6 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 7 | 0.6705 |  | -15.3 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 8 | 0.5746 |  | -15.9 |  | 121.5 |  | -1.8 |  | 150.2 |  | 47.1 |
| 9 | 0.7615 |  | -12.9 |  | -81.7 |  | -41.9 |  | 55.2 |  | 56 |
| 10 | 1.5367 |  | -21.3 |  | 158.4 |  | 94.2 |  | 26.4 |  | 30.1 |
| 11 | 1.8968 |  | -12 |  | -83 |  | 51.9 |  | 126.4 |  | 58.8 |
| 12 | 2.2229 |  | -22.1 |  | 134.8 |  | -115.9 |  | 171.6 |  | 26 |
| 13 | 2.1707 |  | -17.8 |  | -153 |  | 26.6 |  | 151.4 |  | 49.2 |
| 14 | 2.4930 |  | -20.6 |  | -172 |  | 76.6 |  | 157.2 |  | 143.1 |
| 15 | 2.5104 |  | -16.2 |  | -129.9 |  | -7 |  | 47.2 |  | 117.4 |
| 16 | 3.0567 |  | -16.7 |  | -136 |  | -23 |  | 40.4 |  | 122.7 |
| 17 | 4.0789 |  | -18.1 |  | 165.4 |  | -47.2 |  | 43.3 |  | 123.2 |
| 18 | 4.4556 |  | -21.6 |  | 148.4 |  | 110.4 |  | 161.8 |  | 32.6 |
| 19 | 4.5671 |  | -23.7 |  | 132.7 |  | 144.5 |  | 10.8 |  | 27.2 |
| 20 | 4.7941 |  | -24.3 |  | -118.6 |  | 155.3 |  | 16.7 |  | 15.2 |
| 21 | 5.0039 |  | -22 |  | -154.1 |  | 102 |  | 171.7 |  | 146 |
| 22 | 5.3014 |  | -24.4 |  | 126.5 |  | -151.8 |  | 22.7 |  | 150.7 |
| 23 | 9.6535 |  | -38.5 |  | -56.2 |  | 55.2 |  | 144.9 |  | 156.1 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | ASD |  | ASA |  | ZSD |  | ZSA |  | XPR |  |
| Unit |  | º |  | º |  | º |  | º |  | dB |  |
| Value |  | 5 |  | 11 |  | 3 |  | 3 |  | 10 |  |

Table 7.7.1-2. CDL-B

| Clusters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cluster | Normalized delay |  | Power |  | AoD |  | AoA |  | ZoD |  | ZoA |
| # |  |  | dB |  | º |  | º |  | º |  | º |
| 1 | 0 |  | -8.5 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 |
| 2 | 0.1072 |  | -10.7 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 |
| 3 | 0.2154 |  | -12.5 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 |
| 4 | 0.2096 |  | -11.7 |  | -34.1 |  | 125.5 |  | 115.3 |  | 63.3 |
| 5 | 0.2871 |  | -18.3 |  | -65.4 |  | -88.0 |  | 119.3 |  | 59.9 |
| 6 | 0.2986 |  | -9.7 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 |
| 7 | 0.3752 |  | -11.9 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 |
| 8 | 0.5057 |  | -13.7 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 |
| 9 | 0.3682 |  | -16.1 |  | -67.2 |  | -89.8 |  | 118.2 |  | 82.6 |
| 10 | 0.3697 |  | -11.5 |  | 52.5 |  | 132.1 |  | 102.0 |  | 66.3 |
| 11 | 0.5701 |  | -17.4 |  | -72 |  | -83.6 |  | 100.4 |  | 61.6 |
| 12 | 0.5285 |  | -17.5 |  | 74.3 |  | 95.3 |  | 98.3 |  | 58.0 |
| 13 | 1.1023 |  | -13.3 |  | -52.2 |  | 103.7 |  | 103.4 |  | 78.2 |
| 14 | 1.2760 |  | -14.2 |  | -50.5 |  | -87.8 |  | 102.5 |  | 82.0 |
| 15 | 1.5479 |  | -16 |  | 61.4 |  | -92.5 |  | 101.4 |  | 62.4 |
| 16 | 1.7846 |  | -10.4 |  | 30.6 |  | -139.1 |  | 103.0 |  | 78.0 |
| 17 | 2.0174 |  | -16.1 |  | -72.5 |  | -90.6 |  | 100.0 |  | 60.9 |
| 18 | 2.8301 |  | -20.7 |  | -90.6 |  | 58.6 |  | 115.2 |  | 82.9 |
| 19 | 3.0227 |  | -18.3 |  | -77.6 |  | -79.0 |  | 100.5 |  | 60.8 |
| 20 | 3.6195 |  | -19.9 |  | -82.6 |  | 65.8 |  | 119.6 |  | 57.3 |
| 21 | 4.1076 |  | -23.4 |  | -103.6 |  | 52.7 |  | 118.7 |  | 59.9 |
| 22 | 4.2801 |  | -17.7 |  | 75.6 |  | 88.7 |  | 117.8 |  | 60.1 |
| 23 | 4.7846 |  | -19.8 |  | -77.6 |  | -60.4 |  | 115.7 |  | 62.3 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | ASD |  | ASA |  | ZSD |  | ZSA |  | XPR |  |
| Unit |  | º |  | º |  | º |  | º |  | dB |  |
| Value |  | 10 |  | 22 |  | 3 |  | 7 |  | 8 |  |

Table 7.7.1-3. CDL-C

| Clusters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cluster | Normalized delay |  | Power |  | AoD |  | AoA |  | ZoD |  | ZoA |
| # |  |  | dB |  | º |  | º |  | º |  | º |
| 1 | 0 |  | -12.1 |  | -46.6 |  | -101 |  | 97.2 |  | 87.6 |
| 2 | 0.2099 |  | -8.9 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 3 | 0.2219 |  | -11.2 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 4 | 0.2329 |  | -12.9 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 5 | 0.2176 |  | -10.2 |  | -40.7 |  | -127.5 |  | 100.6 |  | 70.1 |
| 6 | 0.6367 |  | -7.7 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 7 | 0.6448 |  | -9.9 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 8 | 0.6560 |  | -11.6 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 9 | 0.6585 |  | -15.1 |  | 73.1 |  | 55.4 |  | 105.2 |  | 67.4 |
| 10 | 0.7935 |  | -14.8 |  | -64.5 |  | 66.5 |  | 95.3 |  | 63.8 |
| 11 | 0.8214 |  | -18.4 |  | 80.2 |  | -48.1 |  | 106.1 |  | 71.4 |
| 12 | 0.9336 |  | -18.8 |  | -97.1 |  | 46.9 |  | 93.5 |  | 60.5 |
| 13 | 1.2286 |  | -12.8 |  | -55.3 |  | 68.1 |  | 103.7 |  | 90.6 |
| 14 | 1.3084 |  | -14.5 |  | -64.3 |  | -68.7 |  | 104.2 |  | 60.1 |
| 15 | 2.1706 |  | -16.4 |  | -78.5 |  | 81.5 |  | 93.0 |  | 61.0 |
| 16 | 2.7108 |  | -20.9 |  | 102.7 |  | 30.7 |  | 104.2 |  | 100.7 |
| 17 | 4.2593 |  | -21.6 |  | 99.2 |  | -16.4 |  | 94.9 |  | 62.3 |
| 18 | 4.6007 |  | -21.6 |  | 88.8 |  | 3.8 |  | 93.1 |  | 66.7 |
| 19 | 5.4906 |  | -23.5 |  | -101.9 |  | -13.7 |  | 92.2 |  | 52.9 |
| 20 | 5.6082 |  | -24.8 |  | 92.2 |  | 9.7 |  | 106.7 |  | 61.8 |
| 21 | 6.3071 |  | -23.7 |  | 93.3 |  | 5.6 |  | 93.0 |  | 51.9 |
| 22 | 6.6380 |  | -23.4 |  | 106.6 |  | 0.7 |  | 92.9 |  | 61.7 |
| 23 | 7.0433 |  | -29.3 |  | 119.5 |  | -21.9 |  | 105.2 |  | 58 |
| 24 | 8.6531 |  | -30.5 |  | -123.8 |  | 33.6 |  | 107.8 |  | 57 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | ASD |  | ASA |  | ZSD |  | ZSA |  | XPR |  |
| Unit |  | º |  | º |  | º |  | º |  | dB |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

7.7.2 	Tapped Delay Line (TDL) Models

The TDL models for simplified evaluations, e.g., for non-MIMO evaluations, are defined for the full frequency range from 0.5 GHz to 100 GHz with a maximum bandwidth of 2 GHz. Three TDL models, namely TDL-A, TDL-B and TDL-C, are constructed to represent three different channel profiles, the parameters of which can be found respectively in Table 7.7.2-1, Table 7.7.2-2 and Table 7.7.2-3. It is noted that the delay values are normalized so that RMS delay spread is 1 sec. Each TDL model can be scaled in delay so that the model achieves a desired RMS delay spread, according to the procedure described in 7.7.3.

Table 7.7.2-1. TDL-A

| Tap | Normalized delay | Power |
| --- | --- | --- |
| # |  | dB |
| 1 | 0 | -18.8 |
| 2 | 0.3816 | -5.4 |
| 3 | 0.4023 | -7.6 |
| 4 | 0.5864 | -9.4 |
| 5 | 0.4608 | -11.4 |
| 6 | 0.5372 | -13.6 |
| 7 | 0.6705 | -15.3 |
| 8 | 0.5746 | -15.9 |
| 9 | 0.7615 | -12.9 |
| 10 | 1.5367 | -21.3 |
| 11 | 1.8968 | -12.0 |
| 12 | 2.2229 | -22.1 |
| 13 | 2.1707 | -17.8 |
| 14 | 2.4930 | -20.6 |
| 15 | 2.5104 | -16.2 |
| 16 | 3.0567 | -16.7 |
| 17 | 4.0789 | -18.1 |
| 18 | 4.4556 | -21.6 |
| 19 | 4.5671 | -23.7 |
| 20 | 4.7941 | -24.3 |
| 21 | 5.0039 | -22.0 |
| 22 | 5.3014 | -24.4 |
| 23 | 9.6535 | -38.5 |

Table 7.7.2-2. TDL-B

| Tap | Normalized delay | Power |
| --- | --- | --- |
| # |  | dB |
| 1 | 0 | -8.5 |
| 2 | 0.1072 | -10.7 |
| 3 | 0.2154 | -12.5 |
| 4 | 0.2096 | -11.7 |
| 5 | 0.2871 | -18.3 |
| 6 | 0.2986 | -9.7 |
| 7 | 0.3752 | -11.9 |
| 8 | 0.5057 | -13.7 |
| 9 | 0.3682 | -16.1 |
| 10 | 0.3697 | -11.5 |
| 11 | 0.5701 | -17.4 |
| 12 | 0.5285 | -17.5 |
| 13 | 1.1023 | -13.3 |
| 14 | 1.2760 | -14.2 |
| 15 | 1.5479 | -16 |
| 16 | 1.7846 | -10.4 |
| 17 | 2.0174 | -16.1 |
| 18 | 2.8301 | -20.7 |
| 19 | 3.0227 | -18.3 |
| 20 | 3.6195 | -19.9 |
| 21 | 4.1076 | -23.4 |
| 22 | 4.2801 | -17.7 |
| 23 | 4.7846 | -19.8 |

Table 7.7.2-3. TDL-C

| Tap | Normalized delay | Power |
| --- | --- | --- |
| # |  | dB |
| 1 | 0 | -12.1 |
| 2 | 0.2099 | -8.9 |
| 3 | 0.2219 | -11.2 |
| 4 | 0.2329 | -12.9 |
| 5 | 0.2176 | -10.2 |
| 6 | 0.6367 | -7.7 |
| 7 | 0.6448 | -9.9 |
| 8 | 0.6560 | -11.6 |
| 9 | 0.6585 | -15.1 |
| 10 | 0.7935 | -14.8 |
| 11 | 0.8214 | -18.4 |
| 12 | 0.9336 | -18.8 |
| 13 | 1.2286 | -12.8 |
| 14 | 1.3084 | -14.5 |
| 15 | 2.1706 | -16.4 |
| 16 | 2.7108 | -20.9 |
| 17 | 4.2593 | -21.6 |
| 18 | 4.6007 | -21.6 |
| 19 | 5.4906 | -23.5 |
| 20 | 5.6082 | -24.8 |
| 21 | 6.3071 | -23.7 |
| 22 | 6.6380 | -23.4 |
| 23 | 7.0433 | -29.3 |
| 24 | 8.6531 | -30.5 |

7.7.3 	Scaling of Delays

Both CDL and TDL models can be scaled in delay so that a desired RMS delay spread can be achieved. The scaled delays can be obtained according to the following equation:

τn,scaled = τn,model · DSdesired

in which

τn,model	is the normalized delay value of the nth cluster in a CDL or a TDL model

τn,scaled	is the new delay value (in nsec) of the nth cluster

DSdesired	is the wanted delay spread (in nsec)

Example scaling parameters for which the RMS delay spreads are aligned with the typical 5G evaluation scenarios are FFS.

----------------------------------------------- End of Text Proposal ------------------------------------------------