3GPP TSG-RAN WG1 Meeting #85	R1-164489

Nanjing, China, 23 – 27 May, 2016

Agenda item:       6.2.11.4

Source:	NEC

Title:	Enhancements to Coordinated Multi-point Operation: Impact of interference averaging

Document for:		Discussion

1. Introduction

The study for enhancing CoMP operation has been initiated in [1]. In this document we discuss an issue pertaining to CSI feedback for CoMP.

2. Discussion

In Rel. 11 CoMP the concept of CSI process was defined in order to obtain CSI feedback for multi-node coordination. The motivation behind this mechanism of CSI process is to allow each node (or transmission point (TP)) to obtain CSI feedback corresponding to any desired interference hypothesis in a UE transparent manner. A UE that is configured with a CSI process is simply informed about the REs on which it should estimate a channel matrix, as well as REs (referred to as interference measurement resources (IMR)) on which it should measure interference.  Each IMR is formed by four contiguous REs (in one RB-pair) and spans the system bandwidth. A schematic is shown in Fig.1 in which four IMRs are indicated in an RB-pair.

To illustrate the use of IMRs let us consider the scenario in Fig 2. Here a CSI process is configured for a UE whose measurement size is two. Under that process the  UE is supposed to measure pilots from TP-1 (which is assumed to be the serving TP). These pilots would be transmitted on a NZP-CSI-RS. Furthermore, the UE is configured to measure interference from TP-2 on an IMR. TP-1 and TP-2 are both in the measurement set of that UE so that signals transmitted by those TPs are received by that UE with a sufficiently strong signal strength.

Let us consider the model for the signal received by the UE on any RE in that IMR on some RB-pair. This IMR completely overlaps with a ZP-CSI-RS and  TP-1 is muted on its REs. Suppose that TP-2 transmits PDSCH on that IMR.

Where  denotes the channel matrix modeling the channel seen from TP-2,  captures the additive noise and out-of-measurement set interference.  denotes the precoder matrix used by the interferer TP-2 at the measurement instance and  denotes a data symbol vector.

The UE will estimate an interference covariance matrix  using all the four REs in that IMR. Then, depending on the configured frequency domain granularity for CSI feedback, it may further average its estimate over IMRs across a few contiguous RB-pairs. Let us suppose that the obtained estimate averages out the dependence on the data symbols. However, even under this optimistic assumption, we can see that the obtained covariance estimate can depend on  . We will henceforth refer to this interference measurement as “snapshot” interference measurement.

Observation: The interference covariance estimate determined by a UE to compute sub-band CSI under snapshot interference measurement can depend on the transmit precoding matrix employed by the interferer.

The resulting CSI reported by the UE has the so-called flashlight effect in that it does not have enough averaging over the choice of transmit precoders that could be used by TP-2. Notice that if the UE is scheduled to receive data from TP-1 in some RBs of a subsequent sub-frame and if TP-2 is not muted on those RBs, it is very likely that the interfering TP-2 will be transmitting data intended for some other user using a precoder different from .

Thus, the interference seen by UE from TP-2 post scheduling can be very different than the one it sees from TP-2 during measurement instance, even if the physical channel does not change significantly. Further, in the absence of enough averaging, the CSI reported by the UE has larger variability. These factors significantly hamper the ability of  the outer loop link adaptation to mitigate this interference mismatch.

A more desirable CSI feedback would be one in which the interference during measurement instance is fully averaged over all choices of transmit precoders. Or one where the precoder used by TP-2 is a scaled identity precoder (with an appropriate scalar). Such a feedback has the benefit of (spatial) interference averaging which can be substantially useful. We will henceforth refer to this interference measurement as interference measurement with interference averaging.

However, a straightforward approach to realize interference averaging in which the interfering TP transmits non-data symbols using a scaled identity precoding matrix has the drawback of wasting REs that could have been used for PDSCH. This additional overhead must be accounted for.

3. Simulation Results

We present system level simulation results to compare the performance obtained with snapshot interference measurement and interference measurement with interference averaging, respectively. The CoMP scheme used is a DPS scheme that exploits coordinated muting and is detailed in [2]. The simulation parameters are summarized in Table II.

Our results are given in Table I. These results we obtained over 20 different user and RRH drops.

As we can see from the results presented in Table I there is indeed a significant benefit from interference averaging. This trend has been observed for different choices of CoMP schemes and algorithms as well.

| CoMP scheme | 5th percentile user throughput, Kbps | Median user throughput, Kbps | Average user throughput, Kbps |
| --- | --- | --- | --- |
| Baseline (Rel-10) | 862.9 | 3000.3 | 4045.9 |
| DPS (snapshot interference CSI feedback) | 807.4 Mean: (-1.6%) Min: (-10.2%) Max: (+9.7%) | 2735.1 Mean: (-6.9%) Min: (-10.4%) Max: (-2.6%) | 3718.9 Mean: (-6.0%) Min: (-8.0%) Max: (-4.3%) |
| DPS (interference averaging CSI feedback) | 1080.2 Mean: (+30.6%) Min: (+23.9%) Max: (+37.3%) | 3276.4 Mean: (+10.1%) Min: (+4.9%) Max: (+15.6%) | 4091.8 Mean: (+2.3%) Min: (+1.1%) Max: (+3.5%) |

Table I: System level simulation results.

4. Conclusion

In this contribution we demonstrated the positive impact of interference averaging. In our view, the difference  in performance between the averaged and snapshot interference measurement is significant. As a result, we believe that assumptions made for interference measurement should be clearly stated in all the evaluation results. Approaches to achieve interference averaging should be investigated once a consensus view of its benefits is reached.

References

[1] RP-160665, “Further enhancements to Coordinated Multi-Point Operation”

[2] R. Arnott et.al., “Standards Compliant CoMP Scheduling over LTE Heterogeneous Wireless Networks”, IEEE RAWNET 2015.

Fig.1: Illustration of an RB-pair with four IMR

Fig.2: Illustration of UE with measurement set size 2, configured with a CSI-process under which TP-1 is the serving TP and TP-2 is the interfering TP.