3GPP TSG-RAN WG1 #85 	R1-164704

23th – 27th May 2016

Nanjing, China

Agenda item:	7.1.5.1

Source: 	Qualcomm Incorporated

Title: 	Channel coding evaluation assumptions - performance and complexity

Document for:	Discussion/Decision

# Introduction

In RAN#71, the technology study item for 5G new RAT (NR) has been approved [1]. One important aspect is the physical layer channel coding design to support many of the use cases and services envisioned for NR, such as scaling to larger bandwidths and lower latencies within existing mobile broadband services, while expanding the network to additional more challenging communications services such as URLLC.

The purpose of this document is provide more detailed discussion on channel coding performance and computational complexity tradeoffs which was first mentioned [2]. Although performance is an important metric which is evaluated as part of the previous RAN #84b agreement, the computation complexity needed to achieve such performance gains as well as their tradeoffs need to be considered.

# Computational Complexity

The decoding algorithms for the candidate NR codes for EMBB are evaluated below by analysis of the operations per iteration. This provides a computational cost per iteration of decoding and per information bit of transmitted information, and thus will directly scale up with the throughput at the application layer. Comparison of these metrics do not necessarily directly translate to area, but they are fundamental indicators of the computation complexity between coding techniques and can provide insight into further power savings and area savings observations seen [3].

## LTE Turbo Codes

Branch metric computation

Forward metric computation

Backward metric computation

LLR computation

Extrinsic information computation

Following computations (BCJR algorithm) are required in each of the MAP stages [4]:

- Path-metric computation [Eqn. (1)],

- FSM computation [Eqn. (2)],

- RSM computation [Eqn. (3)],

- LLR computation [Eqn. (4)],

- Extrinsic computation [Eqn. (5)],

Table 1 Complexity calculations for Turbo MAP decoder per half-iteration

| Eq. | ADD/SUB | MUL by ±1 | Max* |
| --- | --- | --- | --- |
| (1) |  |  | -- |
| (2) |  | -- |  |
| (3) |  | -- |  |
| (4) |  | -- |  |
| (5) | 2 | -- | -- |
| Total per info bit |  |  |  |
| LTE Turbo code | 83 | 48 | 30 |

M = # of memory of the component code. The  # of trellis state is 2^M
n is such that 1/n is the code rate of component convolutional code

## LDPC Codes

LDPC decoding is typically performed using layered belief propagation (LBP), and the use of offset (scaled) min-sum has been identified an efficient implementation which can achieve performance close to belief propagation [5]. Note that in terms of complexity layered vs. flooding have the same overall complexity for the same total iteration count. It is also observed that layered requires roughly half the number of total iterations than flooding, which we have observed from simulations. In this sense, it is advantageous to perform layered decoding.

Following operations are performed at the i-th variable node, j-th check node, l-th layer (l{0,1,…,L-1}):

variable-node processing

check-node processing

(offset scaled min-sum algorithm)

variable-node processing

Figure 1 Layered belief propagation

Table 2 LDPC LBP complexity of operation

| Eq. | ADD/SUB | ABS | Sign() | MIN | MUL by |
| --- | --- | --- | --- | --- | --- |
| (I) | (E/L).L | -- | -- | -- | -- |
| (II) | -- | -- | E | -- | -- |
| (III) | 2M | E | -- | 2E-3M | 2M |
| (IV) | (E/L).L | -- | -- | -- | -- |
| Total | 2E+2M | E | E | (2E-3M) | 2M |
| Total per info bit | (2(+(1-R))/R) | (/R) | (/R) | ((2-3(1-R))/R) | 2*()/R |

E is the total number of edges in the graph, L is the number of layers, M is the number of check-nodes in the LDPC code, dv is the average variable node degree in the bipartite graph, R is the code-rate.

## Polar Codes

Polar codes are decoded using successive cancelation (SC) decoding and SC-list (SC-L) decoding. The latter has been considered to obtain superior performance at the cost of increased complexity. Key challenge with implementing SC-L decoding is the added complexity and the reduced throughput achievable [6]. SC decoding on the other hand is capable of achieving much higher throughputs, albeit, it cannot perform as well as traditional codes such as LDPC, Turbo. There are key operations performed in decoding polar codes which can be decomposed into:

- Single-parity check (SPC) decoding – known as f() operation in literature [7]

- Repetition decoding – known as g() operation in literature [7]

- Path-metric (PM) calculation (while traversing the tree for L-SC)

- Sort operations to pick the best-L from 2 sets of L pre-sorted metrics. Note that 2 options are possible:

  - Higher latency operation: 2L clock cycles are needed for this operation

  - Low latency operation: sorting of 2L metrics done in ~1 cycle

    - Complexity is much higher (shown in GREEN)

Table 3 Complexity of polar SC-L decoding

| Component | ADD/SUB | ABS | Sign() | MIN [lower-latency] | MUL by |
| --- | --- | --- | --- | --- | --- |
| SPC | 0.5*N*log(N)*L | -- | 0.5*N*log(N)*L | 0.5*N*log(N)*L | 0.5*N*log(N)*L |
| REP | 0.5*N*log(N)*L | -- | 0.5*N*log(N)*L | -- | -- |
| PM | L*N | L*N | -- | -- | -- |
| Sort | L*N*R | -- | -- | (2*L)*log(2*L)*N*R , [ 2L22r.L2 2r.LL]*N*R | -- |
| Total | L*N*(log(N) + 1), L*N*log(N) + L2r.LL*N*R | L*N | L*N*log(N) | L*N*0.5*log(N) + (2L)*log(2L)*N*R, L*N*0.5*log(N) + 2L2 2r.LL*N*R | 0.5*N*log(N)*L |
| Total per info bit | L*(log(N) + 1)/R, L*(log(N) + 1)/R +L | L/R | L*log(N)/R | L*0.5*log(N)/R + (2L)*log(2L), [L* 0.5*log(N)/R + 2L2 2r.LL] | 0.5*L*log(N)/R |

Note that even with “lower latency” construction, the throughput is limited to maximum of 1 info-bit/clock cycle. For a Fclk=1GHz, this will amount to 1 Gbps which is still much lower than the LDPC peak Tput that can be achieved with the same clock. 2r bit joint processing has been proposed in literature [8] to improve throughput by factor of r, however the complexity of these schemes are 22r larger and which suggests that chip area may scale by a much larger amount (this is mainly due to increase in sorting complexity). Hence the cost for achieve EMBB throughputs with polar decoders seems prohibitively large.

# Performance Comparison

In this section we compare complexity vs. performance to attain 1% BLER. Number of iterations (LDPC, Turbo)/list-sizes (polar) are varied to showcase the tradeoff between complexity and performance. Each of the dots in the plot below correspond to different iterations/list-sizes translated to absolute total number of nominal operations. LDPC decoder assumes “flooding”, while turbo assumes max-log-MAP. Max*-log-MAP will improve performance at the cost of complexity.

As can be seen from the plots below, there is a crossover point in terms of LDPC complexity/performance vs. polar wherein LDPC is better than polar for higher # of iterations. Latency/throughput is not considered here, and in this respect, polar definitely has a disadvantage as will be shown in next section.

Proposal 1: The performance and computation complexity tradeoff should be considered when evaluating and selecting channel codes for NR.

# Further Discussion

From the above results, we find that new code architectures can outperform existing LTE turbo code in both performance and computational complexity. This can translate into more efficient implementations in terms of hardware or power savings. For future study, it is important to also consider other aspects to channel code which are important for forward compatibility and overall system design.

## Decoding Latency

It is equally important to address decoding latency per codeblock as well as throughput. For example, in the case of Polar codes, the decoding latency for each code block on the SC actually limits the peak throughput that can be achieved for short bursts, even though there are methods in literature to pipeline blocks to efficiently address extremely large packet sizes. Moreover, when considering SC-L decoding, optimized hardware designs implemented and published for Polar codes seem to at best achieve 1-2 Gbps decoding [6].

LDPC on the other hand can achieve very high throughputs (e.g. 20 Gbps) as it lends itself well with parallelizability and also building low-area decoders.

Turbo decoder is in the middle of the pack (e.g. 2-3 Gbps), as though it has limitations in parallelizability which have been addressed by interleaver design to reduce memory conflicts amongst MAPs, the code design itself is fundamentally inefficient since it achieves the rate compatibility through puncturing a large fraction of bits.

Proposal 2: Code block decoding latency should be considered when evaluating and selecting channel codes for NR.

## Implementation Area

In order to address low decoding latency and high throughput, the the hardware implementations associated with each coding technique should also be considered. As has been pointed out in previous contributions (e.g., [3]), the area of existing implementations for the EMBB coding candidates can be different and may favour LDPC for support both high throughput as well as low decoding latency, and the routing complexity associated with quasicyclic parity matrix structures has evolved to be both computational and hardware efficient.

For polar codes, it was mentioned in an earlier section that the sorting may be implementable across a many-cycle algorithm, or be implemented in hardware to speed up latency. Both cases offer tradeoffs which may be prohibitive when applying to EMBB.

# Conclusions

Proposal 1: The performance and computation complexity tradeoff should be considered when evaluating and selecting channel codes for NR.

Proposal 2: Code block decoding latency should be considered when evaluating and selecting channel codes for NR.

# References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- “Requirements and Evaluation”, R1-162209, Qualcomm Incorporated, RAN1 84b, Busan Korea.

- “Performance and complexity of Turbo, LDPC and Polar codes”, Nokia-ALU, RAN1 84b (R1-162897), Busan, Korea

- L.Bahl, J.Cocke, F.Jelinek, and J.Raviv, "Optimal Decoding of Linear Codes for minimizing symbol error rate", IEEE Transactions on Information Theory, vol. IT-20(2), pp. 284-287, March 1974.

- J. Chen, A. Dholakia, E. Eleftheriou, M. Fossorier, X-Y Hu, “Reduced-complexity Decoding of LDPC codes”, IEEE Transactions on Communications, vol. 53, no. 8, August 2005.

- G. Sarkis, P. Giard, A. Vardy, C. Thibeault, and W. J. Gross, “Unrolled polar decoders, part ii: Fast list decoders,” arXiv preprint arXiv:1505.01466, 2015

- A. Alamdar-Yazdi and F. R. Kschischang, “A simplified successivecancellation decoder for polar codes,” IEEE Commun. Lett., vol. 15, no. 12, pp. 1378–1380, 2011

- B. Yuan, “Algorithm and VLSI Architecture for Polar Codes Decoder”, Ph. D. thesis, Univ. of Minnesota, July 2015.