3GPP TSG RAN WG1 Meeting #85 	R1-164732

Nanjing, China, May 23-27, 2016

Agenda Item:	7.2.2

Source:	Huawei, HiSilicon, CMCC, ETRI, Intel, Samsung, Nokia

Title:	Discussion on frequency dependent channel characteristics for InH and UMi

Document for:	Discussion and decision

Introduction

In the RAN1 #84bis meeting in Busan, the LSP for indoor office scenario has been agreed in [1], also a proposal for UMi [6]. Since there were some comments on the frequency dependency on LSP, some LSP were put into the square bracket.

In this contribution, based on further investigation on related topics, it is proposing collection of frequency dependency measurement result to facilitate the understanding on frequency dependent channel characteristics.

Summary on the frequency dependent LSP study

Measurements at different frequencies have been applied for indoor and outdoor scenarios. Proposals in [2][3][4][5][6][7][8][9][10][11][12][13]have revealed the frequency dependency observation from measurement of  different resources.

Huawei

From Huawei’s proposal [2][7], frequency dependency on LSP such as DS, ASA and ZSA can be observed for frequency range up to 100 GHz. Delay spread and angular spread of four to six frequencies are illustrated as following.

(a)

(b)

Figure 1. The Delay and angular spread frequency dependency characteristic,(a)DS (b)ZSA

CMCC

Similar observations are demonstrated in [5] from CMCC. 14GHz and 28GHz measurements are carried out in this scenario for delay spreads. And another measurement cover from 3.5GHz to 28GHz at the same positions is also taken. It could be observed a slight trend that the mean value of rms delay spread decreases while the centre frequency increases.

HHI

Besides, in contribution [3] from Fraunhofer HHI, Indoor LSP modeling of the 7 large-scale parameters is given with measurement at 10, 25, 40, 55, and 70 GHz. Delay spread (DS), azimuth spread of arrival/departure (ASA/ASD), elevation spread of arrival/departure (ESA/ESD), K-factor (KF), and shadow fading (SF) have been analyzed. A generic frequency-dependent offset on mean and standard deviation is proposed and initial parameters are presented.

lgDS=log10DS1s~NμlgDS+γlgDSlog10fc1GHz, σlgDS2

Table 1. plausible values for model based on measurement at 10, 25, 40, 55, and 70 GHz

| Scenarios |  | UMi Street Canyon |  |  |  | Indoor |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS |  | NLOS |  | LOS | NLOS |
|  |  | O2O | O2I | O2O | O2I |  |  |
| lgDS=log10DS1s | μlgDS | -7.1 |  | -6.8 |  | -6.9 | -7.3 |
|  | γlgDS | -0.75 |  | -0.43 |  | -0.6 | -0.2 |
|  | σlgDS | 0.38 |  | 0.36 |  | 0.23 | 0.18 |

Qualcomm

Also in proposal [4] from Qualcomm, the statistics of delay spread are studied based on indoor and outdoor measurements at 2.9, 29 and 61 GHz. delay spread for NLOS links generally decrease with increase in frequency in the both indoor and outdoor setting. 2.9G Hz, 29G Hz and 61G Hz result in shopping mall measurement are shown in Figure 4. As in the indoor office setting, an increase in frequency reduces the RMS delay spread for NLOS links.

Figure 2. CDF of RMS delay spread across all transmitter and receiver locations for NLOS in the InH(left) shopping mall setting(right).

Aalto University

Aalto University reports frequency dependency of large-scale parameters based on ray-tracing, which was carefully optimized to fit the measured large-scale parameters at 15, 28 and 63 GHz. The delay and angular spreads studied in shopping mall and open square show decreasing trend as the frequency increases in most cases, with some cases of frequency independence. The frequency dependence of large-scale parameters can be modeled as a random variable, which normally shows decreasing trend as the frequency increases, and as a special case, they are frequency independent.

Figure 3. Fitting result of DS and ASA frequency dependency

CATT

In proposal [8] from CATT, the statistics of delay spread are studied based on indoor measurements at 26, and 39.5GHz. The delay spread for LOS and NLOS links shows a significantly decrease with the frequency increase.

Table 1 CATT frequency dependency of delay spread

| Scenarios |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS |  | Office NLOS |  |
|  |  | 26G | 39.5G | 26G | 39.5G |
|  |  | 1.37 | 0.88 | 1.39 | 0.42 |
|  |  | 0.32 | 0.32 | 0.27 | 0.56 |

Ericsson

In proposal [6] [9]from Ericsson, the statistics of delay spread are studied based on indoor  measurements at  5.8, 14.8,  and 58.7 GHz and 2.44, 14.8, and 58.7GHz for  outdoor. The delay spread in InH and UMi shows no monotonic trend as a function of frequency. In [9], the Angle spread in azimuth in NLOS shows an increase trend with frequency increase.

Figure 4. RMS delay spread from Ericsson for UMi

Figure 5. The ASA from Ericsson for InH.

Samsung

Not only for indoor, but also the UMi scenario, there is joint contribution [6]  illustrated the frequency dependency in large-scale and small-scale parameters. Measurements from Samsung are described in Figure 4 shows a frequency dependency trend for RMS delay spread and angular spread.

(a)                                                                (b)

(c)                                                                    (d)

Figure 6. Delay and angular spread collection for UMi from Samsung collection,(a)DS (b)ASD (c)ASA (d)ZSA

Intel

The ray tracing data from Intel shows a monotonous decrease trend of delay spread with frequency increase based on frequency 6GHz,15GHz, 28GHz and 73.5GHz [6].

Figure 7. RMS delay spread from Intel

Qualcomm

The statistics of delay spread are studied based on outdoor measurements at 2.9 and 29GHz. the median of delay spread for NLOS links generally decrease with increase in frequency with outdoor configuration.

Figure 8. RMS delay spread from Qualcomm

NYU

The measurement data from NYU shows a monotonous increase trend of delay spread with frequency increase based on frequency 28GHz, and 73GHz [6].

Figure 9. RMS delay spread from NYU

ETRI

The measurement data from ETRI shows a monotonous decrease trend of delay spread with frequency increase based on frequency 28GHz and 38GHz [6].

Figure 10. RMS delay spread from ETRI

Literature survey

Additional, the frequency dependency characteristic also be researched in some papers. In [10], three frequency 2GHz, 5GHz and 17GHz are studied for the RMS delay spread in indoor scenario. A decreasing frequency dependency trend can be observed as show in Figure 11.

Figure 11. RMS delay spread from [10]

In [11][12][13], the similar trend also can be observed, the RMS delay spread goes decrease with the frequency increase.

Based on all the scenarios data set from all company, take delay spread as a typical example, the frequency dependency characteristic can be observed as below:

Table 2. The DS frequency dependency summary of all sections

|  | InH | UMi | shopping mall |
| --- | --- | --- | --- |
| Huawei | yes | N/A | N/A |
| CMCC | yes | N/A | N/A |
| HHI | yes | N/A | N/A |
| Qualcomm | yes | yes | yes |
| Aalto | N/A | N/A | yes |
| CATT | yes | N/A | N/A |
| Samsung | N/A | yes | N/A |
| Intel | N/A | yes | N/A |
| ETRI | N/A | yes | N/A |
| Ericsson | no | no | N/A |
| [11] | yes | N/A | N/A |
| [12] | yes | N/A | N/A |
| [13] | yes | N/A | N/A |
| [14] | N/A | yes | N/A |

Observation: Based on the references investigated in this contribution, frequency dependency on channel characteristics including delay spread and angular spread can be observed in almost all of the references except one.

Proposal: Frequency dependent channel characteristics should included in the model of this SI, e.g. for delay spread and angular spread.

Conclusions

In this contribution, based on further investigation on related topics, it is proposing collection of frequency dependency measurement result as observation.

Observation: Based on the references investigated in this contribution, frequency dependency on channel characteristics including delay spread and angular spread can be observed in almost all of the references except one.

Proposal: Frequency dependent channel characteristics should included in the model of this SI, e.g. for delay spread and angular spread.

- Acknowledgement

Thanks for the professor Haneda Katsuyuki from Aalto University provided valuable results for supporting the discussion of this contribution.

- References

R1-163472, “Large-scale and small-scale channel modeling for indoor hotspot”, Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI, CATT, Busan, April. 11-15, 2016.

R1-161608, “Frequency dependency for large scale parameters”, Huawei, HiSilicon, Ljubljana, March. 14-16, 2016.

R1-161698, “Frequency-dependent LSP modeling”, Fraunhofer HHI, Ljubljana, March. 14-16, 2016.

R1-161665, “Delay spread for indoor office, outdoor open square and street canyon, and shopping mall environments”, Qualcomm, Ljubljana, March. 14-16, 2016.

R1-161658, “Large scale parameters for multiple scenarios”, CMCC, Ljubljana, March. 14-16, 2016.

R1-162718, “Updated joint proposal on large-scale and small-scale parameters for UMi scenario”, Samsung, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, ETRI, Busan, April. 11-15, 2016.

R1-162243, “Discussion on frequency dependent small-scale parameters”, Huawei, HiSilicon, Busan, April. 11-15, 2016.

R1-162311, “Analysis on measurement data and proposed modeling”, CATT, Busan, April. 11-15, 2016.

R1-163254, “On the frequency dependence of LSPs”, Ericsson, Busan, April. 11-15, 2016.

P. Nobles, F. Halsall, ”Delay spread and Received power measurements within a Building at 2 GHz, 5 GHz, and 17 GHz”,In Proc. IEE ICAP 1997.

T.Jämsä,V.Hovinen,A.Karjalainen,J.Iinatti,”Frequency Dependency of Delay Spread and Path Loss in Indoor Ultra-Wideband Channels”, in Proc. IET UWBSTA, 2006.

R. Müller, S. Häfner, D. Dupleich, et al., ” Simultaneous Multi-Band Channel Sounding at mm-Wave Frequencies”, in Proc. EuCAP 2015.

Weiler, ” Simultaneous millimeter-wave multi-band channel sounding in an urban access scenario”, in Proc. EuCAP 2015