3GPP TSG RAN1 Meeting #85 	R1-164833

# Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | xxxx | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the usage of CEModeA and CEModeB for eMTC in TS 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CEModeA and CEModeB are only defined for BL/CE UE in RRC_CONNECTED state with a configured CEMode. For BL/CE UE in RRC_IDLE state, the CEMode has not been configured, and PRACH CE level should be used. In current specification, the description related to "If a UE is configured with CEModeA" or "If a UE is configured with CEModeB" does not cover the case that UE has not been configured with a CEMode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For BL/CE UE without a configured CEMode, “CEModeA” means UE in PRACH CE level 0 or 1, and “CEModeB” means UE in PRACH CE level 2 or 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description related to "If a UE is configured with CEModeA" or "If a UE is configured with CEModeB" does not cover the case that UE has not been configured with a CEMode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.213 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 5.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits transmitted in codeword  on the physical uplink shared channel in one subframe, shall be scrambled with a UE-specific scrambling sequence prior to modulation, resulting in a block of scrambled bits  according to the following pseudo code

Set i = 0

while

if 		// ACK/NACK or Rank Indication placeholder bits

else

if  	// ACK/NACK or Rank Indication repetition placeholder bits

else			// Data or channel quality coded bits, Rank Indication coded bits or ACK/NACK coded bits

end if

end if

i = i + 1

end while

where x and y are tags defined in 3GPP TS 36.212 [3] clause 5.2.2.6 and where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  corresponds to the RNTI associated with the PUSCH transmission as described in clause 8 in 3GPP TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PUSCH for a given block of  subframes. For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including invalid subframes where the UE postpones the PUSCH transmission. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

Throughout this specification, where “CEModeA” or “CEModeB” is used with reference to a BL/CE UE which does not have a CEMode configured, “CEModeA” means UE in PRACH CE level 0 or 1, and “CEModeB” means UE in PRACH CE level 2 or 3.

Up to two codewords can be transmitted in one subframe, i.e., . In the case of single-codeword transmission, .

#### < Unchanged parts are omitted >