3GPP TSG RAN WG1 Meeting #85		                                                  R1-164837

Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | XXXX | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the supported PUCCH format for CEModeA in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For the supported PUCCH formats of a BL/CE UE, we have the following agreements: RAN1#83 agreement: - PUCCH format 1b/2b/3 are not supported by Rel-13 LC/CE UE for FDD. - PUCCH format 2b/3 are not supported by Rel-13 LC/CE UE for TDD. - For CE mode A, LC/CE UE supports PUCCH format 1/1a/2/2a only; additionally LC/CE UE supports PUCCH format 1b for TDD - For CE mode B, LC/CE UE supports PUCCH format 1/1a only. RAN1#84 agreement: - In TDD when PUCCH is not repeated, when ACK/NAK and CSI are due in the same subframe, for the cases when PUCCH format 2b is used in LTE, PUCCH format 2b is used for LC/CE UEs as in LTE. From these agreements, PUCCH format 2b should not be supported for FDD and a BL/CE UE configured with CEModeA. The specification is not consistent with the agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the support of PUCCH format 2b for FDD and a BL/CE UE configured with CEModeA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The supported PUCCH formats for FDD and a BL/CE UE configured with CEModeA are not consistent with the agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.1.1	PUCCH format information

#### < Unchanged parts are omitted >

For FDD and a BL/CE UE configured with CEModeA, only PUCCH formats 1, 1a, 2, 2a are supported. For TDD and a BL/CE UE configured with CEModeA, only PUCCH formats 1, 1a, 1b, 2, 2a, 2b are supported.

For a BL/CE UE configured with CEModeB, only PUCCH formats 1, 1a, are supported.

If each of the serving cells configured for the UE has frame structure type 1, UE procedures for HARQ-ACK feedback are given in subclause 10.1.2.

If each of the serving cells configured for the UE has frame structure type 2, UE procedures for HARQ-ACK feedback are given in subclause 10.1.3.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 1, UE procedure for HARQ-ACK feedback is given in subclause 10.1.2A.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 2, UE procedure for HARQ-ACK feedback is given in subclause 10.1.3A.

Table 10.1.1-1: code rate  corresponding to higher layer parameter maximumPayloadCoderate-r13

| Value of maximumPayloadCoderate-r13 | Code rate |
| --- | --- |
| 0 | 0.08 |
| 1 | 0.15 |
| 2 | 0.25 |
| 3 | 0.35 |
| 4 | 0.45 |
| 5 | 0.60 |
| 6 | 0.80 |
| 7 | Reserved |

Table 10.1.1-2: Number of PRBs for PUCCCH format 4  corresponding to higher layer parameter numberOfPRB-format4-r13

| Value of numberOfPRB-format4-r13 |  |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | Reserved |

#### < Unchanged parts are omitted >