3GPP TSG RAN WG1 Meeting #85	R1-164909

Nanjing, China 23rd - 27th May 2016

Source:	Panasonic

Title: 	Discussion on reference signal design for shortened TTI

Agenda Item:		6.2.10.1

Document for:		Discussion and Decision

Introduction

This contribution is update of R1-162531 and continues the discussion on some details of RS design.

Discussion

In our previous contributions [1][2], we showed general views on reference signal design of different channels, for example PDSCH, PUCCH and PUSCH (afterwards sPDSCH, sPDCCH, sPUCCH and sPUSCH will be used to refer to different channel specially for shortened TTI). Different channel may have different requirement on reference signal, for example

- For sPDCCH, enough channel estimation performance and to avoid unnecessary overhead should be considered.

- For sPDSCH, MIMO or higher rank may need to be supported as it contributes a lot on throughput

- For sPUSCH/PUCCH, higher PAPR/CM should be avoided and to obtain sufficient energy from lower transmission power of UE.

But on the other hand, there could be some common design targets among different channels as follows,

- Latency reduction gain should be guaranteed as much as possible

- Trade-off  between overhead and performance should be considered

Based on above considerations, we think shared reference signals which are transmitted in earlier symbols than associated data may be a good approach to satisfy/compromise above requirements for DL. Here shared reference signals mean reference signals are common among multiple sTTIs in the same subframe.

So we propose

Proposal 1: Shared reference signals which are transmitted in earlier symbols should be considered for shortened TTI for DL.

Following Fig.1 is an example to elaborate this proposal as sDMRS design for sPDSCH/sPDCCH. It shows the case that PDCCH region and TTI length is 1 OFDM symbol. Two OFDM symbols after PDCCH region are used to transmit sDMRSs which are shared by later sTTIs in the same subframe for demodulation. How to map those sDMRSs to different sTTI is FFS. The same method can be applied to other TTI lengths.

Figure 1 sDMRS design for downlink sPDSCH/sPDCCH

Even the reference signal is not shared i.e. dedicated RS, to locate the reference signal in earlier symbol is useful to reduce the latency especially for DL. Therefore,

Proposal 2: Dedicated reference signals which are transmitted in earlier symbols should be considered for shortened TTI for DL.

To use sDMRS for shortened TTI has merit of supporting higher rank and MIMO/beamforming. But it also introduces complexity and overhead. Another possibility is legacy CRS is reused at least for downlink. For example CRSs in 1st OFDM symbol is used for demodulating sTTI located in OFDM symbol #1, #2 and #3. CRS in 4th OFDM symbol is used for demodulating sTTI located in OFDM symbol #4, #5 and #6. So we propose

Proposal 3: CRS which is not later than associated sTTI could be considered for demodulating sPDCCH and sPDSCH.

Figure 2 Reusing CRS for shortened TTI

It is FFS whether both CRS based demodulation and sDMRS base demodulation should be supported or not. If both are supported, RRC signalling would indicate whether legacy CRS based demodulation or sDMRS based demodulation is used when sTTI is configured to UE.

For UL, basically we prefer existing DMRS position considering the main usage of sTTI length is 3/4 symbol or 7 symbols. There is no latency difference between DMRS transmitted in 1st symbol or DMRS transmitted in 4th symbol. The spec impact is also less. So we propose

Proposal 4: Existing DMRS pattern is used for sTTI in uplink.

In [3], transient period is defined before and after the RF power change. In this transient period, EVM is not defined and the receiver may not use such symbol. Depending on the previous/after transmission, this transient period lies within actual data transmission, which reduces effective data/RS symbols. For shorter TTI, the impact of influenced transient period is larger than 1ms TTI. Therefore, we propose

Proposal 5: Transient period should take into account for sTTI discussion.

Conclusion

In this contribution we discussed general issues and some examples of reference signal design for shortened TTI. Based on these discussions, we propose

Proposal 1: Shared reference signals which are transmitted in earlier symbols should be considered for shortened TTI for DL.

Proposal 2: Dedicated reference signals which are transmitted in earlier symbols should be considered for shortened TTI for DL.

Proposal 3: CRS which is not later than associated sTTI could be considered for demodulating sPDCCH and sPDSCH.

Proposal 4: Existing DMRS pattern is used for sTTI in uplink.

Proposal 5: Transient period should take into account for sTTI discussion.

- Reference

[1] R1-160720, “Physical layer aspects of latency reduction for DL”, Panasonic

[2] R1-160721, “Physical layer aspects of latency reduction for UL”, Panasonic

[3] 3GPP TS 36.101 V12.6.0 (2014-12)