3GPP TSG-RAN WG1#87	R1-1612355

Reno, USA 14th - 18th November 2016

Source:	Ericsson

Title:	Categorizing Type I and II CSI Feedback

Agenda Item:	7.1.3.3

Document for:	Discussion and Decision

# Introduction

In RAN1#85, the following high level framework was agreed for NR CSI reporting:

Agreements:

      - NR supports CSI reporting with two types of spatial information feedback

        - Type I feedback: Normal

  - Codebook-based PMI feedback with normal spatial resolution

        - Type II feedback: Enhanced

  - “Explicit” feedback and/or codebook-based feedback with higher spatial resolution

      - For Type I and II, CSI feedback per subband as well as wideband feedback are supported

      - For Type I and II, beam-related feedback can be included

In this contribution, we provide an overview of Type I and Type II CSI feedback, and suggest how they can be defined.

# Discussion

In order to make use of Type I and II feedback definitions, it is first useful to discuss the motivations for defining these two types.  SU-MIMO does not require extremely high resolution CSI feedback, as can be seen from the modest gains of the subband beam selection in Configs 2-4 over Config 1 in the Rel-13 Class A codebooks [1].  This is because SU-MIMO principally relies on the UE to suppress inter-layer interference.  Consequently, schemes where a single beam is selected in wideband and/or subband feedback (as in Rel-10 to Rel-13 dual stage codebook designs), or where a single wideband coefficient is used per layer.(as in Rel-8 2 and 4 port single stage codebook designs) are sufficient.

On the other hand, high resolution CSI feedback is quite beneficial for MU-MIMO operation, as can be seen from the Rel-14 advanced CSI studies [2].  Since MU-MIMO uses multiple antennas at eNB to spatially suppress interference to different UEs, this higher resolution CSI feedback enables more precise nullforming at eNB, thereby substantially improving MU-MIMO performance.  Furthermore, such high resolution feedback needs to be frequency selective in order to provide the best performance [3].  In order to avoid very high overhead from this frequency selective feedback, long term channel properties can be used.  A high resolution version of the W1/W2 framework used in Rel-10 through Rel-13 is well suited to this task, wherein wideband reduced space channel approximation / parameterizations are combined using multiple non-zero amplitude combining weights.  A detailed description of this framework is given in [4].

Observations:

Type I CSI feedback should:

- Be optimized for SU-MIMO operation.

- Require less overhead

Type II CSI feedback should:

- Be optimized for MU-MIMO operation

- Require more overhead, but efficiently represent the channel

  - Wideband, long term channel knowledge should be used to reduce overhead

- Provide accurate subband approximations of the channel

Proposals

- Type I schemes are those where a single DFT beam is selected in wideband and/or subband feedback, or where a single wideband coefficient is used per layer.

- Type II schemes are those where each subband is represented as combination of wideband basis / channel approximation using multiple non-zero amplitude combining weights.

# Conclusions

In this contribution, we have discussed how techniques for Type I and Type II can be defined We made the following observations:

Observations:

Type I CSI feedback should:

- Be optimized for SU-MIMO operation.

- Require less overhead

Type II CSI feedback should:

- Be optimized for MU-MIMO operation

- Require more overhead, but efficiently represent the channel

  - Wideband, long term channel knowledge should be used to reduce overhead

- Provide accurate subband approximations of the channel

Based on these observations, we propose:

Proposals:

- Type I schemes are those where a single DFT beam is selected in wideband and/or subband feedback, or where a single wideband coefficient is used per layer.

- Type II schemes are those where each subband is represented as combination of wideband basis / channel approximation using multiple non-zero amplitude combining weights.

# References

R1-155678, “FD-MIMO Class A Codebook Design”, Ericsson, 3GPP TSG RAN WG1 Meeting #82bis, Malmö, Sweden, 5th - 9th October 2015

R1-1609854, “Orthogonal vs. non-orthogonal basis for advanced CSI reporting”, Ericsson, 3GPP TSG RAN WG1 Meeting #86bis, Lisbon, Portugal, October 10-14, 2016

R1-1609855, “Phase and amplitude vs. phase only beam combining for advanced CSI reporting”, Ericsson, 3GPP TSG RAN WG1 Meeting #86bis, Lisbon, Portugal, October 10-14, 2016

R1- 1612351, “Phase and amplitude vs. phase only beam combining for advanced CSI reporting”, Ericsson, 3GPP TSG RAN WG1 Meeting #87, Reno, USA November 14-18, 2016