3GPP TSG RAN WG1 #87	R1-1612482

# Reno, USA 14th - 18th November 2016

Agenda item:	7.1.3.1

Source: 	Samsung

Title: 	DMRS-based precoder cycling and simulation results

Document for:	Discussion and Decision

# Introduction

In RAN1#86b, the following agreement was made [1].

Agreements:

- DL and UL transmission techniques should be studied to provide robustness against imperfect CSI and mobility

  - e.g.) The techniques using a subset of beams/precoders may include beam cycling, beam broadening, etc.

  - e.g.) In case of DL and FDD, this technical scope may include semi-open-loop MIMO technique being discussed in LTE eFD-MIMO.

According to the agreement (highlighted text), transmissions techniques using a subset of beams/precoders for beam cycling should be studied, the scope of which includes semi-open-loop transmission technique in LTE eFD-MIMO. The companion contributions [2-3] proposes that precoder cycling should be supported for both DL and UL DMRS-based spatial multiplexing. This contribution provides simulation results to show performance gain of semi-dynamic precoder cycling transmission over closed loop transmission in high mobility scenarios.

# Precoder cycling transmission schemes for evaluation

In this evaluation, we assume that a UE feeds back the first PMI (i1,1,i1,2) using LTE Rel. 13 Class A codebook that indicates L = 4 beams (assuming Codebook-Config = 2, 3, 4) for precoder cycling based DL transmission by the gNB/TRP. Depending on whether co-phase for the two polarizations is cycled or not, we have the following two types of rank 1 precoder cycling:

- 4 beams (indicated by (i1,1,i1,2) feedback) are cycled through by the gNB/TRP without any co-phase cycling. For example, the co-phase for each beam is fixed and is reported by the UE, where this reporting is WB.

- Both 4 beams (indicated by (i1,1,i1,2) feedback) and 4 QPSK co-phase values are cycled through by the gNB/TRP.

For precoder cycling, two different granularities, namely RB level and RE level, are considered in this evaluation. The candidate pre-coder cycling schemes is summarized in Table 1. An illustration of rank 1 pre-coder cycling for 4 beams and 4 co-phase values is shown in Figure 1 assuming cycling direction is in frequency domain. For simplicity, pre-coder cycling across one OFDM symbol is shown in the figure.

Table 1: Precoder cycling schemes

| Scheme | Beam | Co-phase |
| --- | --- | --- |
| Scheme 0 | Cycling (RB level) | No cycling (WB) |
| Scheme 1 | Cycling (RE level) | No cycling (WB) |
| Scheme 2 | Cycling (RB level) | Cycling (RB level) |
| Scheme 3 | Cycling (RB level) | Cycling (RE level) |
| Scheme 4 | Cycling (RE level) | Cycling (RB level) |
| Scheme 5 | Cycling (RE level) | Cycling (RE level) |

Figure 1: An illustration of rank 1 precoder (beam and co-phase) cycling

# Simulation results

To compare the performance of candidate precoder cycling schemes in Table 1, non-full-buffer system-level evaluation is carried out for UMa-500m channel model (36.814) in heavy (70% target RU) traffic loading scenario, where UE speed is assumed to be 120 kmph. The SU-MIMO rank 1 transmission with SB scheduling (where SB is 4 PRBs) is considered in the simulation. The detailed results are provided in Table 3 and Table 4 in the Appendix. The results are provided for 16 antenna ports with (N1,N2) = (2,4) and 32 antenna ports with (N1,N2) = (2,8), where we assume that the first dimension is horizontal and the second dimension is vertical. The relevant simulation parameters are enlisted in Table 2.

The performance gains with close-loop (CL) transmission as reference are shown in Figure 2 - Figure 7 for Config 2, 3, and 4, and 16 and 32 ports. From these results, we can make the following observation.

Observation:

- Precoder cycling based transmission shows significant performance gain over closed loop transmission: ~8-20% avg. UPT gain, ~18-65% 5% UPT gain.

- Co-phase cycling (Scheme 2-5) achieves additional gain when compared with no co-phase cycling (Scheme 0-1): ~4-10% additional avg. UPT gain, ~15-35% additional 5% UPT gain

- RE level cycling (Schemes 3-5) achieves additional gain when compared with RB level cycling (Scheme 2): ~1% additional avg. UPT gain, 6-10% additional 5% UPT gain.

- Performance of Scheme 3 (RB level beam cycling, RE level co-phase cycling) and Scheme 5 (RE level beam and co-phase cycling) are very close.

Based on these observations, we make the following proposal.

Proposal: Precoder cycling based transmission using WB beam group feedback is supported in NR. For precoder cycling, at least RB level beam cycling and RE level co-phase cycling is supported.

Figure 2: (N1,N2) = (2,4), Config = 2

Figure 3: (N1,N2) = (2,4), Config = 3

Figure 4: (N1,N2) = (2,4), Config = 4

Figure 5: (N1,N2) = (2,8), Config = 2

Figure 6: (N1,N2) = (2,8), Config = 3

Figure 7: (N1,N2) = (2,8), Config = 4

# Conclusions

In this contribution, simulation results are provided for precoder cycling transmission scheme relying on WB beam group feedback, as proposed in [2-3]. The observations and proposals made are summarized as follows.

Observation:

- Precoder cycling based transmission shows significant performance gain over closed loop transmission: ~8-20% avg. UPT gain, ~18-65% 5% UPT gain.

- Co-phase cycling (Scheme 2-5) achieves additional gain when compared with no co-phase cycling (Scheme 0-1): ~4-10% additional avg. UPT gain, ~15-35% additional 5% UPT gain

- RE level cycling (Schemes 3-5) achieves additional gain when compared with RB level cycling (Scheme 2): ~1% additional avg. UPT gain, 6-10% additional 5% UPT gain.

- Performance of Scheme 3 (RB level beam cycling, RE level co-phase cycling) and Scheme 5 (RE level beam and co-phase cycling) are very close.

Proposal: Precoder cycling based transmission using WB beam group feedback is supported in NR. For precoder cycling, at least RB level beam cycling and RE level co-phase cycling is supported.

# References

- RAN1#86b, Chairman’s notes

- R1-1612478, “DMRS-based spatial multiplexing for DL NR MIMO,” Samsung

- R1-1612479, “DMRS-based spatial multiplexing for UL NR MIMO,” Samsung

Appendix: Simulation Assumptions and Results

Table 2: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Heavy load 70% Target RU, Lambda = 3) |
| Channel model | UMa-500m (36.814) |
| Number of BS (H,V) antenna elements | (8,8) for 16 ports and (8,16) for 32 ports, x-polarized, subarray partition |
| (N1,N2, P) | 16 ports: (2,4,2) and 32 ports: (2,8,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| UE speed | 120 kmph |
| SU/MU pre-coding | CB |
| Scheduling | SU, Proportional fair, SB scheduling (SB = 4 PRBs) |
| Channel estimation | Non-ideal |
| Transmission rank | 1 |
| Receiver | MMSE-IRC |
| Codebook | Rel. 13 Class A: (O1,O2) = (8,8), Codebook-Config = 2,3,4 |

Table 3: Non-full buffer simulation results: 16 ports

| Codebook-Config | Scheme | Avg. UPT | 50% UPT | 5% UPT | RU | Avg. UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | CL | 10.56 | 8.60 | 1.67 | 66.6% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 11.66 | 9.71 | 2.10 | 63.5% | 110.5% | 113.0% | 126.0% |
|  | Scheme 1 | 11.66 | 9.74 | 2.11 | 63.4% | 110.5% | 113.3% | 126.5% |
|  | Scheme 2 | 12.43 | 10.82 | 2.52 | 60.1% | 117.8% | 125.8% | 151.5% |
|  | Scheme 3 | 12.51 | 10.83 | 2.69 | 59.9% | 118.5% | 126.0% | 161.2% |
|  | Scheme 4 | 12.46 | 10.83 | 2.62 | 60.0% | 118.0% | 125.9% | 157.2% |
|  | Scheme 5 | 12.52 | 10.89 | 2.64 | 59.8% | 118.6% | 126.6% | 158.2% |
| 3 | CL | 10.46 | 8.47 | 1.62 | 66.9% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 11.43 | 9.42 | 2.01 | 64.1% | 109.3% | 111.3% | 124.2% |
|  | Scheme 1 | 11.58 | 9.52 | 2.12 | 63.5% | 110.7% | 112.5% | 130.8% |
|  | Scheme 2 | 12.33 | 10.66 | 2.54 | 60.5% | 117.9% | 125.9% | 156.3% |
|  | Scheme 3 | 12.43 | 10.85 | 2.65 | 60.1% | 118.8% | 128.2% | 163.6% |
|  | Scheme 4 | 12.34 | 10.71 | 2.58 | 60.3% | 118.0% | 126.5% | 158.9% |
|  | Scheme 5 | 12.38 | 10.72 | 2.65 | 60.1% | 118.4% | 126.6% | 163.4% |
| 4 | CL | 10.40 | 8.48 | 1.59 | 67.1% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 11.48 | 9.39 | 2.07 | 64.0% | 110.4% | 110.8% | 130.8% |
|  | Scheme 1 | 11.51 | 9.55 | 2.05 | 63.8% | 110.7% | 112.7% | 129.2% |
|  | Scheme 2 | 12.29 | 10.64 | 2.51 | 60.6% | 118.2% | 125.5% | 158.2% |
|  | Scheme 3 | 12.34 | 10.75 | 2.64 | 60.4% | 118.7% | 126.8% | 166.6% |
|  | Scheme 4 | 12.32 | 10.62 | 2.56 | 60.5% | 118.5% | 125.3% | 161.7% |
|  | Scheme 5 | 12.37 | 10.621 | 2.63 | 60.2% | 119.0% | 125.3% | 165.8% |

Table 4: Non-full buffer simulation results: 32 ports

| Codebook-Config | Scheme | Avg. UPT | 50% UPT | 5% UPT | RU | Avg. UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | CL | 13.32 | 11.89 | 3.09 | 56.7% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.44 | 13.17 | 3.68 | 54.0% | 108.4% | 110.8% | 118.8% |
|  | Scheme 1 | 14.44 | 13.17 | 3.68 | 54.0% | 108.4% | 110.8% | 118.8% |
|  | Scheme 2 | 14.88 | 13.66 | 4.23 | 52.4% | 111.7% | 114.9% | 136.8% |
|  | Scheme 3 | 14.90 | 13.71 | 4.23 | 52.2% | 111.9% | 115.4% | 136.9% |
|  | Scheme 4 | 14.88 | 13.66 | 4.23 | 52.4% | 111.7% | 114.9% | 136.8% |
|  | Scheme 5 | 14.90 | 13.71 | 4.23 | 52.2% | 111.9% | 115.4% | 136.9% |
| 3 | CL | 13.28 | 11.86 | 3.13 | 56.7% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.37 | 13.20 | 3.73 | 54.2% | 108.2% | 111.3% | 119.2% |
|  | Scheme 1 | 14.37 | 12.98 | 3.68 | 54.2% | 108.2% | 109.4% | 117.7% |
|  | Scheme 2 | 14.79 | 13.61 | 4.09 | 52.6% | 111.4% | 114.8% | 130.7% |
|  | Scheme 3 | 14.81 | 13.57 | 4.20 | 52.4% | 111.6% | 114.5% | 134.2% |
|  | Scheme 4 | 14.81 | 13.51 | 4.16 | 52.6% | 111.5% | 113.9% | 132.9% |
|  | Scheme 5 | 14.83 | 13.67 | 4.22 | 52.4% | 111.6% | 115.3% | 134.7% |
| 4 | CL | 13.19 | 11.68 | 3.00 | 57.0% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.27 | 12.90 | 3.66 | 54.5% | 108.2% | 110.5% | 122.0% |
|  | Scheme 1 | 14.31 | 13.02 | 3.65 | 54.2% | 108.5% | 111.4% | 121.8% |
|  | Scheme 2 | 14.69 | 13.48 | 4.03 | 52.9% | 111.3% | 115.4% | 134.4% |
|  | Scheme 3 | 14.73 | 13.45 | 4.18 | 52.6% | 111.6% | 115.1% | 139.4% |
|  | Scheme 4 | 14.72 | 13.58 | 4.16 | 52.7% | 111.5% | 116.2% | 138.6% |
|  | Scheme 5 | 14.72 | 13.51 | 4.16 | 52.6% | 111.6% | 115.7% | 138.7% |