3GPP TSG-RAN WG1 Meeting #90	R1-1712911

Prague, Czech Republic, 21st – 25th Aug 2017

Agenda Item:	5.2.1.2.7

Source:	Ericsson

Title:	UE capabilities for sTTI

Document for:	Discussion, Decision

# Introduction

In the ongoing WI on shortened TTI and processing time [1]  the UE capabilities need to be defined before completing the work. Although not all aspects have been agreed upon, the following document is intended to trigger some discussion on the needed UE capabilities to specify the support for sTTI and related functionality.

# UE capabilities

The proposed UE capabilities are listed in Table 1 together with a description, and possibly a comment, of the capability.

Table 1: UE capabilities for sTTI

| Capability | Description | Comment |
| --- | --- | --- |
| Max # DL carriers for 2os TTI in FS1 | 2os TTI supported for FS1 on at least one DL cell |  |
| Max # UL carriers for 2os TTI in FS1 | 2os TTI supported for FS1 on at least one UL cell |  |
| Max # DL carriers for 7os TTI in FS1 | 7os TTI supported for FS1 on at least one DL cell |  |
| Max # UL carriers for 7os TTI in FS1 | 7os TTI supported for FS1 on at least one UL cell |  |
| Max # DL carriers for 2os TTI DL + 7os TTI UL in FS1 | 2os TTI DL + 7os TTI UL supported for FS1 on at least one DL cell |  |
| Max # UL carriers for 2os TTI DL + 7os TTI UL in FS1 | 2os TTI DL + 7os TTI UL supported for FS1 on at least one UL cell |  |
| Max # DL carriers for 7os TTI in FS2 | 7os TTI supported for FS2 on at least one DL cell |  |
| Max # UL carriers for 7os TTI in FS2 | 7os TTI supported for FS2 on at least one UL cell |  |
| Max # DL carriers for 2os TTI DL + 7os TTI UL in FS2 | 2os TTI DL + 7os TTI UL supported for FS2 on at least one DL cell |  |
| Max # UL carriers for 2os TTI DL + 7os TTI UL in FS2 | 2os TTI DL + 7os TTI UL supported for FS2 on at least one UL cell |  |
| Simultaneous transmission of sPUSCH and sPUCCH | Separate capability for sTTI, but similar to existing capability | Under discussion if separate or same as 1 ms |
| Number of CSI processes supported TM10 | Separate capability for sTTI, but similar to existing capability | Number of CSI processes not agreed upon |
| Cross-carrier scheduling | Separate capability for sTTI, but similar to existing capability | Not discussed yet if it should be supported |
| 2-level DCI | A two level scheduling mechanism is supported. | Not yet agreed if this should be supported |
| Multi-sTTI scheduling DL | Assignment of multiple sTTIs in a single sPDCCH | Not yet agreed if this should be supported |
| Multi-sTTI scheduling UL | Multiple UL sTTIs grants in a single sPDCCH | Not yet agreed if this should be supported |
| CSI on sTTI | CSI reporting on sTTI supported in addition to 1 ms TTI reports on sTTI | Not discussed yet |
| Transmission modes for sTTI | Similar to existing capabilities but applicable to sTTI |  |
| # os mapping for CRS based 2os sPDCCH | 1os, 2os, or both supported | Under discussion if the different mappings should be associated to different HARQ Ack/UL grant timing. |
| # os mapping for CRS based 7os sPDCCH | 1os, 2os, 3os or all supported | The support of 3os under discussion |
| DMRS based sPDCCH | If DMRS based sPDCCH operation is supported for the supported sTTI lengths | Still not decided on the mapping to physical resources. Combinations reference signals (CRS/DMRS) for sPDCCH and sPDSCH not discussed yet |
| sPUCCH format support | In case multiple sPUCCH formats exist for the same payload size (for example FH and no FH format) the UE need not support all of them | Still under discussion on what formats to support |

It should be noted that the above list need not be exhaustive (pending still ongoing discussions) and that some of the capabilities might be removed (pending still ongoing discussions).

One special consideration is taken relating to the number of layers supported for sTTI where it is proposed to re-use the existing capability signalling for 1 ms operation. In case a higher number of layers is supported for 1 ms, compared to what is possible with sTTI operation, the number of layers are limited by the limit in the specification.

# Conclusion

The document has discussed UE capabilities for sTTI feature. It is proposed to split the UE support into a set of capabilities. It should be noted that the above list need not be exhaustive (pending still ongoing discussions) and that some of the capabilities might be removed (pending still ongoing discussions).

Companies are encouraged to provide feedback on the above list.

# References

- RP-170113, New Work Item on shortened TTI and processing time for LTE, Ericsson, RAN#75, March 2017.