3GPP TSG RAN WG1 Meeting #90	 		   			 		 R1- 1714599

# Prague, Czechia, 21th – 25th August 2017

# Source:	ETRI

# Title:		Discussion on downlink DMRS design

# Agenda item:	6.1.2.3.3

# Document for:	Discussion/Decision

Introduction

The following agreement on the additional DMRS structure was made in the 3GPP RAN1#89 meeting [1]:

Agreements:

- Support additional DMRS symbols with same density in frequency domain compared to front loaded DMRS

  - FFS: Necessity of reduced DMRS density in additional DMRS symbols

In addition, regarding the front-loaded DMRS design for data channel, the following agreements were made in the 3GPP RAN1-NR#2 meeting [2]:

Agreements:

- The working assumption made in RAN1#89 for DM-RS is updated and agreed as follows for CP-OFDM:

- A UE is configured by higher layers with DMRS pattern either from the front-loaded DMRS Configuration type 1 or from the front-loaded DMRS Configuration type 2 for DL/UL:

- Configuration type 1:

- One symbol:

  - Comb 2 + 2 CS, up to 4 ports

- Two symbols:

  - Comb 2 + 2 CS + TD-OCC ({1 1} and {1 -1}), up to 8 ports

    - Note: It should be possible to schedule up to 4 ports without using both {1,1} and {1,-1}.

- Configuration type 2:

- One symbol:

  - 2-FD-OCC across adjacent REs in the frequency domain, up to 6 ports

- Two symbols:

    - 2-FD-OCC across adjacent REs in the frequency domain + TD-OCC (both {1,1} and {1,-1}) up to 12 ports

      - Note: It should be possible to schedule up to 6 ports without using both {1,1} and {1,-1}.

- From UE perspective, frequency domain CDMed DMRS ports are QCLed.

- FFS: Whether the front-load DMRS configuration type for a UE for UL and DL can be different or not.

- Note: If there are significant complexity/performance issues involved in the above agreements, down-selection can still be discussed

In this contribution, based on the above working assumption and agreement, we discuss considerations in designing NR downlink DMRS.

Discussions

Frequency-domain density of additional DMRS

Additional DMRS with higher time-domain density is required to support high Doppler scenario where the channel coherence time is short. As seen in the above agreements, in the RAN1#89 meeting, it was agreed to have the same frequency domain density between front-loaded DMRS and additional DMRS. And in the RAN1-NR#2 meeting, basic patterns for the front-loaded DMRS are decided, which have two different configuration types. Figure 1 shows examples of front-loaded and additional DMRS patterns based on configuration type 2 for 1 or 2 layer cases, where two different frequency-domain densities are employed.

Figure 1. Examples of front-loaded and additional DMRS patterns based on configuration type 2.

It was shown in Figure 2 that the DMRS pattern with high frequency-domain density performs better in low SNR region, whereas that with low frequency-domain density performs better in high SNR region. Hence, it is natural to support configurable frequency domain density for both the front-loaded and additional DMRSs.

Figure 2. Spectral efficiency vs. SNR for the example DMRS patterns

Proposal 1: NR should support configurable frequency domain DMRS density for front-loaded and additional DMRSs.

Scope of DMRS configurability

In NR, DMRS can be configured UE-specifically according to several aspects such as MIMO scheme, frequency/time-domain channel conditions, RS overhead, latency requirement, and so on. By doing so, balancing among the channel estimation performance, overhead reduction, and latency reduction is achievable. However, there is still room for further improvement in spectral efficiency. Consider a scenario where a large number of RBs (e.g., 15 RBs) are allocated to a single UE. If we observe a snapshot of a channel impulse response in the frequency-domain as seen in Figure 3, some parts may experience severe fluctuation (i.e., channel coherence bandwidth is narrow) and other parts may experience weak fluctuation (i.e., channel coherence bandwidth is wide). In this situation, if only one DMRS density/pattern is allowed for the entire UE bandwidth, inefficient DMRS allocation is inevitable. For example, if low density DMRS is allocated within the entire UE bandwidth, channel estimation accuracy will be degraded within the sub-band with the narrow channel coherence bandwidth. Oppositely, if only high density DMRS is used, spectral efficiency will be decreased due to unnecessary DMRS overhead within the sub-band with the wide channel coherence bandwidth. Therefore, employing different DMRS densities/patterns for different sub-bands within a specific UE bandwidth can be considered.

Observation 1: Having different per-RB frequency-domain densities/patterns within a UE bandwidth can be beneficial in achieving higher spectral efficiency, especially when a UE bandwidth is large.

Figure 3. Snapshot of channel impulse response in the frequency-domain (15 RBs)

One may concern that allocating DMRS to single UE with different per-RB frequency-domain density prevents the use of DFT-based channel estimation and interpolation which require uniform DMRS allocation in the frequency domain. This issue can be resolved by carefully implementing interpolation algorithm. One example is multi-stage DFT-based interpolation where interpolation is done in an increasing order of DMRS density.

Assuming two different frequency domain DMRS densities are available, i.e., 2 RSs per RB and 4 RSs per RB, as seen in Figure 4, RBs with 2 RSs per RB is first frequency interpolated to have the 4 RS/interpolated symbol per RB. Then, after further DFT interpolation, we can have the channel information over the entire frequency domain. Time domain interpolation can be further performed.

Figure 4. Example of frequency domain interpolation of non-uniformly allocated DMRSs

Proposal 2: NR should support configurable frequency domain DMRS density/pattern for each RB scheduled to a single UE.

Conclusion

In this contribution, we discussed some important issues in downlink DMRS design including front-loaded DMRS configurability, additional DMRS frequency domain density, and scope of DMRS configurability. Our observations and proposal are reproduced below:

Proposal 1: NR should support configurable frequency domain DMRS density for front-loaded and additional DMRSs.

Observation 1: Having different per-RB frequency-domain densities/patterns within a UE bandwidth can be beneficial in achieving higher spectral efficiency, especially when a UE bandwidth is large.

Proposal 2: NR should support configurable frequency domain DMRS density/pattern for each RB scheduled to a single UE.

References

- 3GPP Chairman’s notes RAN1#89, May 2017.

- 3GPP Chairman’s notes RAN1-NR#2, Jun. 2017.

- R1-163984, “Discussion on phase noise modeling,” RAN1#85, May 2016.

Appendix

Simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30 GHz |
| System bandwidth | 80 MHz |
| Subcarrier spacing | 120 kHz |
| Channel coding | LTE Turbo |
| MCS | 16QAM 3/4 |
| Number of layers | 1 |
| Channel estimation | Realistic |
| Equalizer | LMMSE |
| Channel model | CDL-A (DS = 100ns) |
| TRP antenna configuration | (M,N,P,Mg,Ng) = (4,8,1,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65, directivity 8dB) |
| UE antenna configuration | (M,N,P,Mg,Ng) = (2,4,1,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65, directivity 8dB) |
| Phase noise model | Multi-pole/zero model [3] |