3GPP TSG RAN WG1 Meeting #90                                                         	         R1-1712480

Prague, P.R. Czechia 21th – 25th August 2017

Agenda Item:          5.2.2.3.3

Source: 		Intel Corporation

Title:	Consideration on channel access mechanism for autonomous uplink access

Document for: 	Discussion/Decision

# Introduction

A new work item on ‘enhancements to LTE operation in unlicensed spectrum’ was approved in TSG RAN Meeting #75 [1]. The objective of the new WI from RAN1 perspective is twofold:

- Specify support for multiple starting and ending positions in a subframe for UL and DL on SCell with Frame structure type 3.

- (Starting in RAN1#90): Study, and specify if needed, support for autonomous uplink access with Frame Structure type 3 considering solutions from the L2 latency reduction work item

This contribution is on the second objective in the above, especially on the channel access mechanism for autonomous uplink access. In this contribution, we use AUL and SUL, as short for autonomous UL and scheduled UL, respectively, for simplicity.

# LBT type, priority class, and MCOT sharing

Figure 1: Cat-4 LBT performed by AUL UE

As the AUL transmission is not a transmission scheduled within an eNB’s obtained MCOT, the UE needs to obtain the right to access the medium by itself. Therefore, the Cat-4 LBT is the only LBT type that can be allowed for AUL transmission. Since the AUL transmission is commenced by each UE autonomously, it is also reasonable to let each AUL UE to choose an appropriate LBT priority class by itself and, thereby, the MCOT, depending on its buffer status. On the other hand, it is not foreseen that any change will be necessary for CWS adaptation mechanism defined in Rel-14 eLAA WI. Both SUL and AUL transmissions should be subject to the CWS adaptation upon the reception of HARQ-ACK feedback.

Proposal 1: The Cat-4 LBT is used for AUL transmission. The LBT priority class is selected by each AUL UE.

Proposal 2: The AUL transmission is subject to the CWS adaptation at the UE.

# Mechanism to avoid collision between AUL and with SUL

As it was briefly discussed in our companion paper [2], there is a need to consider how to avoid collisions between the scheduled transmissions and AUL. It is also possible that the AUL transmissions can collide with each other.

TDM’ed AUL access

To avoid collisions between different AUL transmissions and also with SUL, it may be helpful to let each AUL UEs to have different PUSCH starting offset. For instance, an AUL transmission may begin within symbol 0 with some random offset. The UEs are expected to perform self-defer until the chosen starting instance. With this random offset, possible collision between TDM’ed AUL transmissions and also with scheduled DL and UL transmissions can be avoided to some degree. For the partial symbol 0, i.e., between the chosen random starting instance and the start of symbol 1, the CP of symbol 1 is extended like in Rel-14 eLAA design.

Proposal 3: For TDM’ed AUL transmission, a random PUSCH starting position is chosen within the symbol 0. The CP of symbol 1 is extended for the partial symbol transmission in symbol 0.

FDM’ed AUL access

In the case of FDM’ed AUL transmission, the use of random offset can induce unintended blocking between UEs. For instance, a UE starting transmission earlier can make the LBT failed at the UEs who have chosen a relatively longer random offset. Therefore, in the case of FDM’ed AUL transmissions, a common offset is necessary. This common offset can be signaled to UEs.

Proposal 4: For FDM’ed AUL transmission, a common PUSCH starting position is singled to and used by AUL UEs.

Figure 2: Efficient FDM AUL UE multiplexing with single subframe transmission restriction with LBT gap

On the other hand, the FDM of AUL is intended for efficient small packet transmission. Thus, it is not expected that a single interlace transmission by one FDM’ed AUL UE will last for multiple consecutive subframes. If then, the UE may utilize the entire interlace in TDM manner. On the other hand, it is also possible that only a few number of FDM configured AUL UEs are transmitting in a given subframe. Then, the AUL transmissions of a single or a few interlaces may occupy the channel and block other transmissions including scheduled DL/UL, TDM’ed AUL, and also FDM’ed AUL transmissions. Therefore, it may be preferable to limit the FDM’ed AUL to single subframe transmission with vacating the last symbol to create the LBT gap for potential other transmissions as illustrated in Figure 2.

Proposal 5: The FDM’ed AUL transmission is limited to single subframe transmission with last symbol vacated to create LBT gap.

# MCOT sharing with eNB

The ETSI BRAN harmonized standard allows the channel access initiating device to grant an authorization to one or more associated responding devices to transmit on the current operating channel [3]. With this, the eNB can share the MCOT obtained by an AUL UE. The AUL UE can indicate the remaining channel occupancy time in the UCI. It is up to eNB on how to utilize it or even not to utilize it. For instance, the eNB may decide to transmit HARQ-ACK feedback for the AUL transmission by the corresponding UE which obtained the MCOT. In another instance, the eNB may decide to transmit PDCCH/PDSCH to other UEs. As it is allowed by ETSI BRAN harmonized standard, the eNB may transmit without performing LBT, if its transmission follows the preceding AUL transmission within 16 us. Otherwise, the eNB is required to perform 25 us single interval LBT.

Proposal 6: The channel occupancy obtained by AUL UE can be shared with eNB.

# Conclusion

In this contribution, we discussed the channel access mechanism for autonomous uplink access and made the following proposals:

Proposal 1: The Cat-4 LBT is used for AUL transmission. The LBT priority class is selected by each AUL UE.

Proposal 2: The AUL transmission is subject to the CWS adaptation at the UE.

Proposal 3: For TDM’ed AUL transmission, a random PUSCH starting position is chosen within the symbol 0. The CP of symbol 1 is extended for the partial symbol transmission in symbol 0.

Proposal 4: For FDM’ed AUL transmission, a common PUSCH starting position is singled to and used by AUL UEs.

Proposal 5: The FDM’ed AUL transmission is limited to single subframe transmission with last symbol vacated to create LBT gap.

Proposal 6: The channel occupancy obtained by AUL UE can be shared with eNB.

# References

[1] RP-170848, New Work Item on Enhancements to LTE operation in unlicensed spectrum, Nokia, Ericsson, Intel, Qualcomm, 3GPP TSG RAN Meeting #75, March, 2017

[2] R1-1712478, “A framework to enable autonomous uplink access,” 3GPP TSG RAN WG1 #90, Intel Corporation, Aug., 2017

[3] ETSI EN 301 893, Harmonised European Standards, Final draft, V2.1.0, Mar. 2017