3GPP TSG RAN WG1 Meeting                 　　	                       R1-1712752

Prague, Czech Republic 21st – 25th Aug. 2017

Source:	Mitsubishi Electric

Title:	Evaluation of DL DMRS designs

Agenda Item:	6.1.2.3.3 Remaining details on DMRS

Document for: 	Discussion/Decision

Introduction

In the RAN1#89 meeting and NR AH #2, the following agreements were made [1]:

R1-1711784 WF on DMRS design, MediaTek, Huawei, HiSilicon, China Telecom, ZTE, ASUSTek, CeWiT, Spreadtrum, Tejas Networks, CATR, Deutsche Telekom, IITH, Xinwei, MTI, CHTTL, China Unicom, IITM, ITRI Agreements: - The working assumption made in RAN1#89 for DM-RS is updated and agreed as follows for CP-OFDM: - A UE is configured by higher layers with DMRS pattern either from the front-loaded DMRS Configuration type 1 or from the front-loaded DMRS Configuration type 2 for DL/UL: - Configuration type 1: - One symbol: - Comb 2 + 2 CS, up to 4 ports - Two symbols: - Comb 2 + 2 CS + TD-OCC ({1 1} and {1 -1}), up to 8 ports - Note: It should be possible to schedule up to 4 ports without using both {1,1} and {1,-1}. - Configuration type 2: - One symbol: - 2-FD-OCC across adjacent REs in the frequency domain, up to 6 ports - Two symbols: - 2-FD-OCC across adjacent REs in the frequency domain + TD-OCC (both {1,1} and {1,-1}) up to 12 ports - Note: It should be possible to schedule up to 6 ports without using both {1,1} and {1,-1}. - From UE perspective, frequency domain CDMed DMRS ports are QCLed. - FFS: Whether the front-load DMRS configuration type for a UE for UL and DL can be different or not. Note: If there are significant complexity/performance issues involved in the above agreements, down-selection can still be discussed

R1-1711771 WF on signalling DMRS configuration LG Electronics, ZTE Agreements: - The number of front-load DMRS symbols can be 1 or 2 when the number of DMRS ports allocated to UE is equal or less than N - N is 4 for Configuration 1 and 6 for Configuration 2. - FFS the details to determine 1 or 2 symbols

In addition, the following agreement was made regarding CSI-RS density.

R1-1706441 WF on CSI-RS density LG Electronics, InterDigital, Intel, ITRI Agreements: - At least CSI-RS for CSI acquisition, NR supports CSI-RS density d RE/RB/port for x-port CSI-RS - Value(s) of d are at least d=1,1/2. - For d<1, PRB-level comb-type transmission is supported. - FFS whether offset value(s) can be the same or different across antenna ports - FFS on supporting d>1 in the consideration of use case, e.g., NZP CSI-RS for IMR. - FFS on the supported combinations of value(s) of x and d.

The following agreement regarding the number of additional DMRS symbols was made in the RAN1#89 meeting.

R1-1709305 WF on additional DMRS structure Samsung, Qualcomm Incorporated, Ericsson, CATT, Nokia, Alcatel-Lucent Shanghai Bell, vivo, Mitsubishi Electric Co., NTT DOCOMO, INC., ETRI, KT Corp., MediaTek Inc. Also supported by Interdigital Agreements: - Support additional DMRS symbols with same density in frequency domain compared to front loaded DMRS - FFS: Necessity of reduced DMRS density in additional DMRS symbols

In this contribution, we evaluate DMRS configurations focusing on the number of additional DMRS symbols and reduced density DMRS. Both link level and system level simulations results will be shown in the contributions.

The contribution is organized as follows. In Section 2, link level simulation results using the high speed train scenario at 30GHz, are shown to evaluate effectiveness of additional number of DMRS symbols. In Section 3, link level evaluations are conducted to assess the performance of the reduced density DMRS configurations. In Section 4, system level simulation results for the reduced density DMRS configurations are shown. Section 5 concludes the contribution with proposals.

Link level evaluation results: number of extra symbols

It was agreed to support 2-frontloaded symbol configuration to support up to 4 and 6 ports for configuration type 1 and 2, respectively. The agreement has an impact on the number of additional DMRS symbols, since it was agreed to keep the frequency density same as FL. It should be noted that additional DMRS symbols are used to track fading fading channel and number of additional DMRS symbol affects the spectral efficiency.

As mentioned in our contribution on DL DMRS designs [2], the number of extra DMRS symbols that are added in the slot should depend on the condition of the channel. The additional DMRS symbols are used for tracking time-varying channels and it is not necessary to allocate the same number of DMRS symbols.

In this section, we evaluate effectiveness of the number of additional symbols in the high speed train scenario at 30GHz. The evaluation assumptions are shown in Table 1 in the appendix. The DMRS configurations considered in the contribution are shown in Figure 1 and Figure 2. In the evaluation, we consider the configurations which contain one or two additional DRMS symbols. Configuration Type 2 is considered in the evaluation. MIMO transmission and reception are assumed.

| Figure 1 One additional DMRS symbols | Figure 2 Two additional DMRS symbols |
| --- | --- |

Figure 3 Spectral efficiency vs. SNR with different number of additional symbols, K=7dB

Dependency of the spectral efficiency on the number of additional symbols is evaluated with the link level simulation. The simulation result is shown in Figure 3. Form the figure, it is clear that the number of additional symbols have small impact on the tracking capability in the high speed train scenario at 30GHz. The configuration with two additional symbols suffer a small performance loss due to extra additional symbols compared to the one-symbol configuration. It is clear that the number of additional symbols should be configurable, being adaptable for different use cases. From the above result, we make the following proposal:

Proposal 1: Support configurable number of additional DMRS

Evaluation results: DMRS density reduction: high speed train scenario

As described in the past contribution [2], configurable density for DMRS is useful to improve the spectral efficiency of the system. In addition, benefits of having configurable DMRS density per RB are described in [3].

In this section, link level evaluation results are presented to demonstrate the performance of the proposed reduced density DMRS design. The DMRS pattern considered in the simulation is shown in Figure 4. In Figure 5, throughput performance at 30GHz in the high speed train scenario for different values of DMRS density in the high speed train CDL-D model are shown. The simulation parameters are shown in Table 1. In the figures, “d” indicates the density of DMRS. Two-bit FD-OCC was used across adjacent subcarriers to multiplex two layers and the pattern was repeated four times in the time domain in a 14-symbol slot. In the evaluation, channel estimates are averaged over time and frequency in one resource block. Furthermore, channel estimates are averaged across the resource blocks used in the simulation, i.e., 100 resource blocks. As shown in our previous contribution, the interpolation method does not have a big impact on the performance. Closed-loop precoding is used in the evaluation

It is clear from Figure 5 that due to nonselective channel characteristic in the high speed train scenario, spectrum efficiency gain can be obtained by using the combed DMRS pattern. The spectral efficiency gain for 14-symbol slot can be computed as follows:

spectral efficiency gain=1-16168×d1-16168

It should be noted that the above gain can be obtained in an ideal condition. As mentioned in the companion contribution, effectiveness of the proposed method relies on frequency flatness of the channel. Thus, in practice, the above spectral efficiency gain may not be achievable due to frequency selectivity of the channel.

Observation 1: With the reduced density DMRS configuration, spectrum efficiency gain can be obtained in the high speed train scenario

Figure 4 Reduced density DMRS configuration for 14-symbol slot, d=1/2

Figure 5 Throughput performance of reduced density DMRS, high speed train scenario at 30GHz, 500km/hr, K factor=7 dB, 60kHz subcarrier spacing

Figure 6 Throughput performance of reduced density DMRS, high speed train scenario at 30GHz, 500km/hr, K factor=7 dB, 120kHz subcarrier spacing

Figure 7 Throughput performance of reduced density DMRS, high speed train scenario at 30GHz, 500km/hr, K factor=7 dB, 480kHz subcarrier spacing

In addition, performance of the reduced density configuration is evaluated with subcarrier spacing values of 120kHz and 480kHz. The subcarrier spacing for 120kHz and 480kHz are shown in Figure 6 and Figure 7, respectively. It is clear from the figures that performance gain in spectral efficiency can be observed when d=1/10 or d=1/50. When subcarrier spacing is high, i.e., 480kHz, performance gain obtained by using d=1/10 is reduced due to frequency selectivity.

Evaluation results: DMRS density reduction: system level simulation results

In this contribution, system level simulation was conducted to assess the performance of the proposed DMRS configuration for MU-MIMO. The simulation assumption is shown in Table 2. In the scenario, throughput performance was investigated. In the evaluation, probability of collision between data from the serving cell and data transmitted from the adjacent cell was controlled to evaluate the performance of the proposed DMRS configuration. The simulation was conducted at 30GHz, and 12 layer-multiplexing with 2 layers assigned to each UE is considered.

Power boosting of 6dB is assumed for DMRS. In addition, as shown in Figure 8, DMRS patterns are configured such that RBs with DMRS are staggered between neighbouring cells to reduce inter-cell interference for DMRS. In the evaluation, configuration type 2 is implemented and mapping with four-bit OCC to multiplex 4 layers as shown in Figure 9 is assumed.

Figure 8 Staggered pattern for DMRS between adjacent cells, an example using DMRS configuration type 2

In Figure 10, throughput performances of the proposed and conventional DMRS configuration at 0% collision probability are shown. From the result, the proposed configuration yields inferior throughput performance compared to the conventional configuration due to estimated channel using interpolation. Note that since there is no intercell interference, better channel estimation can be obtained by having DMRS in all RBs.

Figure 9 Port mapping for configuration type 2

Figure 10 Throughput performance, 0% collision probability, 30GHz

However, when the collision probability is increased, effectiveness of the proposed method is clear. In Figure 11 and Figure 12, the throughput performances of the proposed and conventional DMRS configurations are shown when collision probability is set at 25% and 75%, respectively. Due to more frequent collision, effectiveness of power boosting for DMRS becomes more evident, as shown in Figure 11 and Figure 12.

SLS evaluations are also conducted at 4GHz. The results are shown in Figure 13, Figure 14 and Figure 15. It is clear from the figures that for collision probability greater than zero, the proposed DMRS configurations yield throughput performance gain.

Figure 11 Throughput performance, 25% collision probability, 30GHz

Figure 12 Throughput performance, 75% collision probability, 30GHz

Figure 13 Throughput performance, 0% collision probability, 4GHz, 30km/hr

Figure 14 Throughput performance, 25% collision probability, 4GHz, 30km/hr

Figure 15 Throughput performance, 75% collision probability, 4GHz, 30km/hr

Based on the evaluation results presented in Section 3 and 4, we make the following proposal:

Proposal 2: Support configurable DMRS density in the frequency domain

Proposal 3: Support density reduced DMRS configurations with density d RE/RB/port

Conclusion

The following observation and proposals are made based on the evaluation results shown in this contribution.

Observation 1: With the reduced density DMRS configuration, spectrum efficiency gain can be obtained in the high speed train scenario

Proposal 1: Support configurable number of additional DMRS

Proposal 2: Support configurable DMRS density in the frequency domain

Proposal 3: Support density reduced DMRS configurations with density d RE/RB/port

- References

- [1]	Draft Minutes Report of 3GPP TSG RAN WG1 AH#2, Qingdao, P.R. China, June, 2017

- [2]	R1-1712751, “Views on DL DMRS designs,” Mitsubishi Electric, Prague, Czech Republic Aug., 2017.

- [3]	R1-1710612, “Discussion on downlink DMRS design,” ETRI, Qingdao, P.R. China, June, 2017.

- [4]	R1-168174, “WF on CDL-D model for high speed train scenario at 30GHz for RRH and relay,” Mitsubishi Electric, ETRI, Ericsson, 3GPP RAN1#86, Göteborg, Sweden, Aug., 2016

- Appendix

Table 1 Evaluation assumptions for reduced DMRS density

| FEC, modulation | Turbo code, 256QAM, 64QAM, 16QAM, QPSK |
| --- | --- |
| Channel estimation | OFDM: Estimated with RS |
| Channel model | CDL-D shown in [4], (ASD, ASA, ZSA, ZSD)=(5, 1, 15, 5), K-factor=7.0dB |
| Center frequency, simulation BW, carrier separation | 30GHz, {80MHz, 60kHz }, {160MHz, 120kHz }, {640MHz, 480kHz } |
| Relay and RRH antenna elements | 256 |
| Antenna architecture for RRH and relay | (8,16,2,1,1) |
| Moving direction of the train | Moving at direction of ϕ=0o, θ=90o |
| RRH antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max= 8 dBi |
| Relay antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max=8 dBi |
| Phase noise model | BS model in R1-164041 |
| AFC | CP is used for AFC for OFDM |
| Interpolation | Time and frequency domain averaging |
| Precoding | Closed loop, Estimates from DMRS are interpolated and averaged over all RBs : one precoder is used for all RBs |
| Number of data symbols, IDFT | 1200, 2048 samples |
| CP size for OFDM and DFT-s-OFDM | 144 samples |
| Maximum speed | 500 km/h |
| Number of OFDM symbols | 14 |

Table 2 SLS parameters for MU-MIMO evaluation

| Parameters | Dense urban |
| --- | --- |
| Mobility | 3km/hr for 30GHz, 30km/hr for 4GHz |
| Layout | Single layer: Macro layer: Hex. Grid |
| Inter-BS distance | Macro layer: 100m (30GHz), 200m (4GHz) |
| Carrier frequency | 30GHz, 4GHz |
| Simulation bandwidth | 80MHz |
| Channel model | 3D Uma NLOS (4GHz), Umi street canyon LOS (30GHz) |
| BS Tx power | 44dBm (4GHz), 33dBm (30GHz) |
| BS antenna height | 25m (4GHz), 10m (30GHz) |
| BS antenna | (M,N,P,Mg,Ng) = (8,32,2,1,1) for 30GHz (M,N,P,Mg,Ng) = (1,8,2,1,1) for 4GHz (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | (M,N,P,Mg,Ng) = (2,4,2,1,2) for 30GHz, dH=0.5 (M,N,P,Mg,Ng) = (1,2,2,1,1) for 4GHz, dH=0.5 |
| UE antenna gain | Follow the modelling of TR36.873 |
| UE receiver noise figure | 9dB |
| UE antenna height | Follow TR36.873 |
| Traffic model | Full Buffer model |
| UE receiver | MMSE-IRC (12 orthogonal DMRS ports) |
| Feedback assumption | Ideal |
| Channel estimation | Real estimation, least square estimation |