3GPP TSG RAN WG1 Meeting #90                                           	R1-1712761

Prague, Czechia, 21th – 25th August 2017

Agenda item:	5.1.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on search space monitoring after search space reconfiguration or change

Document for:	Discussion/Decision

# Background

In RAN1#89 the following discussion regarding search space monitoring after search space change took place:

R1-1708748	Discussion on search space monitoring after search space change.	Qualcomm Incorporated

Conclusion:

- It is RAN1’s understanding that the UE is not required to monitor candidates that start before m+45 belonging to a new configuration (believed to be already captured in 36.331 s.11.2)

- There is currently not consensus on whether the UE is required to monitor candidates which start at or after m+45 belonging to a new configuration in a search space that starts before m+45

  - Discuss offline and revisit on Thursday to see whether consensus can be reached.

Thursday

On whether the current specifications require a UE to monitor candidates which start at or after m+45 belonging to a new configuration in a search space that starts before m+45:

- yes: Huawei, HiSilicon, Ericsson, Nokia, LG Electronics, Neul

- no: Qualcomm, Sony, Sierra, Intel

Conclusion:

Send an LS to RAN2 (R1-1709715, Alberto), stating that there are different views in RAN1 as to whether the current specifications require a UE to monitor candidates which start at or after m+45 belonging to a new configuration in a search space that starts before m+45, and requesting RAN2’s confirmation of the required UE behaviour according to the RAN2 specifications.

Also include the conclusion re. candidates starting before m+45.

Decision: Final LS is approved n R1-1709716.

# Outcome of RAN2 discussion and further discussion in RAN1

The LS was discussed in RAN2, and a response is available in R2-1706195 with the following conclusion:

RAN2 confirms that, based on RAN2 requirements, after a (re)configuration via RRC signalling,   the UE is required to receive the UL grant from the start of a search space  after the processing delay has passed, i.e. the UE is required to monitor the NPDCCH from the first NPDCCH search space which starts at or after the 45ms processing delay.

Note that the topic originally discussed in RAN1 included the general notion of “search space change”, but then the discussion was focused on the particular case of “search space change” after RRC reconfiguration. To complement the RAN2 agreement, and following the same technical principles of simplicity and ease of testing, RAN1 should confirm that the UE shall only monitor “entire search spaces” after a search space change. In view of this, we make the following proposal which accounts for all the search space changes.

Proposal: The UE is not required to monitor partial search spaces after a search space change. The search space change includes the case of a search space changing because of a reconfiguration (e.g. after RRC Reconfiguration) and the case of switching between different search spaces (e.g. monitoring of CSS after PDCCH order).