3GPP TSG RAN WG1 Meeting #90	R1-1713751

Prague, Czech Republic, 21 – 25 August 2017

Agenda Item:	6.1.3.3.8

Source:	Huawei, HiSilicon

Title:	On supporting ultra-reliability in a resource efficient way

Document for:	Discussion and decision

Introduction

The WI on New Radio (NR) Access Technology was approved at RAN #75. The objective of this WI is to specify NR functionalities for eMBB and URLLC. As for URLLC, the target is to meet the performance requirements on latency and reliability set forth by [1], where the discussion on reliability started from last meeting. During last meeting, there were some discussions on reliability for URLLC, including both data channel and control channel but no agreements were reached. In this contribution, we provide our views on DL data channel reliability for URLLC, especially focusing on importance of supporting ultra-reliability in a resource efficient way.

Discussion

Repetition transmission

To meet the RAN requirement on latency and reliability, one straightforward way is to reply on repetitions as discussed in [3]. Compared to the traditional stop-and-wait HARQ transmission scheme, the high reliability and low latency can be achieved by scheduling several repetitions before ACK/NACK feedback. It is obvious that the number of repetitions is the key parameter for overall performance. According to analysis in [2], the code rate needed to achieve 10%BLER is about 1.5 times higher than the code rate with 1e-5 BLER target under same SINR. This implies that a packet selecting a MCS corresponding to 10% BLER could achieve 1e-5 BLER with 2 repetitions. However, this is only feasible if MCS can be selected properly which may not be possible in practice. Considering the fluctuations between CQI measurement occasion and data transmission occasion due to fast-fading and interference variation, the SINR during transmission duration could be lower than the reported CQI, so we may need to assign more resource for URLLC with 1e-5 BLER, e.g. increase the number of repetitions.

The system-level performance for two schemes with different number of repetitions L is provided in Table 1 and the detailed UE BLER distribution is shown in Figure 1.For simplicity, the BLER target is assumed to 1e-3 but similar results are expected for 1e-5 BLER target. Two schemes are compared: (1): L=2; (2): L=4. It can be seen that the ratio of users satisfying the target BLER is 57.14% for L=2. The BLER target cannot be guaranteed from system point of view due to the CQI reporting delay such that the when SINR during data transmission is lower than reported CQI, the data decoding performance will be degraded, i.e. the BLER target cannot be guaranteed due to the impairment in link adaptation. One simple way to enhance the reliability is to increase the number of repetitions, e.g. L=4. One may expect that the reliability can be guaranteed at the cost of more resources. However, as shown in Table 1, the ratio of user satisfying target the BLER drops to 0 for L=4, i.e.  all UE’s BLER are larger than target BLER, as shown in Figure 1. The underlying reason is that the increasing in the number of repetitions leads to serious congestions in the system. With the increasing number of repetitions, more resources will be allocated to each packet transmission which results in the increasing of overall queuing delay. In some cases, this directly leads to packet overdue, and in some other case, the time left for repetition is not enough to ensure the reliability requirement.

Table 1 System performance for two repetition schemes: L=2 vs. L=4

| Repetition number | Proportion of UEs UE satisfying target BLER of 0.1% |
| --- | --- |
| L=2 | 57.14% |
| L=4 | 0 |

Figure 1 UE BLER distribution for two schemes: L=2 vs. L=4

In order to analyze the impact from congestion, the ratio of packet transmission failure due to congestion is studied and a new metric, BLERc is defined in the following:

BLERc=Packets Tx failure due to overdue+Packet Tx failure due to inadequate resourceTotal number of packets

The distribution of BLERc is shown in Figure 2. It can be seen that for L=4, the BLERc for all UEs are larger than the target BLER, which implies that congestion becomes the major cause of packet transmission failure for L=4. In contrast, the packet transmission failure due to congestion is much lower for L=2.

Figure 2 UE BLERc distributions for two schemes: L=2 vs. L=4

Observation 1: A moderate number of repetitions for the same TB is not sufficient to ensure the required reliability due to the SINR differences between CQI measurement and data transmission.

Observation 2: An excessive number of repetitions for the same TB leads to low resource efficiency and serious congestion from system perspective.

Observation 3: The support of ultra-reliability shall not compromise resource efficiency at the cost of excessive repetitions.

Efficient repetition schemes

In this section, three schemes with improved resource efficiency are discussed.

Scheme 1: Dynamic repetition configuration

Different repetition patterns can be considered to allow some scheduling flexibility to handle different channel condition and delay budget, e.g. as shown in Figure2, An interlaced repetition pattern can be used when the delay budget is sufficient. While a contiguous repetition pattern can be used when the delay budget is insufficient. A larger repetition number can be used for low SINR while smaller repetition number can be used for moderate SINR.A combination of higher-layer and physical layer signaling can be considered to enable dynamic selection of repetition patterns.

Figure 2 Dynamic repetition pattern: Packet 1 with small delay budget and packet 2 with large delay budget

Scheme 2: Early termination

This scheme is proposed for UL grant free transmissions and can be applied for DL transmissions as well. Early termination could avoid redundant transmission to some extent. This scheme can work jointly with dynamic repetition scheme (scheme1), the redundant transmission could be reduced. As shown in Figure 3, both interlaced repetition and early termination are adopted, in which the third and forth transmission are redundant, early termination could avoid the forth transmission

Figure 3  Interlaced repetition with early termination feedback. ACK feedback delay = 3 TTI for example

Scheme 3: Adaptive repetition based on Low latency- CSI (LL-CSI)

The transmit power, MCS and/or resource allocation for the subsequent repetitions for the same TB can be dynamically adjusted if LL-CSI can be available at the gNB, as shown in Figure 4. Compared to the basic repetition scheme, this could further improve both the ratio of user satisfying reliability requirement and spectrum efficiency significantly. Some system performance is shown in [3].

Figure 4 Adaptive repetition based on LL-CSI

Proposal: The following schemes are supported to improve the resource efficiency in addition to the data repetitions

- Dynamic repetition configuration

- Early termination by ACK

- Adaptive repetition based on Low latency- CQI (LL-CQI)

Conclusion

In this contribution, we provide our view on the data channel design aspects for URLLC and have the following observation and proposals

Observation 1: A moderate number of repetitions for the same TB is not sufficient to ensure the required reliability due to the SINR differences between CQI measurement and data transmission.

Observation 2: An excessive number of repetitions for the same TB leads to low resource efficiency and serious congestion from system perspective.

Observation 3: The support of ultra-reliability shall not compromise resource efficiency at the cost of excessive repetitions.

Proposal: The following schemes are supported to improve the resource efficiency in addition to the data repetitions

- Dynamic repetition configuration

- Early termination by ACK

- Adaptive repetition based on Low latency- CQI (LL-CQI)

- References

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- R1-1713749, “CSI reporting for high reliability in URLLC”, Huawei, HiSilicon

- R1-1712214, “PDSCH reliability for URLLC”, Huawei, HiSilicon

- Appendix

Table A1 Simulation assumptions

| Parameters | Description |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 60kHz |
| Simulation bandwidth | 20 MHz |
| Channel model | 3D UMa |
| BS Tx power | 46 dBm |
| BS antenna configurations | 2TX |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8dBi |
| UE antenna configurations | 2RX |
| UE antenna height | Follow the modelling of TR 36.873 |
| UE antenna gain | Follow the modelling of TR 36.873 |
| UE receiver noise figure | 9 dB |
| CQI reporting period | 20ms |
| Traffic model | URLLC: FTP Model 3 with MAC packet size 32bytes |
| Traffic load (Resource utilization) | λ=100 |
| UE distribution | 20% Outdoor in cars: 30 km/h, 80% Indoor: 3 km/h URLLC: 10 UE/sector |
| UE receiver | MMSE-IRC |