3GPP TSG-RAN WG2 # 101	Tdoc R2-1802365

Athens, Greece, 26th Feb – 2nd Mar, 2018	(Revision of R2-1800423)

Agenda Item:	10.4.1.3.3

Source:	Ericsson

Title:	I-RNTI discussion

Document for:	Discussion

# Introduction

This contribution is an update of a previous paper addressing UE Context ID. In this revision, focus is on I-RNTI length and security aspects.

# Discussion

## I-RNTI Size

As in LTE, for the resumeID, the length of the I-RNTI is a result from compromising unique numbers with feasible length to be transmitted in msg3, e.g., in an rrcConnectionResumeRequest message. Another requirement is that when an I-RNTI is detected by an NG-RAN node, it should be possible to identify the node holding the context.

In LTE release 13, the size of the resumeID in LTE is 40 bits, allowing for over 1 trillion unique Resume IDs. This seems like more than enough, and from a pure bit-length perspective, it seems reasonable to propose the I-RNTI for LTE connected to 5GC and NR to also be a 40-bit field. This would make it particularly easy from a mgs3-grant perspective to connect LTE cells to 5GC, in that the same grant as for Release 13 can be used.

To identify the node holding the context, it should be possible to identify where the context is located e.g. via identifiers such as the gNB ID for NR cells or ng-eNB ID for LTE cells connected to 5GC.

For NR, the gNB ID is between 22-32 bits [1]

For LTE connected to 5GC, the ng-eNB is either 18, 20 or 21 bits [2]

For NR, the gNB ID is between 22-32 bits [1]

For LTE connected to 5GC, the ng-eNB is either 18, 20 or 21 bits [2]

In RAN3, it has been agreed that:

From this, we conclude that, given the maximum number of bits spent for a gNB ID, (irrespective of if it is a cleartext gNB ID, or only a representation with equal bits) if the I-RNTI is 40 bits long, only 8 bits remain for UE-specific identities in such a gNB, to identify the UE within a given node hosting the context.

With a maximum length gNB ID representation of 32 bits, it seems that a 40-bit I-RNTI is too small.

To allow for an equal amount of unique UE specific identities (resume identities within a node) as for LTE, when the UE part is 20 bits, it would be necessary to increase the resume ID for NR with the increase in number of bits of gNB ID compared to an ng-eNB/eNB. Thus a 32-bit gNB representation and an addition of 20 bits gives an I-RNTI length of 52 bits.

A 52 bit I-RNTI should be adopted when UE context is stored in gNB to provide same space for UE specific identifiers as for eNB.

If such an I-RNTI should be sent in an rrcConnectionResumeRequest in LTE, this is currently not possible to fit in a regular msg3 grant. One option would be to solve this in a similar manner as for LTE, i.e., to use truncated version of I-RNTI.

When representing a 52-bit I-RNTI over LTE connected to 5GC, a truncated version of a full-size I-RNTI can be used.

Another option would be to solve the msg3 limitation for LTE with a message 3.5, as is described in [3].

When the I-RNTI should be sent in an rrcConnectionResumeRequest, msg3-grant over NR, it should be possible to either send as a full-size I-RNTI, or truncated.

## I-RNTI Security aspects

As the I-RNTI is meant to be included in MSG.3, it is going to be sent in clear-text over the air i.e. it can be read by anyone. Since this is the case, it is good if the I-RNTI does not reveal sensitive information that should usually be sent encrypted or not sent at all. One such piece of information that may be revealed would be a UE’s mobility pattern. Since the I-RNTI need to somehow reflect the gNB / ng-eNB ID of the node storing the context, it would be possible to track UE’s only by reading the I-RNTI. One way to make this aspect more difficult is to avoid in the specifications a clear bit string split between gNB/ng-eNB ID part and the UE ID part of an I-RNTI. That should not be problematic as the gNB ID information is in fact not needed by the UE’s at all.

The UE should treat the I-RNTI as a random value. Any splitting of the identifier in UE context and RAN node ID part should be transparent or unknown to the UE.

Given that it is expected that the I-RNTI will be frequently used in NR it would be good to have a solution where the gNB ID is not easily derivable from the UE AS Context ID. It is proposed to initiate a discussion with RAN3 and SA3 on possible ways to hide node ID in the I-RNTI

Discuss with RAN3 and SA3 how to best hide RAN node ID in I-RNTI.

# Conclusion

In section 2 we made the following observations:

Observation 1	With a maximum length gNB ID representation of 32 bits, it seems that a 40-bit I-RNTI is too small.

Based on the discussion in section 2 we propose the following:

Proposal 1	A 52 bit I-RNTI should be adopted when UE context is stored in gNB to provide same space for UE specific identifiers as for eNB.

Proposal 2	When representing a 52-bit I-RNTI over LTE connected to 5GC, a truncated version of a full-size I-RNTI can be used.

Proposal 3	The UE should treat the I-RNTI as a random value. Any splitting of the identifier in UE context and RAN node ID part should be transparent or unknown to the UE.

Proposal 4	Discuss with RAN3 and SA3 how to best hide RAN node ID in I-RNTI.

# References

R2-1712130, LS on Length of NR Cell Identity, RAN2 #100, Reno, Nevada, USA, 27th Nov – 1st Dec 2017.

3GPP TS 38.413, NG Application Protocol (NGAP) Release 15, v 0.4.0, 2017-10

R2-1802632, Message 3.5 in LTE connected to 5GC, Ericsson