3GPP TSG-RAN WG2 Meeting #101			     R2- 1802993

Athens, Greece, February 26 – March 2, 2018

Agenda item:	10.3.1.8 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	TP on Restart condition of sCellDeactivationTimer for configured UL grant and configured DL assignment

Document for:	Discussion and Decision

Text Proposal

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in subclause 6.1.3.10;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry.

The MAC entity shall for each configured SCell:

1>	if an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including:

3>	SRS transmissions on the SCell;

3>	CQI/PMI/RI/CRI reporting for the SCell;

3>	PDCCH monitoring on the SCell;

3>	PDCCH monitoring for the SCell;

3>	PUCCH transmissions on the SCell, if configured.

2>	start or restart the sCellDeactivationTimer associated with the SCell in the slot when the SCell Activation/Deactivation MAC CE was received;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2;

2>	trigger PHR according to subclause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bandwidthPartInactivityTimer associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if uplink grant or downlink assignment is configured on the activated SCell:

2>	if there is a MAC PDU transmitted or received on the uplink grant or the downlink assignment:

3>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CQI/PMI/RI/CRI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

End of Text Proposal