3GPP TSG-RAN WG2 #101	Tdoc R2-1803159

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.3.3.5

Source:	Ericsson

Title:	PDCP duplication

Document for:	Discussion, Decision

# Introduction

In this contribution, remaining aspects of PDCP duplication is discussed in relation to the transmit operation and discarding of packets. Furthermore, the FFS on duplication support for SRBs in CA is discussed. In RAN2#AH1801 the following agreements were made:

Agreements:

- Upon packet duplication activation, only PDCP SDUs/PDUs not submitted to lower layers are duplicated.

- Baseline is that packed duplication is support for data PDUs

- For packet duplication, when to submit PDCP PDUs to lower layers is up to UE implementation.  FFS on UE behaviour when duplication is deactivated and what PDCP data volume is used.

- After packet duplication is activated, for DC duplication, PDCP data volume is indicated to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the secondary RLC entity

- After packet duplication is activated, for CA duplication, PDCP data volume is included in both the LCG associated with the primary RLC entity and the LCG associated with the secondary RLC entity.

- Packet duplication does not impact RLC data volume

- The UE shall discard packets that have been acknowledged by RLC in the other RLC leg.   PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.  RLC procedures and PDCP discard procedures are not impacted by this agreement.

- The deactivated RLC entity is not re-established

- For CA and DC upon deactivation of PDCP data duplication, the UE transmitting PDCP entity should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity

- When configuring duplication, RRC can also set the initial state (active or inactive) for DRBs.

- If SRB is configured to use duplication, the state is always active

- FFS Duplication is supported for SRBs for CA

# Discussion

## Support for SRB duplication in CA

For SRBs we believe the support of duplication with CA provides a viable method to enable high reliability services in non-DC architectures, i.e. standalone NR. Therefore, it should not be excluded.

From a protocol architecture perspective, there are no new aspects of SRB duplication in CA. Introducing SRB duplication in CA would be merely an alignment with DRB duplication in CA. Thus, there are no particular issues or complexity to support CA duplication also for SRBs.

The alignment of duplication aspects of the SRBs and DRBs is possible without increasing the complexity.

Further, supporting CA duplication for SRBs does not require any modification on the RRC specifications due to the fact that, at the moment, the logical channel configuration (and thus the ID) and association with the SRBs and DRBs is done in the CellGroupConfig information element. In particular, within the RLC-Bearer-Config, a logical channel ID (or more) may be mapped to SRBs whose identity is fixed in the RRC specification (i.e., value 1 for SRB1, value 2 for SRB2, and value 3 for SRB3).

RLC-Bearer-Config ::=								SEQUENCE {

-- ID used commonly for the MAC logical channel and for the RLC bearer.

logicalChannelIdentity						LogicalChannelIdentity,

-- Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this

-- RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the

-- uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling

-- restrictions ('moreThanOneRLC' in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

servedRadioBearer							CHOICE {

srb-Identity                           SRB-Identity,

drb-Identity                           DRB-Identity

}																											OPTIONAL,	-- Cond LCH-SetupOnly

reestablishRLC								ENUMERATED {true}												OPTIONAL, 	-- Need N

rlc-Config									RLC-Config														OPTIONAL,	-- Cond LCH-Setup

mac-LogicalChannelConfig					LogicalChannelConfig											OPTIONAL	-- Cond LCH-Setup

}

LogicalChannelIdentity ::= 					INTEGER (1..maxLC-ID)

Current RRC specification support the assignment of one or more logical channel ID for a single SRBs when CA duplication is activated.

For these reasons, the support of CA duplication for SRBs come without any additional complexity or modifications in the RRC specifications.

Compared to the support of SRB duplication in DC, there are also no further requirements on the UP, when duplicating SRBs in CA. I.e. procedures on PDCP for duplicate transmission and duplicate discarding as well as data availability reporting are generic and applicable to SRBs likewise as to DRBs for both DC and CA.

Support of SRB duplication in CA does not have any new requirements for UP.

Furthermore, radio link monitoring and radio link failure triggering for SRBs in duplication in CA can readily reuse the SCell failure indication upon maximum number of RLC retransmissions reached, as specified.

In architectures in NR standalone, where URLLC should be supported, SRB duplication provides the required reliability e.g. to recover from outages on one carrier. With SRB duplication, handover related signalling is improved in terms of achieving a higher reliability of conveying a message within a certain latency bound.

Both DL and UL CA duplication should be discussed. For DL CA duplication, same as for DRB, establishing of a secondary RLC entity for the SRB should needs to be specified. With DL CA duplication, in a non-DC architecture for example, the critical handover command can be conveyed reliably to the UE. With UL CA duplication, additionally, e.g. measurement reports required to initiate a handover command can be done in a more reliable way.

Example: due to temporary outage on a carrier, e.g. a handover command cannot be successfully transmitted to the UE within a certain latency bound, i.e. due to (sudden) low SINR, it requires both segmentation and several retransmissions until it is successfully received in the UE in full. If the handover command is received too late, the UE might have gone out of coverage even more, making it more difficult for each retransmission of the handover command to be received. Due to frequency diversity, transmission of the handover command on another carrier is likely to be uncorrelated, i.e. may not be subject to retransmission delays. In this case receiving the handover command via the second carrier, where it is duplicated, decreases the latency until the command is received.

Given the expected benefits of duplication with CA, that it is already supported by UP procedures, and the minimal impact to CP to configure it, we propose:

Duplication in CA is supported for SRBs.

## Transmit operation

In the current Rel-15 NR TS 38.323, the PDCP duplication function is captured in the transmitter side as follows:

When submitting a PDCP Data PDU to lower layer, the transmitting PDCP entity shall:

[…]

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if pdcpDuplication is configured and activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities.

-	else, if pdcpDuplication is configured but not activated:

-	submit the PDCP Data PDU to the primary RLC entity.

This way, for each PDCP SDU, the corresponding PDCP Data PDU is duplicated and provided to each RLC entity – irrespective of transmission opportunities and their ratio on the transmission legs. For the split bearer operation, the following NOTE was added:

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

indicating that submission of PDCP PDUs to lower layers before transmission opportunity is indicated should be kept minimal by UE implementations. This way, low skew times below the UL transmission legs are ensured, since data transmitted to lower layers is more according to the UL grant ratio. This way, for split operation, eventually lower reordering times in the receiving PDCP entity are ensured.

For duplication operation, similar considerations must be done: instead of considering minimizing the reordering time among the transmission legs, the reception time differences of the duplicates must be considered, since PDCP data duplication can only be considered useful for e.g. URLLC traffic if the reception time differences are not too high, i.e. the duplicate is not already outdated when received too late after the original.

Furthermore, for PDCP data duplication, pushing down all the data immediately to both RLCs may lead to this data being stuck in an RLC for which no grant will arrive (for some time). This will lead to the further risk of duplicate transmissions drifting apart, due to different amounts of granted resources on the links.

In RAN2 AH#1801, it had been agreed that for packet duplication, when to submit PDCP PDUs to lower layers is up to UE implementation.  Same as for split bearer operation we nevertheless need to specify a NOTE limiting potentially bad UE behaviour associated with this, as described above. Thus, we propose:

- In duplication, the UE transmitter minimizes the amount of PDCP PDUs submitted to lower layers before receiving a request from lower layers to minimize the reception time difference of the duplicates in the receiving PDCP entity.

## Retransmission and duplication

Beside duplicate transmission, also duplicate retransmission should be considered. PDCP specifies two retransmission procedures, i.e. PDCP re-establishment and PDCP data recovery. PDCP duplication may be kept activated when undergoing these procedures, i.e. duplication should be applied before and after the procedure. To maintain the intended reliability with duplication, thus duplication should also be applied to retransmissions involved in these procedures. It is noteworthy that due to release/re-establishment of lower layers involved in these procedures, the PDCP “re-transmissions” may actually be the first data transmissions of the corresponding PDUs, which emphasizes further that PDCP data duplication should be applied for those PDUs in order to maintain the reliability requirements.

- At PDCP re-establishment, for AM DRBs, when retransmitting PDCP SDUs, if PDCP duplication is configured and activated, duplicated PDCP PDUs of these SDUs are submitted to both associated RLC entities.

- At PDCP data recovery, for AM DRBs, when retransmitting PDCP PDUs, if PDCP duplication is configured and activated, duplicated PDCP PDUs are submitted to both associated RLC entities.

## Control PDU transmission

Regarding PDCP Control PDUs, it had been agreed that as a baseline, duplication of the control PDU is not supported when duplication is activated for data PDUs. It must be further clarified on which leg the Control PDU is submitted when two legs are available in duplication. We believe the simplest option is to define the primary leg as the sole leg where control PDUs are submitted, given that it can never be deactivated.

- Confirm that PDCP Control PDUs are not duplicated. Control PDUs are submitted only to the primary lower layer.

## On parameter name

Regarding the PDCP parameter “pdcpDuplication”, there is currently a discrepancy compared to the name of the RRC parameter “ul-duplication”. Furthermore, we should differentiate between whether duplication is configured according to the parameter, and/or activated, which does not relate to the parameter. It is noted that activation/deactivation may be done via RRC signalling or MAC control element. Therefore, we propose the following (also considered in text proposal below):

- Use formulation “if ul-duplication is configured and duplication is activated/deactivated” in PDCP specification.

## Remaining issues of PDU discard after successful delivery (AM)

In the current transmit operation, when duplication is configured and activated, when submitting a PDCP Data PDU, the PDU is duplicated and submitted to both associated RLC entities (TS 38.323, section 5.21). The transmission of the PDUs on the lower layers may be subject to different latencies resulting from e.g. RLC queueing, RLC retransmissions, RLC segmentation, HARQ retransmissions, different numerologies on the involved carriers, un-synchronized scheduling, etc. Therefore, when providing PDCP duplicates to the two associated lower layers, reception of the duplicates cannot be assumed on the same time. When the reception time difference becomes too large, reception of the later duplicate becomes redundant, thus transmission of the duplicate should be avoided in this case.

PDCP discarding when duplicating had been discussed at RAN2 AH#1801 with the above agreements, which are applicable mainly for AM operation. In this contribution we discuss the remaining open issues to solve also the UM operation.

According to the agreement, the UE shall discard packets that have been acknowledged by RLC in the other RLC leg.  And, PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.

We believe that this behaviour should be implemented as part of the PDCP discard procedure, which this way is made applicable to discard upon successful delivery of PDUs (beside SDUs). We believe that this goes beyond existing PDCP discard procedures, thus fulfils the agreement of “RLC procedures and PDCP discard procedures are not impacted by this agreement”. I.e. does not change the existing PDCP SDU discard behaviour.

Discarding of the PDCP PDU when successful delivery is indicated by RLC is specified as part of the PDCP discard procedure.

As discussed in [1], indicating successful delivery of the PDCP PDU by RLC may not confirm the reception of the PDCP PDU with absolute reliability since it may be discarded at the PDCP receiver in case of header decompression failure. It is stated in [1] that therefore PDCP SDUs should be kept for potential retransmission at handover later. However, not retransmitting PDCP PDUs that were only confirmed by RLC as successfully delivered, is the common PDCP behaviour according to the PDCP re-establishment procedure anyway. Therefore, indeed PDCP SDUs shall be discarded and not re-transmitted during re-established if their successfully behaviour had been confirmed by RLC only.

PDCP PDUs for which successful delivery is confirmed by RLC are not considered for retransmission, i.e. the corresponding PDCP SDU is discarded.

## PDU discard after time (UM)

Same as for AM operation, also in UM operation, due to submission of PDCP PDU duplicate to the two RLC entities at the same time, but their potentially different transmission times on these RLC entities, reception time differences of the duplicates may increase beyond a point where a duplicate reception is still useful. This case where duplicate transmissions among the RLC entities drift apart, must thus be avoided.

Example: for URLLC support, any queueing delay that a duplicate transmission might undergo, while the original transmission takes place immediately, destroys the purpose of introducing duplication in the first place: if a PDU would be pushed as original and duplicate to RLC1 and RLC2, transmitted via RLC1 but queuing up for some time in RLC2 (since previous duplicate packets were pushed to RLC2 as well) the duplicate PDU would be delayed and transmitted only after the RLC2 queueing time. Then, at the time of the reception, the PDU duplicate would be worthless.

For AM, successful transmission is indicated to PDCP, and from that point onwards, no duplicate of this PDU is required anymore. For UM, another mechanism needs to be established. Therefore, a PDCP PDU should be discarded for duplicate transmission if it could not be transmitted within a certain time after the original had been sent. The time should be configurable according to traffic latency targets. Since successful transmission is indicated in AM towards PDCP based on the RLC status report frequency, which may be configured conservatively for overhead avoidance, it may be useful to use the timer-based discard also in AM mode, leading to a quicker discard.

This can be achieved by a timer which can be implemented on PDCP or RLC. If implemented in RLC, the timer would relate to a maximum queueing time in the respective RLC entity. Otherwise, a duplicateDiscardTimer could be implemented on PDCP.

Both options would be different to the current PDCP SDU discard timer, since the PDCP SDU discard timer relates to a maximum queueing time before the first (original) transmission of the PDU, while the required duplicateDiscardTimer related to transmission time differences between a first transmitted PDU and the transmission of its duplicate.

As the duplication function is introduced on PDCP layer, we prefer a solution where a PDU duplicateDiscardTimer is introduced on PDCP, as implemented in the text proposal below

For DC and CA PDCP duplication, a PDCP PDU discard timer is started when a PDCP PDU is initially submitted to lower layers. At expiry of the timer:

  - PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

  - PDCP should indicate to lower layers to discard the corresponding PDCP PDU.

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	Duplication in CA is supported for SRBs.

Proposal 1	In duplication, the UE transmitter minimizes the amount of PDCP PDUs submitted to lower layers before receiving a request from lower layers to minimize the reception time difference of the duplicates in the receiving PDCP entity.

Proposal 2	At PDCP re-establishment, for AM DRBs, when retransmitting PDCP SDUs, if PDCP duplication is configured and activated, duplicated PDCP PDUs of these SDUs are submitted to both associated RLC entities.

Proposal 3	At PDCP data recovery, for AM DRBs, when retransmitting PDCP PDUs, if PDCP duplication is configured and activated, duplicated PDCP PDUs are submitted to both associated RLC entities.

Proposal 4	Confirm that PDCP Control PDUs are not duplicated. Control PDUs are submitted only to the primary lower layer.

Proposal 5	Use formulation “if ul-duplication is configured and duplication is activated/deactivated” in PDCP specification.

Proposal 6	Discarding of the PDCP PDU when successful delivery is indicated by RLC is specified as part of the PDCP discard procedure.

Proposal 7	PDCP PDUs for which successful delivery is confirmed by RLC are not considered for retransmission, i.e. the corresponding PDCP SDU is discarded.

Proposal 8	For DC and CA PDCP duplication, a PDCP PDU discard timer is started when a PDCP PDU is initially submitted to lower layers. At expiry of the timer:

a.	PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

b.	PDCP should indicate to lower layers to discard the corresponding PDCP PDU.

# References

R2-1801233, Issues on PDCP duplication, LG Electronics France, 3GPP RAN2#101, Athens, Greece, 26th February – 2nd March 2018

# Text proposals

The following text proposal is based on the current running CR for TS 38.323, as provided in R2-1801554.

Next Change

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	stop and reset the duplicateDiscardTimer for each PDCP Data PDU.

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer.

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	if ul-duplication is configured and duplication is activated:

-	duplicate the resulting PDCP Data PDU and submit it to both associated RLC entities and (re-) start the duplicateDiscardTimer associated with this PDCP Data PDU.

-	else:

-	submit the resulting PDCP Data PDU to lower layer.

[…]

Next Change

### 5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-Size];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity.

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if ul-duplication is configured and duplication is activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities.

-	start the duplicateDiscardTimer associated with this PDCP PDU (if configured).

-	else, if ul-duplication is configured but duplication is not activated:

-	submit the PDCP Data PDU to the primary RLC entity.

-	else:

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 36.322 [5]) in the two associated RLC entities is less than ul-DataSplitThreshold:

-	submit the PDCP PDU to the primary RLC entity.

-	else:

-	submit the PDCP PDU to either the primary RLC entity or the secondary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity, or for duplication to minimize the PDCP PDU reception time difference of duplicates in the receiving PDCP entity.

Next Change

## 5.3	Discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the duplicateDiscardTimer expires for a PDCP PDU, or the successful delivery of a PDCP PDU is confirmed by lower layers, the transmitting PDCP entity shall discard the PDCP Data PDU along with the corresponding PDCP SDU.

If the discarded PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

When ul-duplication is configured and duplication is deactivated, discard for all duplicated PDCP Data PDUs submitted to the secondary RLC entity is indicated to this secondary RLC entity.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

Next Change

## 5.5	Data recovery

For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the PDCP entity shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), transmit a PDCP status report as described in subclause 5.4.1;

-	perform retransmission of all the PDCP Data PDUs previously submitted to re-established or released AM RLC entity in ascending order of the associated COUNT values for which the successful delivery has not been confirmed by lower layers. If ul-duplication is configured and duplication is activated, duplicate the to be retransmitted PDCP Data PDUs and submit each duplicated PDU to the associated RLC entities and (re-) start the duplicateDiscardTimer associated with each of these PDCP Data PDUs

After performing the above procedures, the transmitting PDCP entity shall follow the procedures in subclause 5.2.1.

Next Change

- 7.3	Timers

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

b) duplicateDiscardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon submission of a duplicate PDCP Data PDU.

The receiving PDCP entity shall maintain the following timers:

c) t-Reordering

The duration of the timer is configured by upper layers TS 38.331 [3]. This timer is used to detect loss of PDCP Data PDUs as specified in the subclause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.