3GPP TSG-RAN WG2 #101	R2-1803328

Athens, Greece, 26th February – 2nd March 2018

Title:	Draft LS to RAN4 on A3 and A6 offsets in NR (E400)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson [to be changed to RAN2]

To:	RAN4

Contact Person:

#### Name:	Icaro Da Silva

E-mail Address:	icaro.leonardo.da.silva@ericsson.com

1. Overall Description

Similar to LTE, in NR events A1 and A6 are used to handle primary and secondary cell handovers. More precisely, the UE sends a measurement reports once the neighbor cell become amount of offset O1 better than PCell/PSCell (A3 event) or becomes amount of offset O2 better than SCell. These offsets are defined using the parameter MeasTriggerQuantityOffset in ReportConfigNR depending on the triggering quantity. However, the exact values for the offsets are not yet defined.

Since the reporting resolution for RSRP, RSRQ and SINR are the same as in LTE, it is reasonable to assume that the same offset values should also be used in NR. Therefore, we solicit RAN4’s view on this topic, i.e., to define the offset values between -15 to +15 dB with 0.5 dB resolution, as in LTE.

2. Actions:

ACTION: RAN2 respectfully asks RAN4 to consider the following proposals:

- Defining the same values for A3 and A6 offsets as in LTE, i.e, -15 dB to +15 dB with 0.5 dB resolution.

3. Date of Next RAN2 Meetings:

RAN2#101-Bis, April 16th – April 20th, 2018