3GPP TSG-RAN WG2 Meeting #101	     R2-1803557

Athens, Greece, 26th February - 2nd March 2018

Source: 	AT&T

Title:	Remaining details of CSI-RS for RRM configuration

Agenda Item:	10.4.1.4.1

Document for:	Discussion

# Introduction

During RAN2#100 the following agreements were made regarding NR measurement objects (MO):

Agreements

1	UE determines which MO corresponds to the serving cell frequency from the frequency location of the CD-SSB that is contained within the serving cell configuration.

2	More than one MO with  CSI-RS resources for measurement can be associated to the same SSB location in frequency. The SSB is at least used for timing reference.

3	In case that more than one MO with  CSI-RS resources for measurement is associated to the same SSB location in frequency the UE is indicated which MO corresponds to the serving carrier.

FFS whether the indication is in MO or serving cell configuration

Additionally during RAN2 NR Adhoc #3 the following agreements were made:

Agreements

1:	Within an MO, the SSB location is always indicated with GSCN with no additional offset.

FFS Whether the subcarrier offset is also required. To be checked with RAN1

2:	For MO with CSI-RS, an NR-ARFCN is used to indicate a frequency reference. Location of CSI-RS is relative to this reference

FFS Whether this reference is the same as point A in agreement 3i.

3:	For reconfiguration with sync (e.g. for inter-frequency HO and SCG change) and for configuration of SCells the SSB location is indicated with GSCN.

3i	In addition the reconfiguration provides an NR-ARFCN to identify point A.

FFS Whether the subcarrier offset is also required. To be checked with RAN1

4:	For idle/inactive reselection and reselection from LTE to NR, the SSB location is always indicated with GSCN with no additional offset.

This contribution discusses implications of above agreements specifically on the reaming details of CSI-RS for RRM configuration.

# Details of CSI-RS for RRM configuration

Based on the above agreements, NR MO may contain configuration for measurements based on either SS blocks or CSI-RS resources (or both) configured to the UE. The IE CSI-RS-ResourceConfig-Mobility is used to configure multiple parameters such as the CSI-RS bandwidth, frequency location (relative to a reference point), and associated SSB (if any).

In their LS on RRC parameters which are per-cell or per-BWP [1], RAN1 left FFS RRM related parameters such as CSI-RS-ResourceConfig-Mobility;

| Initial access/RACH/Mobility |  |  |
| --- | --- | --- |
| Group name | RAN1 parameter name | Guidance |
| RRM | RRM-MeasurementConfig SMTC-Config SMTC-Config-Idle SSB-MeasurementTimingConfiguration csi-rs-ResourceConfig-Mobility ReferenceSignalConfig SS-RSSI-MeasurementConfig SS-RSSI-MeasurementSlotConfig SS-RSSI-MeasurementSymbolConfig | FFS |

However, there are several reasons why per-BWP configuration is the more appropriate configuration option. First, since the UE may perform RRM measurements outside of its active BWP on a given carrier, it is critical for the UE to determine the CSI-RS frequency location and bandwidth relative to its different configured BWPs in order to determine whether regular intra-frequency or inter-frequency (e.g. with measurement gaps) requirements apply depending on whether the CSI-RS for mobility is inside or outside the active bandwidth part respectively.

In addition, it is possible for the network to configure the same CSI-RS resources for multiple purposes including CSI acquisition, beam management, and connected mode RRM measurements. In this case, since RAN1 agreed that CSI-RS for non-mobility purposes is per-BWP, it seems more natural and straightforward to apply the same framework for CSI-RS for mobility:

| General parameter |  |  |
| --- | --- | --- |
| RAN2 parameter name | RAN1 feature | Guidance |
|  | CA | Per UE |
|  | CSI/SRS/PTRS Resource Configuration | Per BWP |
|  | Configured UL grant | Per BWP |
|  | CSI & beam management framework | Per BWP |

Proposal 1: The NR MO should support a per-BWP configuration of CSI-RS resources for mobility to support RRM measurements inside or outside the active bandwidth part on a given carrier.

# Conclusion

In this contribution, we discussed implications of recent RAN1/RAN2 agreements on the measurement configuration of CSI-RS resources for mobility. The following proposal is offered for consideration:

Proposal 1: The NR MO should support a per-BWP configuration of CSI-RS resources for mobility to support RRM measurements inside or outside the active bandwidth part on a given carrier.

# References

[1] R1-1801281 LS on correction to RRC parameters for NR, RAN1