3GPP TSG-RAN WG2 #101	Tdoc R2-1803855

Athens, Greece, 26 Feb - 2 Mar, 2018

Title:	[DRAFT] LS on messagePowerOffsetGroupB

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN2)

To:	RAN1

Contact Person:

#### Name:	Mats Folke

Tel. Number:	+46761271385

E-mail Address:	mats.folke@ericsson.com

1. Overall Description:

In the Random Access procedure for LTE in 36.321, section 5.1.2, the MAC layer determines the selection of Random Access Preambles group B according to

-	if Random Access Preambles group B exists and any of the following events occur:

-	the potential message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is greater than messageSizeGroupA and the pathloss is less than PCMAX,c (of the Serving Cell performing the Random Access Procedure) – preambleInitialReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB;

-	the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC header is greater than messageSizeGroupA;

-	select the Random Access Preambles group B;

RAN2 has agreed to use the parameter messagePowerOffsetGroupB for NR. RAN2 thinks the values from LTE can be reused for NR and ask RAN1 to confirm.

messagePowerOffsetGroupB			ENUMERATED {minusinfinity, dB0, dB5, dB8, dB10, dB12,

dB15, dB18}

2. Actions:

To RAN1 group.

ACTION: 	RAN2 asks RAN1 to confirm the values for parameter messagePowerOffsetGroupB.

3. Date of Next RAN2 Meetings:

RAN2#101-Bis	16th – 20th April 2018	Sanya, CN

RAN2#102	21st – 25th May 2018	Busan, KR