3GPP TSG-RAN2 Meeting #102 	R2-1808533

# Busan, South Korea, 21st - 25th May 2018						 Resubmission of R2-1806066

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0229 | rev | 1 | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PDCP release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The PDCP entity has the PDCP SDUs stored in PDCP reordering buffer at the time of PDCP release. Discarding the stored PDCP SDUs at the PDCP release may degrade the service quality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCP release procedure is introduced for AM DRBs, UM DRBs, SRBs, and LWA bearers. For AM DRBs, UM DRBs and LWA bearers, in the transmitting side, stored PDCP SDUs and PDCP PDUs are discarded. In the receiving side, PDCP PDUs received from lower layer are processed, and stored PDCP SDUs are delivered to the upper layer. For SRBs, stored PDCP SDUs and PDCP PDUs are discarded in both transmitting side and receiving side. Impact analysis: Impacted functionality: PDCP release procedure for all beraers. Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCP release procedure is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS36.322 CR 0135 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

5.X	Release procedure

When upper layers request a PDCP entity release, the UE shall:

-	if it is the PDCP entity configured for the DRB mapped on RLC AM:

- 	process the PDCP Data PDUs that are received from lower layers due to the release of the lower layers, as specified in the subclause 5.1.2.1.2 in the receiving side;

-	deliver all stored PDCP SDUs, if any, to upper layers in ascending order of associated COUNT values in the receiving side;

-	discard all stored PDCP SDUs and PDCP PDUs in the transmitting side;

-	release the PDCP entity;

-	if it is the PDCP entity configured for the DRB mapped on RLC UM:

- 	process the PDCP Data PDUs that are received from lower layers due to the release of the lower layers, as specified in the subclause 5.1.2.1.3 in the receiving side;

-	deliver all stored PDCP SDUs, if any, to upper layers in ascending order of associated COUNT values in the receiving side;

-	discard all stored PDCP SDUs and PDCP PDUs in the transmitting side;

-	release the PDCP entity;

-	if it is the PDCP entity configured for the SRB:

-	discard the PDCP Data PDUs that are received from lower layers due to the release of the lower layer in the receiving side;

-	discard all stored PDCP SDUs and PDCP data PDUs in the transmitting side;

-	release the PDCP entity;

-	if it is the PDCP entity configured for the LWA bearer:

-	process the PDCP Data PDUs that are received from lower layers due to the release of the lower layers, as specified in the subclause 5.1.2.1.4 in the receiving side;

-	deliver all stored PDCP SDUs, if any, to upper layers in ascending order of associated COUNT values in the receiving side;

-	discard all stored PDCP SDUs and PDCP PDUs in the transmitting side;

-	release the PDCP entity.

End of changes