# 3GPP TSG RAN WG2 Meeting #103         			                           R2-1811080
Gothenburg, Sweden, 20th August - 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 00xx | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on missing SSB measurement information for blind HO and intra-NR redirection (RIL Q128, Q129, Q130) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4 LS (R4-1809547), it indicated some of the SSB-related assistance information are missing in current TS 36.331 spec and TS 38.331 spec. However, some of their ASN.1 suggestions seem to be wrong, e.g. adding missing SSB timing offset for NR PSCell addition in 38.331. So, we can’t directly implement the changes suggested by R4-1809547. As follows, we will list the impacted two procedures in TS 38.331, and discuss one by one respectively: - For RRC Release with redirection to an inter-frequency NR cell (i.e. Intra-NR redirection) - It is suggested to provide additional optional information for SCS of SSB and SMTC periodicity/offset/duration in RedirectedCarrierInfo to provide the needed information for UE to search for the SSB of target cell. - For NR SCell addition and (intra-NR) handover procedure - It is suggested to provide additional optional information for timing offset of SSB for the target cell based on the timing reference of current serving cell in ServingCellConfigCommon of to avoid UE blindly search for the SSB of target cell the whole SSB period. For 1), it is reasonable to add SCS of SSB and SMTC of redirected NR cell in RedirectedCarrierInfo, to avoid UE blind search of target cell. However, these missing IEs are indicated as “optional” in R4-1809547. it didn’t indicate when such IEs can be absent and the corresponding UE behaviour if such IEs are absent. So, we think the UE behaviour should be clarified when such IEs are absent. - SMTC of target cell: based on below RAN1 agreement in RAN1#88, we think UE will assume 5ms SMTC periodicity when this IE is absent RAN1#88 agreement If the network does not provide indication of SS burst set periodicity and information to derive measurement timing/duration the UE should assume 5 ms as the SS burst set periodicity - SCS of target cell SSB: RAN2 discussed similar issue in NR and agreed SCS is always indicated in handover and serving cell addition: subcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond HOAndServCellAdd Following same logic, SCS should also be mandatory in case of NR-NR redirection, to avoid UE blindly search SCS of target cell. For 2), first note that it seems to be wrongly suggested to add missing target cell SMTC info for NR PSCell addition in TS38.331. The missing SMTC info should come from eNB (e.g. calculated based on SFTD reporting before PSCell is configured). So, it needs to be added in LTE RRC as opposed to NR RRC carried in container. We provide another CR to TS 36.331 (R2-18011079) for it. Next, we discuss the remaining two subcases: - intra-NR handover and PSCell addition in future NR-NR DC: R4-1809547 suggested to add target cell SMTC info in ServingCellConfigCommon. However, we think this missing info should be added in ReconfigurationWithSync as opposed to ServingCellConfigCommon. That is because ServingCellConfigCommon is a cell specific IE and should be same as the one broadcast in SIB. However, the missing info (i.e. SMTC of target cell based on the timing reference of current serving cell) should be UE specific because it depends on both serving cell and target cell, i.e. different UEs may have different offsets. - NR SCell addition: with the same justification for intra-NR handover, we think the SMTC of target cell based on timing reference of current serving cell should be added in SCellConfig as opposed to ServingCellConfigCommon. Furthermore, such IE is also indicated as “optional” in R4-1809547. Following the same justification for intra-NR redirection, we think it is necessary to capture the statement that “if this IE is absent, UE will assume 5ms SMTC periodicity” in the field description of the IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In IE RedirectedCarrierInfo of RRCRelease message, modify the IE nr to a SEQUENCE structure including frequency, SCS of SSB and SMTC. - Frequency and SCS of target cell SSB are mandatory present. - SMTC of target cell SSB is optional IE as indicated in RAN4 LS. And add its field description that “This IE indicates the SMTC of the redirected target NR cell SSB based on the timing reference of current serving cell. If this IE is absent, UE will assume 5ms SMTC periodicity” - In IE ReconfigurationWithSync, add new IE smtc-targetNRcell to indicate the SMTC of the target cell SSB based on the timing reference of current serving cell - SMTC of target cell SSB is optional IE as indicated in RAN4 LS. And add its field description that “This IE indicates the SMTC of the target NR cell SSB based on the timing reference of current serving cell. If this IE is absent, UE will assume 5ms SMTC periodicity” - In IE SCellConfig, add new IE smtc-targetNRcell to indicate the SMTC of the target NR SCell SSB based on the timing reference of current serving cell - SMTC of target SCell SSB is optional IE as indicated in RAN4 LS. And add its field description that “This IE indicates the SMTC of the target NR cell SSB based on the timing reference of current serving cell. If this IE is absent, UE will assume 5ms SMTC periodicity” 1) and 2) apply to SA only. 3) applies to both NSA and SA. New IE is added after the extension marker. Inter-operability If either UE or Network is not implemented according to this CR, then UE has to blindly search target NR cell SSB in intra-NR redirection, intra-NR handover and NR SCell addition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE has to blindly search target NR cell SSB in intra-NR redirection, intra-NR handover and NR SCell addition, which will result in UE unnecessary power consumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcRelease					RRCRelease-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCRelease-IEs ::=		SEQUENCE {

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need N

cellReselectionPriorities			CellReselectionPriorities							OPTIONAL,	-- Need M

suspendConfig						SuspendConfig						OPTIONAL,	-- Need R

deprioritisationReq				SEQUENCE {

deprioritisationType				ENUMERATED {frequency, nr},

deprioritisationTimer				ENUMERATED {min5, min10, min15, min30}

}																												OPTIONAL,	-- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

RedirectedCarrierInfo ::=			CHOICE {

nr									 NR-Config,

eutra								ARFCN-ValueEUTRA,

...

}

NR-Config ::= SEQUENCE {

carrierFreq                      ARFCN-ValueNR,

ssbSubcarrierSpacing             SubcarrierSpacing,

smtc-targetNRcell                SMTC                  OPTIONAL,      -- Need S

...

}

SuspendConfig ::= SEQUENCE {

fullI-RNTI							I-RNTI-Value,

shortI-RNTI-value					ShortI-RNTI-Value,

ran-pagingCycle						ENUMERATED{ffsTypeAndValue},

ran-NotificationAreaInfo			RAN-NotificationAreaInfo 	OPTIONAL,	-- Need M,

periodic-RNAU-timer					ENUMERATED {ffsValue},

nextHopChainingCount				NextHopChainingCount

}

CellReselectionPriorities ::=		SEQUENCE {

freqPriorityListEUTRA				FreqPriorityListEUTRA			OPTIONAL,		-- Need M

freqPriorityListNR					FreqPriorityListNR				OPTIONAL, 		-- Need M

t320								ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

spare1}						OPTIONAL,		-- Need R

...

}

-- FFS Maximum number of frequency in priority list

FreqPriorityListEUTRA ::=			SEQUENCE (SIZE (1..ffsValue)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=				SEQUENCE (SIZE (1..ffsValue)) OF FreqPriorityNR

FreqPriorityEUTRA ::=				SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityNR ::=				SEQUENCE {

carrierFreq							ARFCN-ValueNR,

cellReselectionPriority				CellReselectionPriority

}

RAN-NotificationAreaInfo		::=  CHOICE {

cellList				PLMN-RAN-AreaCellList,

ranAreaConfigList		PLMN-RAN-AreaConfigList

}

PLMN-RAN-AreaCellList	::=			SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

-- Sum of cells from all PLMNs does not exceed 32

PLMN-RAN-AreaCell	::=	SEQUENCE {

plmn-Identity				PLMN-Identity		OPTIONAL,

ran-AreaCells				SEQUENCE (SIZE (1..32)) OF	CellIdentity

}

PLMN-RAN-AreaConfigList	::=			SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig	::=	SEQUENCE {

plmn-Identity				PLMN-Identity		OPTIONAL,

ran-Area					SEQUENCE (SIZE (1..16)) OF	RAN-AreaConfig

}

RAN-AreaConfig	::=	SEQUENCE {

trackingAreaCode			TrackingAreaCode,

--Sum of RAN-AreaCodes all PLMNs does not exceed 32

ran-AreaCodeList			SEQUENCE (SIZE (1..32)) OF	RAN-AreaCode		OPTIONAL

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

Editor’s Note: FFS Whether fulll and/or short I-RNTI(s) are defined as optional fields in SuspendConfig.

Editor’s Note: FFS Signalling optimizations and/or default configuration for RNA Area and/or corrections (TAI vs. TAC).

Editor’s Note: FFS Whether RejectWaitTimer is needed in RRCRelease message.

Editor’s Note:

RRCRelease field descriptions

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised. The UE shall be able to store a depriotisation request for up to X frequencies (applicable when receiving another frequency specific deprioritisation request before T325 expiry).

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

suspendConfig Indicates configuration for the RRC_INACTIVE state.

NR-Config Indicate the information to redirect the UE to inter-frequency NR cell. It includes target NR frequency, SCS of the target NR cell SSB, and optional SMTC of the redirected target NR cell SSB based on the timing reference of current serving cell. If the SMTC of target cell SSB (i.e. IE smtc-targetNRcell is absent), the UE will assume 5ms SMTC periodicity as specified in section 4.1 of TS 38.213.

PLMN-RAN-AreaConfig field descriptions

cellList Indicates a list of cells configured as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or RAN area code(s) to configure a UE.

ranAreaConfigList Indicates a list of RAN area codes or RA code(s) as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN.

Editor’s Note: FFS Confirm the number X of deprioritisation frequencies the UE shall be able to store.

End of change

Start of 2nd change

CellGroupConfig information element

-- ASN1START

-- TAG-CELL-GROUP-CONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig	::= 						SEQUENCE {

cellGroupId									CellGroupId,

rlc-BearerToAddModList 						SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig				OPTIONAL,   -- Need N

rlc-BearerToReleaseList						SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity			OPTIONAL,   -- Need N

mac-CellGroupConfig							MAC-CellGroupConfig												OPTIONAL,	-- Need M

physicalCellGroupConfig						PhysicalCellGroupConfig											OPTIONAL,	-- Need M

spCellConfig								SpCellConfig													OPTIONAL, 	-- Need M

sCellToAddModList							SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig				OPTIONAL,	-- Need N

sCellToReleaseList							SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex				OPTIONAL,	-- Need N

...,

[[

reportuUplinkTxDirectCurrent			ENUMERATED {true}												OPTIONAL	-- Cond BWP-Reconfig

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=						SEQUENCE {

servCellIndex						ServCellIndex															OPTIONAL,	-- Cond SCG

reconfigurationWithSync 			ReconfigurationWithSync 												OPTIONAL,	-- Cond ReconfWithSync

rlf-TimersAndConstants				SetupRelease { RLF-TimersAndConstants }									OPTIONAL,	-- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                         								OPTIONAL,	-- Need S

spCellConfigDedicated				ServingCellConfig														OPTIONAL,	-- Need M

...

}

ReconfigurationWithSync ::=			SEQUENCE {

spCellConfigCommon					ServingCellConfigCommon													OPTIONAL,	-- Need M

newUE-Identity						RNTI-Value,

t304								ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated				CHOICE {

uplink								RACH-ConfigDedicated,

supplementaryUplink				RACH-ConfigDedicated

}																											OPTIONAL,	-- Need N

...,

[[

smtc-targetNRcell                     SMTC               OPTIONAL,      -- Need S

]]

}

SCellConfig ::=						SEQUENCE {

sCellIndex							SCellIndex,

sCellConfigCommon					ServingCellConfigCommon													OPTIONAL,	-- Cond SCellAdd

sCellConfigDedicated				ServingCellConfig														OPTIONAL,	-- Cond SCellAddMod

...,

[[

smtc-targetNRcell                     SMTC              OPTIONAL,     -- Need S

]]

}

-- TAG-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportuUplinkTxDirectCurrent Enables reporting of uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 ([14], Table 8.1.1-1). n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets on-going RLF timers and counter. In EN-DC, rlf-TimersAndConstants cannot be released.

sCellToAddModList List of seconary serving cells (SCells) to be added or modified.

sCellToReleaseList List of seconary serving cells (SCells) to be released

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc-targetNRcell This IE indicates the SMTC of the target cell SSB based on the timing reference of current serving cell in case of synchronous reconfiguration. If this IE is absent, UE will assume 5ms SMTC periodicity as specified in section 4.1 of TS 38.213.

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. In EN-DC, rlf-TimersAndConstants cannot be released.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

SCellConfig field descriptions

smtc-targetNRcell This IE indicates the SMTC of the target cell SSB based on the timing reference of current serving cell in case of NR SCell addition. If this IE is absent, UE will assume 5ms SMTC periodicity as specified in section 4.1 of TS 38.213.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed in the same message. Otherwise it is absent. |
| L | T |
| L | T |
| ReconfWithSync | The field is mandatory present in case of SpCell change, PSCell addition and security key change; otherwise it is optionally present, need M. |
| SCellAdd | The field is mandatory present, need M, upon SCell addition; otherwise it is not present |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

End of change