3GPP TSG-RAN WG2 Meeting #103												R2-18xxxxx

Gothenburg, Sweden, 20th - 24th August 2018

Source: 	vivo

Title:  	Evaluation of the RLM for NR-U

Agenda Item:	11.2.2.2

Document for:	Discussion and Decision

# Introduction

In order to coexist with WiFi devices, gNB in NR unlicensed needs to sense channels based on LBT process, leading to discontinuous transmission on a carrier with limited maximum transmission duration. This means that there is high probability the physical layer in the UE indicates out-of-sync to higher layers when RLM-RS(s) cannot be transmitted by the gNB due to LBT failure. As a result, the UE would probably declare RLF. In this contribution, we will focus on the evaluation of RLM for NR unlicensed. Based on the system-level simulation results, we attempt to give some quantitative analysis on the relationship between LBT failure probability and the total number of RLF occurrences of cell center users and cell edge users.

# Discussion

## RLM and RLF

In NR, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate. If SSB is configured as RLM-RS, Qout shall correspond to 10% block error rate of a hypothetical PDCCH. The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate. If SSB is configured as RLM-RS, Qin shall correspond to 2% block error rate of a hypothetical PDCCH.

The physical layer in the UE shall in frames where the radio link quality is assessed indicate out-of-sync (OOS) to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE shall in frames where the radio link quality is assessed indicate in-sync (IS) to higher layers.

If higher layers in the UE receive N310 consecutive OOS indications, T310 timer will start. Before expiry of T310, if higher layers receive N311 consecutive IS indications, T310 stops. Otherwise, UE declares RLF if T310 expires. An example of RLM procedure in NR is shown in the following figure.

Figure 1. An example of RLM procedure in NR

## Simulation Scenarios and Evaluation Metrics

This simulation focuses on the dense-urban (macro-layer) deployment scenario, in which all macro cells are on the same frequency layer. The number of unique RLM-RS(s) using different set of resources per BWP is set to X. For example, when SSB is configured as RLM-RS, a case for X=4 is illustrated in Figure 2 below.

Figure 2. A case for X=4

As for quantitative analysis, four metrics below are used:

- : The probability of indicating OOS to higher layers per indication period, which can be calculated according to the following equation,

: The probability of not indicating any indication to higher layers per indication period, which can be calculated according to the following equation,

- : The probability of indicating IS to higher layers per indication period, which can be calculated according to the following equation,

- : The total number of RLF occurrences among all the cell users over evaluation period.

## Analysis of Simulation Results

In RLM procedure, the threshold Qout and Qin are correspond to 10% block error rate of a hypothetical PDCCH and 2% block error rate of a hypothetical PDCCH. Therefore, it is necessary to access the CDF of cell users’ SINR and evaluate the link curve performance of polar code. More details of the simulation assumptions can be found in annex A, annex B, and annex C.

The simulation results are shown in the figure below.

Figure 3. LBT failure probability vs. RLF occurrences

The following table summarizes the simulation results.

Table 1. Summary of simulation results

| The total number of RLF occurrences |  | LBT failure probability |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | =0 | =0.1 | =0.2 | =0.3 | =0.4 | =0.5 | =0.6 |
| X=1 | Cell center users | 95 | 387 | 1024 | 1884 | 2834 | 3581 | 4153 |
|  | Cell edge users | 2235 | 3069 | 3532 | 3929 | 4226 | 4425 | 4516 |
| X=2 | Cell center users | 0 | 0 | 8 | 139 | 738 | 1907 | 3167 |
|  | Cell edge users | 64 | 300 | 881 | 1721 | 2695 | 3487 | 4088 |
| X=3 | Cell center users | 0 | 0 | 0 | 17 | 265 | 1194 | 2619 |
|  | Cell edge users | 0 | 5 | 80 | 459 | 1361 | 2531 | 3539 |
| X=4 | Cell center users | 0 | 0 | 0 | 6 | 155 | 954 | 2384 |
|  | Cell edge users | 0 | 0 | 6 | 111 | 666 | 1806 | 3085 |
| X=5 | Cell center users | 0 | 0 | 0 | 5 | 127 | 858 | 2294 |
|  | Cell edge users | 0 | 0 | 1 | 35 | 367 | 1402 | 2761 |
| X=6 | Cell center users | 0 | 0 | 0 | 4 | 110 | 822 | 2254 |
|  | Cell edge users | 0 | 0 | 0 | 16 | 232 | 1149 | 2577 |
| X=7 | Cell center users | 0 | 0 | 0 | 4 | 107 | 812 | 2243 |
|  | Cell edge users | 0 | 0 | 0 | 8 | 186 | 1003 | 2442 |
| X=8 | Cell center users | 0 | 0 | 0 | 3 | 107 | 804 | 2223 |
|  | Cell edge users | 0 | 0 | 0 | 7 | 144 | 926 | 2371 |

As shown in Figure 3 and Table 1, there is a positive correlation between the LBT failure probability and the total number of RLF occurrences among all the cell users, regardless of cell centre users or cell edge users. For instance, the number of RLF occurrences increase largely from 387 to 1024 (164.6%) for the cell centre users and the number of RLF occurrences will increase slightly from 3069 to 3532 (15.1%) for the cell edge users, when the number of RLM-RS is set to 1 and LBT failure probability increases from 0.1 to 0.2. This is because LBT failure will cause the RLM-RS(s) cannot be successfully transmitted by the gNB over an indication period. The physical layer in the UE indicates OOS to high layers, which helps to accelerate the pace of RLF process.

Observation 1: The total number of RLF occurrences has positive correlation with LBT failure probability.

Observation 2: LBT failure has greater impact on cell center users than cell edge users.

Give that the LBT failure probability is 10%. If the maximum number of RLM-RS(s) is set from 1 to 2, the number of RLF occurrences will decrease largely from 387 to 0 (100%) for the cell centre users and the number of RLF occurrences will decrease largely from 3069 to 300 (90.2%) for the cell edge users. This means that the more the number of RLM-RSs, the users are less likely to declare RLF. The reason is that the probability of indicating OOS will decrease and the probability of indicating IS will increase when the number of RLM-RS(s) configured increases.

Observation 3: Increasing the number of RLM-RS(s) helps to reduce the total number of RLF occurrences.

Regarding the number of RLM-RS(s), although more configured RLM-RS resources can reduce the number of RLF occurrences, the extra benefit against radio resource overhead and more sophisticated design needs to be carefully considered. For example, given that the LBT failure probability is 40%, when the maximum number of RLM-RS(s) is set from 5 to 8, for the cell center users, the total number of RLF occurrences will only decrease by 15.8%.

Observation 4: When the maximum number of RLM-RS(s) is larger than 5, the benefit from more RLM-RS resources may be small.

According to the observations given above, we think RAN2 should discuss further on the RLM procedure in NR-U in order to make progresses on mobility aspects of NR unlicensed. Some solutions to reduce the number RLF occurrences due to LBT failure should be studied. The corresponding text proposal capturing the simulation results can be found in [3].

Proposal 1: To capture the simulation results in TR 38.889.

## Potential solutions

### Solution 1: Increase the number of RSs

Figure 1: Increasing the transmission opportunities of RS(s) in both frequency domain and time domain

According to the simulation results give above, one potential solution to reduce the false RLF detection is to increase the transmission opportunities of reference signals for the RLM. From our understanding, we can increase the RLM reference signaling configured for the UE in both the time domain and the frequency domain, as illustrated in Figure 1. In the time domain, we can increase the number of configured references signals in each RLM evaluation period. In the frequency domain, the RLM reference signals in each RLM evaluation period can be configured in different frequencies (e.g. different frequency channels within a BWP, different BWP(s), and different cells).

The main drawbacks of increasing the number of configured RLM RS(s) are the increased resource cost and the increased power consumption of RS monitoring.

### Solution 2: UE detection of RS LBT failure

Figure 2: UE detection of LBT failure

This solution is to allow the UE to detect the RS LBT failure via the UE measurement. For example, if the receive signal strength of the RLM RS is below a threshold (e.g. Threshold_1 as given in Figure 2), the UE does not declare OOS. Thus the false RLF due to the LBT failure can be avoided. The solution can also reduce the number of RS(s) configured to save the network resource and the UE power.

The main drawbacks of this solution is the complexity of the UE to detect the LBT failure. How to differentiate the temporary air-interface blockage between the UE and the gNB and the LBT failure should be further studied.

# Conclusions

Based on the evaluations given above, we have the following Observations and Proposals：

Observation 1: The total number of RLF occurrences has positive correlation with LBT failure probability.

Observation 2: LBT failure has greater impact on cell center users than cell edge users.

Observation 3: Increasing the number of RLM-RS(s) helps to reduce the total number of RLF occurrences.

Observation 4: When the maximum number of RLM-RS(s) is larger than 5, the benefit from more RLM-RS resources may be small.

Proposal 1: To capture the simulation results in TR 38.889.

Proposal 2: To capture the following solutions in TR 38.889:

- Solution 1: Increase the transmission opportunities of RLM RS(s) in each RLM evaluation period

  - More RLM RSs can be configured in each RLM evaluation period.

  - More RLM RSs can be configured in different frequency domains (e.g. different frequency channels within a BWP, different BWP(s), and different cells)

- Solution 2: Allow the UE detection of the RS LBT failure

  - When the measurement result of the RLM RS is below a threshold, the UE does not declare OOS.

# Reference

[1] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”.

[2] 3GPP TS 38.213, “Physical layer procedures for control”.

[3] R2-1809883, Text proposals for the evaluation results of the RLM for NR-U, vivo

# Annex

## Annex A: Simulation assumptions for RLM procedure

Table 2. Initial parameters for RLM

| Attributes | Values or assumptions |
| --- | --- |
| N310 | 4 |
| N311 | 4 |
| T310 | 500 ms |
| The number of RLM-RSs (X) | Up to 8 |
| RLM-RS resource | SSB |
| SSB periodicity | 10 ms |
| Indication period | 10 ms |
| Evaluation period | 5 s |
| The number of gNBs | 19 |
| The number of users in the network | 570 (19*3*10) |

## Annex B: Simulation assumptions for system-level simulation

Table 3. System parameters of system-level simulation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT. |
| Beam selection | Based on the criteria of maximizing receive power after beamforming. |
| ISD | 200 m |
| BS Tx power | 33 dBm |
| Minimum distance between gNB and UE | 35m |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1,1,2,1,2) (dH, dV) = (0.5, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (Mg, Ng, M, N, P,) = (1, 1, 1, 1, 2); (dV,dH) = (0, 0)λ. The polarization angles are 0 and 90 |
| UE array orientation | ΩUT,uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | Omni |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |

Based on the simulation assumptions listed above, system-level simulation can be carried out to derive the CDF of cell center users and cell edge users, respectively. Note that the distance between cell center user and gNB is uniform distribution between 35 and 0.9*2/3*ISD, while the distance between cell edge user and gNB is uniform distribution between 0.9*2/3*ISD and 2/3*ISD in this contribution.

The CDF curves of cell center users and cell edge users are shown in the following figure.

Figure 4. CDF of cell users’ SINR

## Annex C: Simulation assumptions for link-level simulation

Table 4. System parameters of link-level simulation

| Attributes | Values |
| --- | --- |
| Channel mode | TDL-A |
| Channel coding type | Polar code |
| Modulation | QPSK |
| Code rate | 1/3 |
| Decoding algorithm | SCL |
| The number of RBs | 50 |
| Target BLER | 10-1, 2*10-2 |
| TB-CRC type | CRC-24 |
| Simulation frames | 10000 |

According to the simulation assumptions listed above, the link curve performance of polar code can be assessed via link-level simulation. And then the threshold Qout and Qin can be determined based on Figure 4.

Figure 4. SNR vs. BLER