3GPP TSG-RAN WG2 Meeting #103bis                                                  R2-1813725

Chengdu, China, 8th - 12th Oct 2018

Source:	CATT

Title:	Mode 2 Resource Allocation

Agenda Item:	10.x.x

Document for:	Discussion and Decision

Introduction

At 3GPP RAN#80 meeting, a new SI “Study on NR V2X” was approved. One of the objectives is to study sidelink resource allocation mechanism. On the last RAN1 meeting, this topic was initially discussed in RAN1 and reached the following agreements:

At least two sidelink resource allocation modes are defined for NR-V2X sidelink communication - Mode 1: Base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s) - Mode 2: UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources - Notes: - eNB control of NR sidelink and gNB control of LTE sidelink resources will be separately considered in corresponding agenda items. - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where - UE autonomously selects sidelink resource for transmission - UE assists sidelink resource selection for other UE(s) - UE is configured with NR configured grant (type-1 like) for sidelink transmission - UE schedules sidelink transmissions of other UEs RAN1 to continue study details of resource allocation modes for NR-V2X sidelink communication

In this contribution, we will discuss which issues needs to be addressed for NR Mode 2 resource allocation from RAN2 perspective.

Discussion

From RAN2 perspective, when discussing NR Mode 2 resource allocation, at least the following aspects need to be considered:

Cross-carrier resource configuration

On the last RAN1 meeting, the following agreement was reached:

- NR Uu can assign NR sidelink resources for the following: - Shared licensed carrier between Uu and NR sidelink - Dedicated NR sidelink carrier

Since NR Uu supports the sidelink resource allocation on both shared carrier and dedicated NR sidelink carrier, hence it is obvious that cross-carrier configuration should be supported for NR V2X Mode 2 resource allocation.

Proposal 1: RAN2 should support cross-carrier resource configuration in NR V2X.

Geography-based resource configuration

In LTE, geography-based resource configuration is introduced to avoid the near-far effect. The geography is classified into multiple zones, and adjacent zones will use different Tx pool. In NR, whether geograhy-based resource allocation can be reused for Mode 2 resource allocation needs further discussion.

Proposal 2: Suggest RAN2 to study whether geography-based resource configuration should be supported for NR Mode 2 resource allocation.

Tx resource pool configuration

According to RAN1 agreements, four sub-modes of Mode 2 are defined till now:

- Sub-mode a)

In our understanding, this sub-mode is similar as the LTE V2X Mode 4 resource allocation. UE should first acquire the Tx resource pool, and then UE can perform the autonomously resource selection.

- Sub-mode b) / d)

In the discussion of RAN1, sub-mode b) and d) are mainly introduced based on [2] and [3]. That is to say a local manager UE or header UE will be introduced and it should first be granted with a Tx resource pool, and then allocate resource for the other UEs under its control.

- Sub-mode c)

The configured grant can also be regarded as a kind of Tx resource pool.

From the above analysis, it can be deduced that when discussing Mode 2 resource allocation, it should first discuss how to configure the Tx resource pool. According to LTE V2X, the Tx pool configurations are different for different UEs:

- For out-of-coverage UE: Tx pool is preconfigured;

- For RRC_IDLE UE: Tx pool is configured through SIB21;

- For RRC_CONNECTED UE: Tx pool is configured through dedicated RRC signaling.

But for NR, it has the following different characteristics compared with LTE:

- In LTE, SIB is always broadcasted. But for NR, SI is classfied into minmum SI and other SI. MIB and SIB1 are minmum SI and always broadcated; all the other SIs are other SI and provided on-demand;

- Area specific SIB is introduced, which means the same SI can be valid in more than one cell;

- Besides the RRC_IDLE and RRC_CONNECTED states, a new RRC states (RRC_INNACTIVE) is introduced;

- NR supports UE assistant resource allocation for other UEs;

- NR introduced new QoS parameters.

Based on the above different characteristis, when discussing the Mode 2 Tx resource pool configuration, at least the following issues need to be addressed:

- Issue 1: whether new SIB needs to be introduced to support NR sidelink V2X and if it is needed, how to delivery the new SIB, always broadcasted or delivered on-demand?

- Issue 2: with the introduction of Area specific SIB, what is the impact on the UE behaviors when acquiring the Tx resource pool?

- Issue 3: how to treat the RRC_INACTIVE UE when acquiring the Tx resource pool?

- Issue 4: whether the Tx resource pool in NR still need to link with the QoS parameters? And how to link between them?

Proposal 2: Suggest RAN2 to study whether new SIB for NR V2X should be introduced, and if it is introduced, whether it is always broadcasted or delivered on-demand.

Proposal 3: Suggest RAN2 to study the impact of area specific SIB on UE behaviors when acquiring the Tx resource pool.

Proposal 4: Suggest RAN2 to study how to acquire the Tx resource pool for RRC_INACTIVE UE.

Proposal 5: Suggest RAN2 to study whether and how to link the Tx resource pool with the new NR V2X QoS parameters.

Support of different service types

Based on the NR V2X use cases in [4] and the NR V2X evaluation methodology in [5], the NR V2X services can be classified to three types.

- Type 1: periodic services with fixed packet size;

- Type 2: periodic services with variable packet size;

- Type 3: aperiodic services with variable packet size.

For Type 1 serives, it is similar with leacy LTE V2X services, if Mode 2 resource allocation is used, both leacy one-shot and SPS resource allocation can be used. For Type 2 and Type 3 services, there is no problem to use one-shot resource allocation, but whether SPS resource allocation can be used needs further discussion.

In addition, compared with LTE V2X services, NR V2X services have higher requirements on the latency and reliability. How to meet the tight latency and reliablity requirements in case of Mode 2 resource allocation should be consiered. E.g., whether duplication should be supported and which layer performs the duplication?

Proposal 6: For NR Mode 2 resource allocation, both one shot and SPS resource allocation should be supported.

Proposal 7: Suggest RAN2 to study whether SPS resource allocation need to be supported for periodic services with variable packet size and aperiodic services with varialbe packet size.

Proposal 8: Suggest RAN2 to study how to satisfy the latency and reliablity requirement of NR V2X services.

UE assistant resource allocation

For sub-mode b) and d), UE assistant resource allocation will be used. The procedures of these two sub-modes should be studied in RAN2 which at least includes the following issues:

- Issue 1: how to form the group?

- Issue 2: how to select the local manager UE or header UE in the group？

- Issue 3: How does the local manager UE or header UE manage the resource allocation of the other UEs under its control?

Proposal 9: Suggest RAN2 to study the procedures of the UE assist resource allocation.

Collosion and interference reduction

Mode 2 resource allocation is based on contention, how to reduce the collision in NR should be discussed. Whether CSMA-CA like solution or new solution should be introduced needs further study.

In addition, it is known to all that exceptional pool is introduced in LTE to reduce the interference in case of HO and RLF. During HO and RLF, UE loses the RRC connection with the source cell. If UE still uses the normal Tx pool assigned by the source cell, it may result interference. Hence exceptional pool is introduced. In NR, the same case always exists, whether exceptional pool or other mechanism to avoid the interference needs to be discussed.

Proposal 10: Suggest RAN2 to study how to reduce the collision and interference in NR V2X.

Congestion control

In LTE V2X, congestion control was introduced to improve the Mode 2 resource pool efficiency. It was implemented based on the mapping table amongst the CBR range, PPPP and L1 parameters. In NR V2X, whether and how to perform the congestion control should be studied further.

Proposal 11: Suggest RAN2 to study whether and how to perform the congestion control in NR V2X.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: RAN2 should support cross-carrier resource configuration in NR V2X.

Proposal 2: Suggest RAN2 to study whether geography-based resource configuration should be supported for NR Mode 2 resource allocation.

Proposal 2: Suggest RAN2 to study whether new SIB for NR V2X should be introduced, and if it is introduced, whether it is always broadcasted or delivered on-demand.

Proposal 3: Suggest RAN2 to study the impact of area specific SIB on UE behaviors when acquiring the Tx resource pool.

Proposal 4: Suggest RAN2 to study how to acquire the Tx resource pool for RRC_INACTIVE UE.

Proposal 5: Suggest RAN2 to study whether and how to link the Tx resource pool with the new NR V2X QoS parameters.

Proposal 6: For NR Mode 2 resource allocation, both one shot and SPS resource allocation should be supported.

Proposal 7: Suggest RAN2 to study whether SPS resource allocation need to be supported for periodic services with variable packet size and aperiodic services with varialbe packet size.

Proposal 8: Suggest RAN2 to study how to satisfy the latency and reliablity requirement of NR V2X services.

Proposal 9: Suggest RAN2 to study the procedures of the UE assist resource allocation.

Proposal 10: Suggest RAN2 to study how to reduce the collision and interference in NR V2X.

Proposal 11: Suggest RAN2 to study whether and how to perform the congestion control in NR V2X.

Reference

- RP-181480  New SID: Study on NR V2X

- R1-1809069	 Resource allocation mechanism	AT&T

- R1-1809159	 Resource allocation mechanism	NTT DOCOMO, INC.

- 3GPP TR22.886

- 3GPP TR37.885