3GPP TSG-RAN WG2 Meeting #103bis                                                            	R2-1813838

Chengdu, China, 8th - 12th October 2018

Source:	CATT

Title:	Summary of [103#52][NR late drop] MR-DC measurement and gap configuration framework

Agenda Item:	10.5.1

Document for:	Discussion and Decision

- Introduction

This is the email discussion from RAN2#103:

[103#52][NR late drop] MR-DC measurement and gap configuration framework [CATT]:

•	Discuss possible changes for MR-DC@5GC w.r.t. the solution for EN-DC

•	Note: this might apply to all MR-DC@5GC options: NGEN-DC, NE-DC and NR-DC

Intended outcome: Report to next meeting

Deadline:  Thursday 2018-09-20

In this email discussion, we would like to collect companies’ views on measurement and gap configuration frame for NG-EN DC, NE-DC and NR-DC based on TS 37.340.

- Discussion

  - MR-DC Measurement framework

- General MR-DC measurement framework

The general measurement framework in TS 37.340 [1] is shown below:

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in sub-clause 10.2, the Master node should configure the measurement to the UE. In case of the intra-secondary node mobility described in sub-clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required. The Secondary Node Change procedure described in sub-clause 10.5 can be triggered by both the MN (only for inter-frequency secondary node change) and the SN. For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN. Measurements can be configured independently by the MN (for inter-RAT measurement) and by the SN (intra-RAT measurements on serving and non-serving frequencies). … Each node (MN and SN) can configure independently a threshold for the SpCell quality: when the PCell quality is above the threshold configured by the MN, the UE is still required to perform inter-RAT measurements configured by the MN on the SN RAT (while it's not required to perform intra-RAT measurements); when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN. … When SRB3 is not configured, reports for measurements configured by the SN are sent on SRB1. When SRB3 is configured, reports for measurements configured by the SN are sent on SRB3.

The key points of current measurement framework are summarized as below:

- Both MN and SN can construct measurement configuration and receive measurement reports separately;

- MN can configure measurements via SRB1; SN can configure measurements via SRB3(if supported) or embedded in MN RRC message via SRB1;

- Once SRB3 is configured, the measurement report configured by SN can only be transmitted via SRB3;

- MN supports both intra-RAT measurement and inter-RAT measurement; while SN only support intra-RAT measurement;

- The Secondary Node Change procedure described can be triggered by both the MN (only for inter-frequency secondary node change) and the SN.

- For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN. Both MN and SN can configure independent s-Measures towards UE, with s-Measure configured by MN referring to PCell, and the s-Measure configured by SN referring to PSCell.

Although both EN-DC and MR-DC with 5GC (i.e. NG-EN DC and NE-DC) are contained in sub-clause 10.2, 10.3 and 10.3, it is not clear whether the above descriptions of general measurement framework are also applicable to NG-EN DC and NE-DC. Companies are invited to confirm these bullets.

Question 1-1: Confirm whether the above bullets on measurement framework in TS 37.340 are applicable to NG-EN DC. Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All | All EN-DC measurement principles are applicable to NGEN-DC |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | B,c,d,e | For a, the mesaurment requirements may be provided from MN/SN to SN/MN, so the measurement results should be reported to the node which provides the measurement requirement directly in order to reduce the delay. Form signaling overhead and UE powering saving point of view, it is beneficial for the MN and SN to configure one measurement object once. Based on the above 2 reason, it is beneficial to report the measurement result to MN or SN or both depending the indication form the network. For f, in MR-DC, both MN and SN can configure independent s-Measures towards UE, with s-Measure configured by MN referring to PCell, and the s-Measure configured by SN referring to PSCell. However, the measurement configuration may be based on the measurement requirements from another node and the S-measure will stop all non-serving measurements as long as the RSRP of the Spcell meet the threshold no matter where the measurement requirements come from. So it is necessary to reconsider the S-measure mechanism in MR-DC. |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All |  |
| Intel | All | Bullet a-f in principle can be reused for NG-EN DC. |
| Apple | All |  |
| Samsung | All | EN-DC measurements can be applied for NGEN-DC. |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 1-1:

Except one company, all other companies confirmed all bullets on measurement framework in TS 37.340 are applicable to NG-EN DC.

Summary 1-1: All bullets on measurement framework in TS 37.340 are applicable to NG-EN DC.

Question 1-2: Confirm whether the above bullets on measurement framework in TS 37.340 are applicable to NE-DC. Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | a, b, d,e,f | Regarding C: we don’t think SRB3 is essential for NE-DC, as the main reason for introducing SRB3 in EN-DC (the fast air interface of NR for setting up measurement and reporting measurements) doesn’t hold anymore |
| NEC | a,b,d,e,f | For SRB3 in ‘c’, same view as Ericsson as per the past agreement “SCG SRB is not supported on LTE (i.e. for NE-DC)” at NR Ad-hoc2 in Qingdao. With the same reason, SRB3 aspect can be removed from ‘b’ for NE-DC as follows: b. MN can configure measurements via SRB1; SN can configure measurements via SRB3(if supported) or embedded in MN RRC message via SRB1; |
| Qualcomm | a/, d/, e/, f/ | We think both b/ and c/ are not applicable to NE-DC because we think SRB3 shall not be allowed in NE-DC. - In option 3 (EN-DC), SRB3 was agreed because the requirement of quick RRM configuration and reporting in SN. The performance gain comes from the fact that the latency at SN (NR) operating at higher frequency is considerably shorter than at MN (LTE). - However, in option 4 (NE-DC), SN (LTE) is unlikely to have shorter latency than the MN (NR). Furthermore, LTE currently does not support SRB3 as SN and therefore introducing SRB3 in NE-DC case requires further specification effort, which should be avoided at this stage because RAN2 needs to finalize all late drop design by Dec. 2018. In summary, we support following modified bullet b/ for NE-DC: - MN can configure measurements via SRB1; SN can configure its own measurements embedded in MN RRC message via SRB1; |
| vivo | a, d,e,f | Agree that SRB3 is not supported in NE-DC case. |
| LG | a, d, e, f | RAN2 already agreed following in RAN2 201706 AH meeting: 3 SCG SRB is not supported on LTE (i.e. for NE-DC) |
| OPPO | B,d,e | For a,f, the same reason as that for NGEN-DC. For c, it seems that the SRB3 will not be supported. |
| ZTE | a, d, e, f | RAN2 already agreed that SRB3 is not supported for NE-DC. So SN measurement configuration and SN measurement report can only be embedded in MN RRC messages. |
| MediaTek | a,d,e,f | We understand that SRB3 is not supported for NE-DC |
| Nokia | a,b,d,e,f | No SRB3 in NE-DC has been agreed in RAN2, but SeNB still can configure intra-RAT measurement for PScell change and/or SN initiated SN change cases. |
| Intel | a, b, d, e, f | SRB is not supported for NE-DC. |
| Apple | a, b, d, e, f | SRB3 is not supported on LTE SN. |
| Samsung | a, b, d, e, f | We believe SRB3 is not supported for NE-DC. |
| NTT DOCOMO, INC. | a, b, d, e, f | Same view as other companies that SRB3 is not supported for NE-DC |
| Huawei, HiSilicon | a, b, d, e, f | Same view like Ericsson. |
| CATT | a,d.e.f | SRB3 is not supported for NE-DC. |

Summary of Question 1-2:

All companies agreed SRB3 is not supported for NE-DC. Based on the feedback applicable bullets, we conclude:

Summary 1-2: Bullets a, d, e, and f on measurement framework in TS 37.340 are applicable to NE-DC.

Summary 1-2bis: For NE-DC, MN can configure measurements via SRB1 while SN can only configure measurements embedded in MN RRC message via SRB1.

The measurement framework of NR-DC has not been discussed yet. Companies are invited to provide their opinions on the open issue:

Question 1-3: Can the above bullets on measurement framework in TS 37.340 be applicable in NR-DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | B,c,d,e | For a,f, the same reason as that for NGEN-DC. |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All |  |
| Intel | All | Bullet a-f in principle can be reused for NR DC. |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All | Regarding d, MN does not need to support inter-RAT measurement for NR-DC. So this bullet can be modified as follows: d. MN supports both intra-RAT measurement and inter-RAT measurement; while SN only support intra-RAT measurement; |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 1-3:

Except one company, all other companies confirmed all bullets on measurement framework in TS 37.340 are applicable to NR-DC.

Summary 1-3: All bullets on measurement framework in TS 37.340 are applicable to NR-DC.

Combined summary 1-1, summary 1-2 and summary 1-3, we conclude:

Proposal 1: Descriptions on measurement framework in TS 37.340 are applicable to NG-EN DC, NE-DC, and NR-DC, except that SN can only configure measurements embedded in MN RRC message via SRB1 for NE-DC.

- Measurement capability coordination

The descriptions on measurement capability coordination in TS 37.340 [2] are shown below:

The MN indicates the maximum number of frequency layers and measurement identities that can be used in the SN to ensure that UE capabilities are not exceeded. If MN and SN both configure measurements on the same carrier frequency then the configurations need to be consistent (if the network wants to ensure these are considered as a single measurement layer). NOTE: The SN cannot renegotiate the number of frequency layers allocated by the MN in this version of the protocol.

The main conclusions of measurement capability coordination were made in RAN2#99bis:

R2-1711753 Measurement Capability Coordination for EN-DC NTT DOCOMO INC. discussion Rel-15 Agreements 1: Working assumption is confirmed (UE receives independent measurement configuration from MN and SN. UE does not do any manipulation of parameters in order to make the measurements configurations consistent (i.e. network is responsible to ensure they are consistent if it wants to ensure these are considered as a single measurement layer) R2-1712045 (Should be R2-1712045) Report of offline discussion #17 on Measurement Capabilities Coordinations DOCOMO Agreements 1 Tthere will be a signalling to coordinate the number of frequency layer to be used in MN and SN. 2 The MN indicates the number of frequency layers that can be used in the SN 3: Re-negotiation (SN signalling to MN for the purpose to ask for more number of frequency layer) is not supported (at least in Rel-15).

The key points of current measurement coordination framework are summarized as below:

- MN can indicate the maximum number of frequency layers that can be used in SN to SN;

- MN can indicate the maximum number of measurement identities that can be used in SN;

- Re-negotiation of the number of frequency layers and measurement identities is not supported in R15;

- If MN and SN both configure measurements on the same carrier frequency then the configurations need to be consistent. UE does not do any manipulation of parameters to make the measurement configurations from the MN and SN consistent (i.e. network is responsible to ensure that they are consistent if it wants to ensure measurements from the MN and SN on the same frequency layer are considered as a single measurement layer)

It is not clear whether these agreements are also applicable to NG-EN DC and NE-DC. And the measurement capability coordination in NR-DC has never been discussed either. Companies are invited to provide their opinions on the following questions:

Q1-4: For the bullets on measurement capability coordination in TS 37.340, can these be applicable in NG-EN DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All | One small comment to the first sentence of ‘d’, i.e. “If MN and SN both configure measurements on the same carrier frequency then the configurations need to be consistent.” This is not entirely correct, because this is not always necessary on top of ‘a’ and ‘b’. |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | a,b,c | For d, even RAN2 had the agreed principle on“the configurations need to be consistent”, but there was no MO configuration coordination mechanism defined in EN-DC. The same principle can be reused for other DCs, but we need to discuss how to keep consistent between MN and SN in stage3. |
| Intel | All in general | For point d, MN and SN should at least have the same SMTC configuration if measurement on the same carrier frequency. |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | What is in 37.340 in section 7.2 | We think the bullets above do not accurately reflect what is in 37.340, we agree with what is in 37.340 in 7.2 but not with the above bullets. |
| CATT | All |  |

Summary of Question 1-4:

Most companies agreed all bullets on measurement capability coordination in TS 37.340 are applicable to NG-EN DC.

Summary 1-4: All bullets on measurement capability coordination in TS 37.340 are applicable to NG-EN DC.

Q1-5: For the bullets on measurement capability coordination in TS 37.340, can these be applicable in NE-DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | a,b,c | Same comments as Q1-4 |
| Intel | All in general | For point d, MN and SN should at least have the same SMTC configuration if measurement on the same carrier frequency. |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | What is in 37.340 in section 7.2 | We think the bullets above do not accurately reflect what is in 37.340, we agree with what is in 37.340 in 7.2 but not with the above bullets. |
| CATT | All |  |

Summary of Question 1-5:

Most companies agreed all bullets on measurement capability coordination in TS 37.340 are applicable to NE-DC.

Summary 1-5: All bullets on measurement capability coordination in TS 37.340 are applicable to NE-DC.

Q1-6: For the bullets on measurement capability coordination in TS 37.340, can these be applicable in NR-DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All |  |
| Qualcomm | All bullets |  |
| Vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | a,b,c | Same comments as Q1-4 |
| Intel | All in general | For point d, MN and SN should at least have the same SMTC configuration if measurement on the same carrier frequency. |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | What is in 37.340 in section 7.2 | We think the bullets above do not accurately reflect what is in 37.340, we agree with what is in 37.340 in 7.2 but not with the above bullets. |
| CATT | All |  |

Summary of Question 1-6:

Most companies agreed all bullets on measurement capability coordination in TS 37.340 are applicable to NR-DC.

Summary 1-6: All bullets on measurement capability coordination in TS 37.340 are applicable to NR-DC.

Combined summary 1-4, summary 1-5 and summary 1-6, we conclude:

Proposal 2: Descriptions on measurement capability coordination in TS 37.340 are applicable to NG-EN DC, NE-DC and NR-DC.

- Measurement result exchange for MR-DC measurement between inter-nodes

The followings are agreements on measurement result exchange between inter-nodes in TS 37.340[1].

Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure. Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure. Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure. In EN-DC, measurement results according to measurement configuration from the MN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message.

The key points of current measurement result exchange between inter-nodes are summarized as below:

- Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure.

- Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure.

- Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure.

- Measurement results according to measurement configuration from the MN are encoded according to SN’s RRC when they are provided by MN to SN in SgNB Addition Request message.

- During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to SN’s RRC when they are provided by MN to SN in SgNB Addition Request message.

It is not clear whether these bullets are applicable in NG-EN DC, NE-DC, and NR-DC.

Question 1-7: Whether the bullets on measurement result exchange between nodes in TS 37.340 can be applicable in NG-EN DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All |  |
| Intel | All |  |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 1-7:

All companies confirmed the above bullets on measurement result exchange between nodes are applicable to NG-EN DC.

Summary 1-7: All bullets on measurement result exchange between nodes are applicable to NG-EN DC.

Question 1-8: Whether the bullets on measurement result exchange between nodes in TS 37.340 can be applicable in NE-DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All | All can be applicable with clarification to ‘d,e’ by Ericsson |
| Qualcomm | a/, b/, c/ (and modified d/, e/ by Ericsson) | In option 4 (NE-DC), LTE works as SN. Following the principle that source adapts the target format, we think the measurement results of SN should be encoded as LTE RRC format when they are provided by MN to SN in SgNB Addition Request message. In summary, we support modified d/, e/ by Ericsson, or the following modified d/ and e/ for NE-DC: - Measurement results according to measurement configuration from the MN are encoded according to LTE RRC when they are provided by MN to SN in SgNB Addition Request message. - During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to LTE RRC when they are provided by MN to SN in SgNB Addition Request message. |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All | Agree with the clarification to ‘d,e’ by Ericsson. One more question for clarification: in NE-DC, will the inter-node RRC messages(e.g. CG-ConfigInfo, CG-ConfigInfo) be defined in 36.331 instead of 38.331? |
| MediaTek | All |  |
| Nokia | All | We are fine with Ericsson change in d and e. |
| Intel | All |  |
| Apple | ALL |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All | All can be applicable with clarification to ‘d,e’ by Ericsson |
| CATT | All |  |

Summary of Question 1-8:

With clarification to ‘d,e’ by Ericsson, all companies confirmed all bullets on measurement result exchange between nodes are applicable to NE-DC.

Summary 1-8: All bullets on measurement result exchange between nodes are applicable to NE-DC.

Question 1-9: Whether the bullets on measurement result exchange between nodes in TS 37.340 can be applicable in NR-DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, c..)? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All |  |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All |  |
| Intel | All |  |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 1-9:

With clarification to ‘d,e’ by Ericsson, all companies confirmed all bullets on measurement result exchange between nodes are applicable to NR-DC.

Summary 1-9: All bullets on measurement result exchange between nodes are applicable to NR-DC.

Combined summary 1-7, summary 1-8 and summary 1-9, we conclude:

Proposal 3: Descriptions on measurement result exchange between nodes in TS 37.340 are applicable to NG-EN DC, NE-DC and NR-DC, with the modification that measurement results are encoded according to SN’s RRC instead of NR RRC when they are provided by MN to SN.

  - MR-DC Measurement gap

The following descriptions are the general descriptions on measurement gaps and gap sharing configurations. The supported measurement gaps and gap sharing configuration are decided by RAN4. Hence, we assume that those are supported in all MR-DC cases. If there is any difference, we can update based on RAN4’s conclusion.

Per-UE or per-FR measurement gaps can be configured, depending on UE capability and network preference. Per-UE gap applies to both FR1 (LTE and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. The UE may also be configured with a per-UE gap sharing configuration (applying to per-UE gap) or with two separate gap sharing configurations (applying to FR1 and FR2 measurement gaps respectively) [8].

- The node which decides measurement gap and gap sharing configuration

For the node which decides measurement gap/gap sharing configuration and measurement gap coordination between MN and SN, further discussion is needed for MR-DC.The following are the descriptions on the node which decides measurement gap and gap sharing configuration in EN-DC [2].

In EN-DC, if per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2. The measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration.

In EN-DC, only SCG can work with FR2 frequency. Per-FR2 gap only influences the scheduling of serving cell(s) in SCG operated in FR2 frequency. Hence, RAN2 concluded that per-FR2 gap is configured by SN while per-UE gap and per-FR1 gap are configured by MN in EN-DC.

The key points of current measurement gap and gap sharing configuration are summarized as below:

- per-UE gap/gap sharing configuration or per FR1 gap/gap sharing configuration is configured by MN;

- per FR2 gap/gap sharing configuration is configured by SN;

The architecture of NG-EN DC is similar as EN-DC. Hence, we have the following question:

Question 2-1: For the bullets on measurement gap and gap sharing configuration in EN-DC, can these be applicable in NG-EN DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, )? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All | The same principles can be applied in NG EN-DC |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All |  |
| Intel | All |  |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 2-1:

All companies confirmed the bullets on measurement gap/gap sharing configuration in EN-DC are applicable to NG-EN DC.

Proposal 4: Descriptions on measurement gap/gap sharing configuration for EN-DC in TS 37.340 are applicable to NG-EN DC.

However, for NE-DC and NR-DC, the MN can be influenced by per-UE gap, per FR1 gap and per FR2 gap.

For the measurement gap and gap sharing configuration in NE-DC, the following are the possible options:

Option 1: Measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

Option 2: Per-UE gap/gap sharing configuration or per FR2 gap/gap sharing configuration is configured by MN, while per FR1 gap/gap sharing configuration is configured by SN.

Option 3: Per-UE gap/gap sharing configuration or per FR2 gap/gap sharing configuration is configured by MN, while per FR1 gap/gap sharing configuration can be configured by MN or SN.

Question 2-2: Which option above is preferred for NE-DC measurement gap/gap sharing configuration?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | Option 3 | When the SN decides to configure a measurement that requires FR1 gaps, it can include the measurement configuration as well as the gap configuration in the CG-Config that is part of the SN Modification Required message. This way, the MN will be aware of the gaps in case it will configure serving cells on FR1 later. If it is the MN that needs to configure a measurement that needs an FR1 gap first, and the gap is not set by SN yet, it can set it and communicate to the SN via the cg-configInfo that is part of the SN Modification request message. Doing it this way is the most flexible because the node that needs the gap first configures it (i.e. no need to wait for the gaps to be configured by the other node). |
| NEC | Option 3 | Agree with Ericsson |
| Qualcomm | Option 1 | In EN-DC, RAN2 made the agreement that FR2 gap is configured by SN because the intention to keep LTE RRC structure without introducing measurement gap configuration for FR2 (as there is no MeNB deployment in FR2). However, we don’t think the above intention is still valid in both NE-DC and NR-DC because: - In NE-DC, SN is LTE which don’t need to introduce FR2 gap configuration in TS 36.331 - In NR-DC, both MN and SN can work in FR2. Considering the EN-DC gap configuration is confusing because both 36.331 and 38.331 can configure gap, we think the re-design of gap configuration is worth for both NE-DC and NR-DC and prefer a same framework for both. Furthermore, RAN2 has introduced explicitly the 3 types of gap configurations in TS38.331: MeasGapConfig ::= SEQUENCE { gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M ..., [[ gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M gapUE SetupRelease { GapConfig } OPTIONAL -- Need M ]] } We think it is straight forward to reuse the above ASN.1 structure and principle of LTE DC gap configuration, i.e. measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap. |
| vivo | Option 3 | This option is the most flexible one. |
| LG | Option 3 | Considering that both MN and SN would influence the FR1 gap/ gap sharing configuration, option 3 seems reasonable. |
| OPPO | Option 3 |  |
| ZTE | Option1 | In EN-DC, both MN and SN can configure measurements on NR FR1 frequencies, while the FR1 gap can only be configured by MN, and the gap calculation is based on the SFN/subframe of PCell. Here, for NE-DC, the situation is quite similar with EN-DC, so we didn’t see much benefit to let SN to configure FR1 gap, and it is not clear which cell’s SFN/Subframe will be used for FR1 gap calculation. Since SRB3 is not supported in NE-DC, in any case the SN configured measurement will be delivered via MN. So it is easy for SN to send gap request to MN in a single inter-node RRC message. |
| MediaTek | Option 1 | For FR1 gap pattern of per FR gap, we think it better let MN to configure it. If both MN and SN could configure it. We have solve some configuration conflict issue. |
| Nokia | Option 1 | In NE-DC, as MN can have FR1 and FR2 cell deployed, MN can configure all kinds of gap type to UE, so Option1 is fine Option2 is not reasonable, Option3 has collision issue if both MN and SN configures FR1 gap simultaneously. |
| Intel | Option 3 | General principle of EN-DC can be applied. In addition, MN may be need to configure FR1 gap if deploy in FR1. |
| Apple | Option 1/3 | For FR1 gap configuration, In NW side, either MN or SN can provide the FR1 gap configuration, and it is dependent on whether MN configures the serving cell on FR-1. In UE side, we donot see much difference to get the gap configuration from MCG or SCG configuration. But we should avoid the different FR1 gap pattern configured in MCG and SCG. |
| Samsung | Option 3 | We believe EN-DC principle should be applied. |
| NTT DOCOMO, INC. | Option 1 | We share the same view as MediaTek that there is conflict issue on Option 3. For each gap type (i.e., per-UE, FR1, FR2), only one node should be responsible for gap configuration of the gap type. |
| Huawei, HiSilicon | Option 1 | If all RRC signalling goes via SRB1, “no need to wait for gaps to be configured by the other node” seems not to really provide any benefit, while resulting in more complexity than option 1. |
| CATT | Option 1 | Option 1 is simple. The same measurement gap configuration framework can be applied to both NE-DC and NR-DC. And we share the same concern on the collision issue if both MN and SN could configure one measurement gap configuration. |

Summary of Question 2-2:

7 companies preferred option 3 while 7 companies preferred option 1. And one company listed either option 1 or option 3 is fine. We propose:

Proposal 5: Discuss which option is applied to NE-DC measurement gap/gap sharing configuration:

- Option 1: Measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

- Option 3: Per-UE gap/gap sharing configuration or per FR2 gap/gap sharing configuration is configured by MN, while per FR1 gap/gap sharing configuration can be configured by MN or SN.

For the measurement gap and gap sharing configuration in NR-DC, the following are the possible options:

Option 1: Measurement gap(s) /gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

Option 2: Per-UE gap/gap sharing configuration is configured by MN, per FR1 gap/gap sharing configuration or per FR2 gap/gap sharing configuration can be configured by MN or SN

Question 2-3: Which option above is preferred for NR-DC measurement gap/gap sharing configuration?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | Option 2 | Same reasoning as Q 2-2, i.e. the node that needs the gap first configures it and communicates to the other. |
| NEC | Option 2 | Same motivation as answer to Q2-2 |
| Qualcomm | Option 1 | Following the same justifications list in our comments for Question 2-2, we think the re-design of gap configuration is worth for both NE-DC and NR-DC and prefer a same framework for them. Since RAN2 has introduced the 3 types of gap configurations in TS38.331, We think it is straight forward to reuse the above ASN.1 structure and principle of LTE DC gap configuration, i.e. measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap. |
| vivo | Option 2 | This option is the most flexible one. |
| LG | Option 2 |  |
| OPPO | Option 2 |  |
| ZTE | Option 1 | We prefer to let MN decide the gap type and configure all types of gap pattern and gap sharing configuration. If SN needs gap, and SN has not received the gap pattern from MN, SN can send gap request to MN. Regarding Option2, it is not clear how it works? At least we have the following concerns: - Which node is responsible for gap type decision? If SN is the first node to configure gap assistant measurement, does it mean SN has the rights to determine the gap type(e.g. perUE, perFR)? - Once SRB3 is enabled, when MN and SN configure measurements at the same time, how to make sure MN and SN will not configure the same gap pattern simultaneously(e.g. one via SRB1, the other via SRB3)? In our understanding, Option2 brings more complexity to gap coordination procedure between MN and SN. |
| MediaTek | Option 1 | Prefer to use simple solution |
| Nokia | Option 2 | Different with NE-DC, both MN and SN may have cells of FR1 and FR2, not necessary to let MN configure all gap patterns for SN. But collision issue for case of both MN and SN configures FR1 gap simultaneously needs to be solved. |
| Intel | Option 2 | It is reason to allow both MN and SN to configure per FR gap since the deployment support in NR-DC could be different from EN-DC. |
| Apple | Option 2 |  |
| Samsung | Option 2 |  |
| NTT DOCOMO, INC. | Option 1 | Same reason as Q2-2. |
| Huawei, HiSilicon | Option 1 (at least for Rel-15) | We have the same remarks like ZTE on option 2, it is not clear how it works, whether MN and SN could simultaneously configure FR1 o FR2 gaps, if not how to chose which node does it… |
| CATT | Option 1 | Same as Q2-2, option 1 is simple. While for option 2, the collision issue when MN and SN configure FR1 or FR2 gap simultaneously needs to be resolved. |

Summary of Question 2-3:

9 companies preferred option 2 while 6 companies preferred option 1. For option 2, some companies also concern that collision issues needs to be resolved and be discussed further. Thus, we propose:

Proposal 6: Discuss which option is applied to NR-DC measurement gap/gap sharing configuration:

- Option 1: Measurement gap(s) /gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

- Option 2: Per-UE gap/gap sharing configuration is configured by MN, per FR1 gap/gap sharing configuration or per FR2 gap/gap sharing configuration can be configured by MN or SN.

- Measurement gap coordination

The following are the descriptions on measurement gap coordination in EN-DC [2].

The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN. For the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE.

The key points of current measurement gap coordination are summarized as below:

- MN->SN:

- the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1);

- the list of MN configured frequencies in FR2 measured by the UE for the case of per FR2 gap;

- SN->MN:

- the list of SN configured frequencies in FR1 and FR2 measured by the UE for the case of per-UE gap;

- the list of SN configured frequencies in FR1 measured by the UE for the case of per FR gap.

The architecture of NG-EN DC is similar as EN-DC. Hence, we have the following question:

Question 2-4: For the bullets on measurement gap coordination in EN-DC, can these be applicable in NG-EN DC? Furthermore, please list the non-applicable bullets and any other features, if any.

| Company | Applicable bullets(e.g. a, b, )? | Non-applicable bullets and reason, or any other features can be considered? |
| --- | --- | --- |
| Ericsson | All | NGEN-DC can adopt all EN-DC measurement principles. |
| NEC | All |  |
| Qualcomm | All bullets |  |
| vivo | All |  |
| LG | All |  |
| OPPO | All |  |
| ZTE | All |  |
| MediaTek | All |  |
| Nokia | All | NGEN-DC can use same mechanism as EN-DC |
| Intel | All | Same principle can be applied. |
| Apple | All |  |
| Samsung | All |  |
| NTT DOCOMO, INC. | All |  |
| Huawei, HiSilicon | All |  |
| CATT | All |  |

Summary of Question 2-4:

All companies confirmed all bullets on measurement gap coordination in EN-DC are applicable to NG-EN DC.

Proposal 7: Descriptions on measurement gap coordination for EN-DC in TS 37.340 are applicable to NG-EN DC.

The node which configures measurement gap in NE-DC and NR-DC may be different from EN-DC. Hence, we have the following question:

Question 2-5: For measurement gap coordination of NE-DC, which information needs to be exchanged from MN->SN and SN->MN?

| Company | Info exchanged ( including MN->SN and SN->MN) | Comments |
| --- | --- | --- |
| Ericsson | MN-> SN: - per UE or FR1 or FR2 gap configuration SN -> MN: - list of SN configured frequencies in FR1/2 for the sake of per UE gap - list of SN configured frequencies in FR2 for the sake of per FR gap - FR1 gap configuration | We have listed the essential information needed, but additional information can also be communicated for optimal gap configuration (e.g. SN -> MN list of FR1 frequencies configured by SN, if the gap FR1 was set by MN) |
| NEC | MN-> SN: - per UE or FR1 gap configuration (gap pattern and purpose) - list of MN configured frequencies in FR2 for FR2 gap SN-> MN: - list of SN configured frequencies in FR1 for per-UE gap - FR1 gap configuration for per-FR gap | For MN->SN, we do not include FR2 gap configuration to align with SN->MN info (i.e. CG-Config) in EN-DC. But fine for discussing whether to include it or not. |
| Qualcomm | MN->SN: - the configured per-UE or FR1 measurement gap pattern, the gap purpose (per-UE or per-FR1) is NOT required; SN->MN: - whether to perform inter-frequency measurement for both cases of per-UE gap and per-FR gap; | MN->SN: - As shown in our comments to Question 2-2, we prefer all gaps (per-UE/FR1/FR2) are configured by MN. Therefore, the list of MN configured frequencies in FR2 measured by the UE is not required to be exchanged with SN - In NE-DC, LTE works as SN and therefore SeNB can’t schedule data transmission in FR2. So, Gap purpose is (per-UE or FR1) also not required to be forwarded to SN SN->MN: - SN just needs to indicate MN whether to perform inter-frequency measurement so that MN can determine the suitable gap type and pattern with SN’s measurement requirement - In our opinion, the list of SN configured frequencies is not needed because LTE’s CRS has fixed periodicity and inter-RAT RRM should not be supported in SN as we answered in Question 1-2. But since it is inter-node msg, infra-vendors are in better position to answer this question. If most companies wanted to exchange SN configured all frequencies list to have a unified framework for both NE-DC and NR-DC (i.e. our comments to Question 2-6), it is also acceptable to us. |
| vivo | MN-> SN: - per UE or FR1 or FR2 gap configuration SN -> MN: - FR1 gap configuration | Only the gap information is sufficient, another node can justify whether the additional new FR gap is needed. |
| LG | MN->SN: - The configured per-UE or FR1 or FR2 gap pattern. SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE for the case of per-UE gap; - the list of SN configured frequencies in FR2 measured by the UE for the case of per FR gap. - The configured FR1 gap pattern. |  |
| OPPO | MN->SN: the configured per UE gap or per FR1 gap or per FR2 gap; SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE for the case of per-UE gap; - The list of SN configured frequencies in FR2 measured by the UE for the case of per FR gap. - The configured FR1 gap pattern. |  |
| ZTE | MN->SN: - Only the configured perUE or FR1 gap pattern. SN->MN: - Just gap request without frequency list. | For MN->SN, gap purpose is not needed, because both perUE and FR1 gap influence the scheduling of all SN serving cells. Frequency list is not needed because SN is not responsible for generating gap patterns. For SN->MN, if SN is the first node to configure LTE inter-freq measurement, and SN has not received any gap pattern from MN, SN can send an gap request to MN as gap request. In this case, the detail measured frequency list is not needed due to the periodicity of LTE CRS is fixed 5ms. |
| MediaTek | MN->SN: - the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR); SN->MN: - the list of SN configured frequencies in FR1 measured by the UE. | This depends on Question 2-2, we assume that option 1 is adopted for it. |
| Nokia | Up to option selection for Question 2-2 | For Option1 selected for Question 2-2, there is no new ASN.1 change to Inter-Node message that is asked for NE-DC supporting. For Option3 selected for Question 2-2, it needs adding per FR gap pattern in CG-Config from SN to MN. |
| Intel | MN->SN: - the configured per-UE or FR1 measurement gap pattern - the list of MN configured frequencies in FR2 measured by the UE for the case of per FR2 gap SN->MN: - the list of SN configured frequencies in FR1/FR2 measured - FR gap cofiguration |  |
| Apple | MN->SN: - the configured per-UE or FR1 measurement gap pattern; SN->MN: - list of SN configured frequencies in FR1 measured by the UE; - FR1 gap configuration | We donot see the motivation for LTE SN to perform inter-RAT measurement on FR2. |
| Samsung | MN- to SN: measured frequencies, MN configured gap, purpose (perUE, perFR1) SN- to MN: measured frequencies, list of serving cells with for each (FR1, FR2) | It would be good to list the info exchanged in case of EN-DC and then discuss what is required in other cases. |
| NTT DOCOMO, INC. | MN -> SN: - the configured per-UE or FR1 measurement gap pattern SN -> MN: - the list of SN configured frequencies in FR1 and FR2 | MN -> SN: The configured gap pattern should be informed since that information should be taken into account for data scheduling on SN. Gap purpose (per-UE or per-FR1) is not needed for SN since both perUE and FR1 gap influence the scheduling of all SN serving cells. SN -> MN: If we assume Option 1 on Q2-2, SN should inform the SN configured frequencies (FR1 and FR2) to let MN provide the measurement gap configuration. |
| Huawei, HiSilicon | MN->SN - per UE or or FR1 gap configuration SN-> MN: - list of SN configured frequencies |  |
| CATT | MN->SN: - the configured per-UE or FR1 measurement gap pattern; SN->MN: - the list of SN configured frequencies; | MN->SN: As the SN(LTE) will not work on FR2 serving frequencies, there is no need to transfer the configured FR2 to the SN. In EN-DC, the gap purpose (per-UE or per-FR1) is exchanged from MN to SN as the SN may configure per FR2 measurement gap if per-FR1 gap (not per UE gap) is configured by MN. However, as we propose in Q2-2, the SN doesn’t configure any measurement gap for NE-DC case. Hence, there is no need to transfer the measurement gap purpose to the SN in NE-DC. SN->MN: Align with NR-DC case. |

Summary of Question 2-5:

Question 2-5 is depended on the option selection of Question 2-2. Hence, we conclude some possible options and suggest RAN2 to discuss further:

Proposal 8: Discuss which option is applied to measurement gap coordination for NE-DC:

- Option 1:

MN-> SN: per UE or FR1 or FR2 gap configuration (gap and gap purpose)

SN -> MN:

- list of SN configured frequencies for the sake of per UE gap or per FR1 gap

- FR1 gap configuration

- Option 2:

MN->SN: per UE or FR1 gap pattern (gap purpose may also be provided)

SN->MN: the list of SN configured frequencies or just gap request without frequency list

Question 2-6: For measurement gap coordination of NR-DC, which information needs to be exchanged from MN->SN and SN->MN?

| Company | Info exchanged ( including MN->SN and SN->MN) | Comments |
| --- | --- | --- |
| Ericsson | MN-> SN: - per UE or FR1 or FR2 gap configuration SN -> MN: - list of SN configured frequencies in FR1/2 for the sake of per UE gap - FR1 or FR2 gap configuration | Same as Question 2-5 |
| NEC | MN-> SN: - per-UE, FR1 or FR2 gap configuration (gap pattern and purpose) SN-> MN: - list of SN configured frequencies in FR1/2 for per-UE gap - FR1, FR2 gap configuration for per-FR gap |  |
| Qualcomm | MN->SN: - the configured per-UE, FR1 or FR2 measurement gap pattern and the gap purpose (per-UE or per-FR1 or per-FR2); SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE for both cases of per-UE gap and per-FR gap; | MN->SN: - As shown in our comments to Question 2-3, we prefer all gaps (per-UE/FR1/FR2) are configured by MN. Therefore, the list of MN configured frequencies measured by the UE is not required to be exchanged with SN. - In NR-DC, NR works as SN and therefore SgNB may schedule data transmission in FR1 or FR2. So, Gap purpose is (per-UE or per-FR1 or per-FR2) required to be forwarded to SN SN->MN: - In NR-DC, NR works as SN and therefore the list of SN configured frequencies in FR1 and FR2 measured by the UE are required for both the cases of per-UE gap and per-FR gap. |
| vivo | MN-> SN: - per UE or FR1 or FR2 gap configuration SN -> MN: - FR1/FR2 gap configuration | Only the gap information is sufficient, another node can justify whether the additional new FR gap is needed. |
| LG | MN->SN: - The configured per-UE or FR1 or FR2 gap pattern. SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE for the case of per-UE gap; - The configured FR1 or FR2 gap pattern. |  |
| OPPO | the configured per UE gap or per FR1 gap or per FR2 gap; SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE for the case of per-UE gap; - The list of SN configured frequencies in FR1 or FR2 measured by the UE for the case of per FR2 or FR1 gap. - The configured FR1 or FR2 gap pattern. | For NR-DC case, if the per RF gap is configured, the MN and SN will not configure the same type of gap. |
| ZTE | MN->SN: - the configured per-UE or FR1 or FR2 measurement gap pattern and the gap purpose (“per-UE” or “per-FR1 and/or per-FR2”). SN->MN: - A list of SN configured frequencies in FR1 and FR2 measured by the UE. | Since we proposed to let MN configure all gap patterns, for MN->SN, MN has to deliver the configured gap pattern and gap purpose to SN if configured. Notice that FR1 gap and FR2 gap can be transmitted at the same time, at least the value range of gapPurpose in CG-ConfigInfo can be extended to cover this. For SN->MN, SN has to transfer all the measured NR frequencies to MN for gap generation. |
| MediaTek | MN->SN: - the configured per-UE or FR1/FR2 measurement gap pattern and the gap purpose (per-UE or per-FR); SN->MN: - the list of SN configured frequencies in FR1 and FR2 measured by the UE. | This depends on Question 2-3, we assume that option 1 is adopted for it, |
| Nokia | Up to option selection for Question 2-3 | For Option2 selected for Question 2-3, it needs adding per FR gap pattern in CG-Config from SN to MN. |
| Intel | MN->SN: - the configured per-UE or FR1 measurement gap pattern - the list of MN configured frequencies in FR2 measured by the UE for the case of per FR2 gap SN->MN: - the list of SN configured frequencies in FR1/FR2 measured - FR gap cofiguration |  |
| Apple | MN->SN: - the configured per-UE or FR1 or FR2 measurement gap pattern; - list of MN configured frequencies measured by the UE; SN->MN: - list of SN configured frequencies measured by the UE - FR1 or FR2 gap configuration |  |
| Samsung | MN- to SN: measured frequencies, MN configured gap, purpose (perUE, perFR1) SN- to MN: measured frequencies, list of serving cells with for each (FR1, FR2) | Same as Question 2-5 |
| NTT DOCOMO, INC. | MN -> SN: - the configured measurement gap pattern and the gap purpose (i.e., per-UE, per-FR1, or per-FR2) SN -> MN: the list of SN configured frequencies in FR1 and FR2 | MN -> SN: Same as NE-DC, the configured gap pattern should be informed since those information is necessary for data scheduling on SN. Gap purpose is also needed for SN in case of NR-DC since SN should know for which the gap pattern configured by MN is influenced (i.e., for FR1, FR2, or both)? SN -> MN: If we assume Option 1 on Q2-3, SN should inform the SN configured frequencies (FR1 and FR2) to let MN provide the measurement gap configuration. |
| Huawei, HiSilicon | MN->SN: - the configured per-UE or FR1 or FR2 measurement gap pattern and the gap purpose (“per-UE” or “per-FR1 and/or per-FR2”). SN->MN: - list of SN configured SSB frequencies and list of configured CSI-RS in FR1/2 for per-UE gap | Then SN also needs to provide information about configured CSI-RS based measurements, which will be different from SSB based measurements. |
| CATT | MN->SN: - the configured per-UE or per-FR1 or per-FR2 or per-FR1&per-FR2 measurement gap pattern(s); the gap purpose( per-UE, per-FR1 or per FR2). SN->MN: - the list of SN configured frequencies; | MN->SN: The MN may configure both per FR1 and per FR2 gap patterns in NR-DC. Then both per FR1 and per-FR2 measurement gap patterns need to be transfered from the MN to the SN. Furthermore, the gap purpose needs also to be provided to the SN together. Based on the gap purpose (e.g. per-FR1 or per FR2 gap), the SN can decide whether serving frquencies may be affected. SN->MN: Same as EN-DC case, the MN needs to be aware of the frequency info to configure measurement gap for NR-DC. |

Summary of Question 2-6:

Question 2-6 is depended on the option selection of Question 2-3. Hence, we conclude some possible options and suggest RAN2 to discuss further:

Proposal 9: Discuss which option is applied to measurement gap coordination for NR-DC:

- Option 1:

MN-> SN: per UE or FR1 or FR2 gap configuration (gap and gap purpose)

SN -> MN:

- list of SN configured frequencies in FR1/2 for the sake of per UE gap

- FR1 or FR2 gap configuration

- Option 2:

MN->SN: the configured per-UE or FR1 or FR2 measurement gap pattern and the gap purpose.

SN -> MN: a list of SN configured frequencies in FR1 and FR2 measured by the UE

  - Other issues

Please identify any other RAN2 issues related to MR-DC measurement and gap configuration framework that needs to be resolved in the email, and the proposed way forward.

| Company | Comments |
| --- | --- |
| Qualcomm | - For measurement gap coordination of both NE-DC and NR-DC, unlike EN-DC, the gap purpose (per-UE or FR1/FR2) is not required to be indicated to UE because existing ASN.1 of measurement gap configuration in TS38.331 had explicitly provided such info. - For SFTD measurements for both NE-DC and NR-DC, we think the report contents and associated requirements (e.g. the number of cells the UE shall be able to measure.) need to wait RAN4 input because RAN4 had indicated that they still need to discuss them in RAN4 LS (R2-1809446) (Copied from R2-1809446) SFTD measurement under NE-DC could be discussed in RAN4 later if it is considered to be necessary. SFTD measurement under NR-NR DC could be discussed in RAN4 later if NR-NR DC is agreed to be specified in Rel-15 ============================================== |
| ZTE | In RAN2 previous meeting, based on RAN4 inputs, it was agreed that UE performs FR2 gap calculation based on the SFN/Subframe of one FR2 serving cell. In NR-DC, if both MCG and SCG include FR2 serving cells. Which one should be choose for FR2 gap calculation when MCG and SCG are not synchronized? Note that RAN4 made this agreement to reduce the interruption time on FR2 serving cells, and NR-DC was not covered at that time. Maybe LS can be sent to RAN4 to get more information. |

Summary of other issues:

Qualcomm suggested that the gap purpose (per-UE or FR1/FR2) is not required to be indicated to UE because existing ASN.1 of measurement gap configuration in TS38.331 had explicitly provided such info for both NE-DC and NR-DC. As there is no description about the gap purpose configuration to UE in TS 37.340, we think this can be discussed this in other topic, e.g. stage 3 discussion on measurement gap configuration in Uu interface. In addition, Qualcomm suggested SFTD measurement under both NE-DC and NR-DC need to wait RAN4 input. Thus, we can wait for RAN4 further input for the open issue.

ZTE wondered which one should be chosen for FR2 gap calculation when MCG and SCG are not synchronized if both MCG and SCG include FR2 serving cells in NR-DC. And suggest to ask RAN4 to get more information. Considering there is no description on FR2 gap calculation in TS 37.340 and it is depended on RAN4 decision, we think RAN2 can wait for RAN4’s conclusion for this issue.

- Conclusion

Based on the discussion, we conclude:

Proposal 1: Descriptions on measurement framework in TS 37.340 are applicable to NG-EN DC, NE-DC, and NR-DC, except that SN can only configure measurements embedded in MN RRC message via SRB1 for NE-DC.

Proposal 2: Descriptions on measurement capability coordination in TS 37.340 are applicable to NG-EN DC, NE-DC and NR-DC.

Proposal 3: Descriptions on measurement result exchange between nodes in TS 37.340 are applicable to NG-EN DC, NE-DC and NR-DC, with the modification that measurement results are encoded according to SN’s RRC instead of NR RRC when they are provided by MN to SN.

Proposal 4: Descriptions on measurement gap/gap sharing configuration for EN-DC in TS 37.340 are applicable to NG-EN DC.

Proposal 5: Discuss which option is applied to NE-DC measurement gap/gap sharing configuration:

- Option 1: Measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

- Option 3: Per-UE gap/gap sharing configuration or per FR2 gap/gap sharing configuration is configured by MN, while per FR1 gap/gap sharing configuration can be configured by MN or SN.

Proposal 6: Discuss which option is applied to NR-DC measurement gap/gap sharing configuration:

- Option 1: Measurement gap(s) /gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

- Option 2: Per-UE gap/gap sharing configuration is configured by MN, per FR1 gap/gap sharing configuration or per FR2 gap/gap sharing configuration can be configured by MN or SN.

Proposal 7: Descriptions on measurement gap coordination for EN-DC in TS 37.340 are applicable to NG-EN DC.

Proposal 8: Discuss which option is applied to measurement gap coordination for NE-DC:

- Option 1:

MN-> SN: per UE or FR1 or FR2 gap configuration (gap and gap purpose)

SN -> MN:

- list of SN configured frequencies for the sake of per UE gap or per FR1 gap

- FR1 gap configuration

- Option 2:

MN->SN: per UE or FR1 gap pattern (gap purpose may also be provided)

SN->MN: the list of SN configured frequencies or just gap request without frequency list

Proposal 9: Discuss which option is applied to measurement gap coordination for NR-DC:

- Option 1:

MN-> SN: per UE or FR1 or FR2 gap configuration (gap and gap purpose)

SN -> MN:

- list of SN configured frequencies in FR1/2 for the sake of per UE gap

- FR1 or FR2 gap configuration

- Option 2:

MN->SN: the configured per-UE or FR1 or FR2 measurement gap pattern and the gap purpose

SN -> MN: a list of SN configured frequencies in FR1 and FR2 measured by the UE

- Reference

- 37.340 v15.2.0

- R2-1813234	Inclusion of measurement gap related agreements	ZTE Corporation, Sanechips CR Rel-15	37.340	15.2.0