3GPP TSG-RAN2 Meeting #103bis 	R2-1814100

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0258 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on pending RNAU on cell reselection while T302 is running |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current UE behavior upon barring alleviation when barring was triggered by reception of a wait timer T302 in RRCReject is not consistent when alleviation is caused by the stop of the timer compared to the case when barring alleviation is caused by the expiry of the timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon cell reselection while T302 is running, barring is alleviated and UE triggers a pending RNA update procedure, if any. Impact Analysis: If the UE implements the change but the NW does not, it should not be any problem. Network will process a pending RNA update in the smilar manner. If the NW implements the change but the UE does not, there will be no major impact. Network will process a NAS recovery in the same way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent UE behavior upon barring alleviation triggered by the stop of T302 (e.g. due to cell reselection) or the expiry of T302. Current specification may also require changes to the NAS specification to handle the case where lower layers indicate barring alleviation and a transition to RRC_IDLE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5.3	Connection control

### 5.3.13	RRC connection resume

#### 5.3.13.6	Cell re-selection while T319 or T302 is running

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running:

2>	if timer T319 is running:

3>	set the variable pendingRnaUpdate to 'FALSE', if that is set to TRUE;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ‘RRC Resume failure’.

2>	else:

3>	stop timer T302;