3GPP TSG-RAN WG2 Meeting #103bis											R2-1814266

Chengdu, China, 8th - 12th October 2018							 revision of R2-1811792

Source: 	vivo

Title:  	Paging enhancements for NR-U

Agenda Item:	11.2.2.1

Document for:	Discussion and Decision

# Introduction

According to the NR IDLE UE behaviours, for one UE the gNB has one transmission opportunity for the paging message for each associated SSB in one paging occasion. Due to the LBT failure, the paging transmission could be blocked in NR-U. This could cause a long delay for the paging reception. According to the RAN2#103 meeting discussion, RAN2 achieved the following agreements:

Agree to allow more paging transmission opportunities per DRX cycle for a UE in NR-U, e.g. both TDM and FDM can be considered.

In this contribution, we discuss the more details on the paging transmission/reception in NR-U.

# Discussion

## More transmission opportunities for multi-beam paging

Figure 1: Multiple paging transmission

According to the NR IDLE UE behaviours [1], “the length of one PO is one period of beam sweeping”. However the IDLE UE may only be able to receive the paging from one beam. Then for each beam, in order to increase the transmission opportunities, the PO should be able to include more than one period of beam sweeping.

Proposal 1: One PO can include more than one period of beam sweeping.

## Consideration on the power consumption for paging reception

Although increasing the transmission opportunities per DRX cycle can improve the transmission for the NR-U paging, the main drawback would be the increased UE power consumption for monitoring the paging transmission. Here we consider that if the paging message is received by the UE and the paging message does not include the UE ID in a paging DRX cycle, the UE can stop the subsequent paging monitoring in the paging DRX cycle.

Proposal 2: The UE can stop the paging monitoring within a paging DRX cycle if one paging message is received by the UE and does not include the UE ID.

# Conclusions

Based on the evaluations given above, we have the following Proposals：

Proposal 1: One PO can include more than one period of beam sweeping.

Proposal 2: The UE can stop the paging monitoring within a paging DRX cycle if one paging message is received by the UE and does not include the UE ID.

# Reference

[1] 3GPP TS 38.304, “User Equipment (UE) procedures in Idle mode and RRC Inactive state”.