3GPP TSG-RAN2 Meeting #103bis	R2-1814581

# Chengdu, China, 8th Oct 2018 - 12th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3619 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to additional SIB1 in eFeNB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2018-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to RAN1 specification, for the R15 UE that supports receiving additional SIB1 transmission, it can assume subframe #3 not containing additional SIB1 (e.g., the subframe #3 in radio frames where SIB1-NB transmission doesn’t occur) as a valid NB-IoT DL subframe when it monitors UE-specific search space. Here we can understand that not all the UEs would assume subframe #3 not containing additional SIB1 as invalid subframe. In order to keep aligned understanding for the validity of subframe #3 after additionalTransmissionSIB1 in MIB-NB is configured, RAN2 introduces the related UE capability. However, we think the scenario of initial connection establishment, e.g, the connection for attaching to core network, hasn’t been covered by the current specifications. For a R15 UE that supports additional SIB1, the UE could assume there exists dedicated transmission for it in subframe #3 not containing additional SIB1 after reception of Msg4. But for eNB, after transmission of Msg4 and before end of UE capability acquisition, the eNB would not schedule dedicated transmission on these subframe #3. This may cause that the UE cannot receive enough repetitions for the dedicated downlink transmission and finally failure to receive the downlink message. Then for example, the UE capability acquisition procedure or RRC reconfiguration procedure may fail. Therefore, we think the UE that supports additional SIB1 cannot assume by default the subframe #3 not containing additional SIB1 transmission as valid DL subframe. An explicit indication to UE would be needed. For the UE, the assumption would be confirmed only after the UE obtains the indication from the eNB. For the eNB, it can transmit such indication in RRC message, e.g, Msg4 only after it acquires the UE capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add an indication in PhysicalConfigDedicated-NB to indicate whether the subframe #3 not containing additional SystemInformationBlockType1-NB transmission can be assumed as a valid NB-IoT DL subframe. Impact Analysis Impacted functionality NPDCCH monitoring. Inter-operability If the network is implemented according to the CR and the UE is not, or vise versa, the NPDCCH tranmitted in UE-specific search space and NPDSCH scheduled by NPDCCH in the UE-specific search space will be decoded in error. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPDCCH tranmitted in UE-specific search space and NPDSCH scheduled by NPDCCH in the UE-specific search space will be decoded in error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the 1st modified section>

- PhysicalConfigDedicated-NB

The IE PhysicalConfigDedicated-NB is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated-NB information element

-- ASN1START

PhysicalConfigDedicated-NB-r13 ::=	SEQUENCE {

carrierConfigDedicated-r13			CarrierConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npdcch-ConfigDedicated-r13			NPDCCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r13			NPUSCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-r13		UplinkPowerControlDedicated-NB-r13	OPTIONAL,	-- Need ON

...,

[[	twoHARQ-ProcessesConfig-r14		ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	interferenceRandomisationConfig-r14	ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	npdcch-ConfigDedicated-v1530	NPDCCH-ConfigDedicated-NB-v1530		OPTIONAL	-- Cond TDD

]],

[[	additionalTransmissionSIB1-validSF3-v15x0	ENUMERATED {true}	OPTIONAL -- Cond additionalSIB1

]]

}

-- ASN1STOP

PhysicalConfigDedicated-NB field descriptions

carrierConfigDedicated Anchor/ non-anchor carrier used for all unicast transmissions.

interferenceRandomisationConfig For FDD: Interference randomisation enabled in connected mode, except for random access procedure in connected mode, see TS 36.211 [21]. For random access in connected mode interference randomisation on non-anchor is used and is not used on anchor carrier, see TS 36.211 [21]. For TDD: the parameter is not present.

npdcch-ConfigDedicated NPDCCH configuration.

npusch-ConfigDedicated UL unicast configuration.

twoHARQ-ProcessesConfig Activation of two HARQ processes, see TS 36.212 [22] and TS 36.213 [23].

uplink-PowerControlDedicated UL power control parameter.

additionalTransmissionSIB1-validSF3 Indicates whether the subframe #3 not containing additional SIB1 transmission can be assumed as a NB-IoT DL subframe when UE monitoring NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.Value TRUE indicates that the subframe #3 not containing additional SIB1 transmission can be assumed as a NB-IoT DL subframe. See TS 36.213 [23].

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| additionalSIB1 | This field is optionally present, need OR, if additionalTransmissionSIB1 is set to TRUE in MasterInformationBlock-NB; otherwise it is not present. |

<End of the 1st modified section>