3GPP TSG-RAN WG2 #103bis	Tdoc R2-1814773

Chengdu, China, 8th – 12th October 2018

Agenda Item:	10.3.3

Source:	Ericsson

Title:	PDCP discard timer

Document for:	Discussion, Decision

# Introduction

In RAN2#103bis potential value extensions to the PDCP discardTimer were discussed. In this contribution we analyse those timer values.

# Discussion

With the PDCP discardTimer maximum queuing time for PDCP SDUs can be limited. The timer is started when an SDU arrives at the PDCP transmit buffer, and upon expiry the PDCP PDU is discarded.

This helps queue management, and additionally limits the amount of outdated data transmitted over the air interface consuming resources redundantly. This function is even more important considering URLLC type of data, where 1) packet delivery later than a given latency target is useless and 2) very robust thus resource consuming transmissions are required. The PDCP discardTimer can therein play a key role in reducing potential resource wastage.

The PDCP discardTimer can currently be configured with the following values:

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}

With NR, latencies below 1ms are achievable considering e.g. mini-slots and higher OFDM SCS. These advancements are currently not reflected in the PDCP discardTimer value range, i.e. it seems infeasible to consider a minimum value of 10ms buffering time only for a system allowing <1ms transmission latency. Furthermore, with the achievable low latency, NR enables a range of different URLLC use-cases among which multiple come with a latency requirement in the order of a few ms, for example control-systems in the industrial automation context. For these systems, late delivery of packets beyond this latency bound is useless, i.e. those packets would anyway be discarded on application level or lead even to system failure. On the other hand, transmitting those packets would consume radio resources redundantly.

Therefore, we propose to extend the PDCP discardTimer value range considering lower values, to enable efficient discarding of otherwise redundantly transmitted data.

Extend PDCP discardTimer value range to consider at least 1ms, 2ms, 5ms.

# Conclusion

In conclusion, we propose the following:

Proposal 1	Extend PDCP discardTimer value range to consider at least 1ms, 2ms, 5ms.