3GPP TSG-RAN2 Meeting #103bis 	R2-1815380

Chengdu, China, 8th – 12th, October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG failure in NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SCG Failure handling for EN-DC is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedure description of SCGFailureInformation reporting in 5.7.x. - Add message defination and corresponding asn.1 for SCGFailureInformationEUTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCG Failure handling for EN-DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.x, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TR 36.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.7.x	EUTRA SCG failure information

#### 5.7.x.1	General

Figure 5.7.x.1-1: EUTRA SCG failure information

The purpose of this procedure is to inform NR about an SCG failure the UE has experienced, e.g. SCG radio link failure in NE-DC as specified in TS 36.331 [10, 5.6.13].

#### 5.7.x.2	Initiation

A UE initiates the procedure to report EUTRA SCG failures when EUTRA SCG transmission is not suspended and in accordance with TS 36.331 [10, 5.6.13]. Actions the UE shall perform upon initiating the procedure, other than related to the transmission of the SCGFailureInformationEUTRA message are specified in TS 36.331 [10, 5.6.13].

#### 5.7.x.3	Actions related to transmission of SCGFailureInformationEUTRA message

The UE shall set the contents of the SCGFailureInformationEUTRA message as follows:

1>	include failureType within failureReportSCG-EUTRA and set it to indicate the SCG failure in accordance with TS 36.33 [10, 5.6.13];

1>	include and set measResultSCGEUTRA in accordance with TS 36.331 [10, 5.6.13]:

1>	for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

NOTE:	Field measResultSCGEUTRA is used to report available results for EUTRA frequencies the UE is configured to measure by LTE RRC signalling.

The UE shall submit the SCGFailureInformationEUTRA message to lower layers for transmission.

END OF CHANGES

START OF CHANGES

6.3.2	Radio resource control information elements

#### –	SCGFailureInformationEUTRA

The SCGFailureInformationEUTRA message is used to provide information regarding EUTRA SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformationEUTRA message

-- ASN1START

SCGFailureInformationEUTRA-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationEUTRA-r15			SCGFailureInformationEUTRA-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationEUTRA-r15-IEs ::=	SEQUENCE {

failureReportSCG-EUTRA-r15				FailureReportSCG-EUTRA-r15			OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

FailureReportSCG-EUTRA-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t307-Expiry,

scg-ChangeFailure, scg-reconfigFailure},

measResultFreqListEUTRA-r15				MeasResultFreqListFailEUTRA-r15		OPTIONAL,

measResultSCG-EUTRA-r15						OCTET STRING						OPTIONAL,

...

}

MeasResultFreqListFailEUTRA-r15 ::=	SEQUENCE (SIZE (1..maxFreq-r15)) OF MeasResultFreqFailEUTRA-r15

MeasResultFreqFailEUTRA-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r15,

measResultCellList-r15				MeasResultListEUTRA			OPTIONAL,

...

}

-- ASN1STOP

SCGFailureInformationNR field descriptions

measResultFreqListEUTRA The field contains available results of measurements on EUTRA frequencies the UE is configured to measure by measConfig.

measResultSCG-EUTRA Includes the MeasResultSCG-Failure IE as specified in TS 36.331 [10]. The field contains available results of measurements on EUTRA frequencies the UE is configured to measure by the LTE RRCConnectionConfiguration message.

END OF CHANGES