3GPP TSG-RAN2#104	R2-1816287

# Spokane, WA, USA, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0075 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Corrections in Paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Section 7, the reference specification for the configuration of paging-SearchSpace is 38.213 which is incorrect, and it should be 38.331. The definition of the index i(s) refers to the paging DCI which is already covered by the definition of PO and thus it is better to refer to the PO |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the reference from 38.213 to 38.331 for the configuration of paging-SearchSpace. - Modify the defintion of index i(s) to refer to a PO. Impact analysis (SA) Impacted functionality: Paging Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no impact. - If the UE is implemented according to the CR and the network is not, there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specificaiton will contain wrong reference and unclear definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent [4]. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO.

In multi-beam operations, the length of one PO is one period of beam sweeping and the UE can assume that the same paging message is repeated in all beams of the sweeping pattern and thus the selection of the beam(s) for the reception of the paging message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

PF, PO are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO, is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to paging-SearchSpace and firstPDCCH-MonitoringOccasionOfPO if configured [3]. Otherwise, the PDCCH monitoring occasions for paging are determined according to the default association (i.e. PDCCH monitoring occasions for paging are same as for RMSI as defined in Section 13 in [4]).

For default association, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

For non-default association (i.e. when paging-SearchSpace is used), the UE monitors the (i_s + 1)th PO where the first PO starts in the PF. The PDCCH monitoring occasions for paging which are not overlapping with UL symbols are sequentially numbered from zero starting from the 1st PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the (i_s + 1)th PO is a set of 'S' consecutive PDCCH monitoring occasions for paging starting from the PDCCH monitoring occasion indicated by firstPDCCH-MonitoringOccasionOfPO (i.e. the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter). Otherwise, the (i_s + 1)th PO is a set of 'S' consecutive PDCCH monitoring occasions for paging starting from the (i_s * S)th  PDCCH monitoring occasion for paging where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SystemInformationBlock1. The Kth PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB.

The Kth PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB.The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value, if configured by RRC or upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by RRC or by upper layers, the default value is applied.)

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID: 5G-S-TMSI mod 1024

Parameters  N, Ns, first-PDCCH-MonitoringOccasionOfPO, PF_offset, and the length of default DRX Cycle are signaled in SIB1.

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

End of changes