3GPP TSG-RAN WG2 Meeting #104	R2-1816292

# Spokane, WA, USA, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0179 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Resume Failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RRC Resume can include cell group configuration, bearer configuration, and measurement configuration [2] and thus has similar functionality as RRC Reconfiguration. In NR, similar to LTE, there is a UE procedure for handling the RRC Reconfiguration failure cases including when the UE can’t comply with the RRC Reconfiguration. However, for RRC resume, the description for not complying with the configuration is missing. For RRC Resume, the running CR captures the failure cases when integrity check fails for RRC Resume message as well as expiration of the timer for the procedure (T319). As mentioned above, the missing part is when the UE can’t comply with the reconfiguration received in the RRC Resume. There are two main options for the UE: - Initiate RRC Re-establishment - Move to RRC_IDLE with the release cause RRC Resume failure The first option is same as RRC Reconfiguration failure while the second is same as expiration of T319. RAN2 has decided to adopt moving to Idle and rely on NAS recovery for T319 expiration since in that case the problem is due to not being able to receive a Resume message or be able to send Resume complete and thus it makes sense for the UE to delete the UE context and start from Idle mode. The same method is also used for cell reselection during Resume. Even though it was possible to send resume request again for this scenario, RAN2 decided not to complicate the Resume request procedure for this corner case scenario. The same behavior of moving to Idle can also be adopted for this scenario since it would be the simplest and in common with other failure cases during Resume |  |  |  |  |  |  |  |  |
| Summary of change: |  | When UE can’t comply with RRC Resume, it moves to Idle and rely on upper layer recovery. Impact analysis Impacted functionality: RRC Resume Inter-operability: If the network is implemented according to this CR while the UE is not, the UE behavior is undefined. If the UE is implemented according to this CR while the network is not, there could be RRC state mismatch between the UE and the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE handling of not complying with RRC Resume will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.13.9	Reception of the RRCRelease by the UE

The UE shall:

1>	perform the actions as specified in 5.3.8.

#### 5.3.13.10	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15.

#### 5.3.13.X	Inability to comply with RRCResume

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCResume message;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ‘RRC Resume failure’.

NOTE 1:	The UE may apply above failure handling also in case the RRCResume message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

END OF CHANGES