3GPP TSG-RAN WG2 Meeting #104	R2-1816614

Spokane, US, 12th - 16th Nov 2018

Agenda Item:	11.2.2.3

Souce:	Huawei, Hisicion

Title:	NR-U with SUL

Document for:	Discussion and decision

1 Introduction

RAN1 has discussed about the deployment scenarios for NR-based access to unlicensed spectrum and agreed to study the additional functionality needed beyond the specifications for operation in licensed spectrum in the deployment scenarios as listed in [1].

- Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)

- NR-U SCell may have both DL and UL, or DL-only.

- Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

- Stand-alone NR-U

- An NR cell with DL in unlicensed band and UL in licensed band

- Dual connectivity between licensed band NR (PCell) and NR-U (PSCell)

In this contribution, we discuss NR-U with SUL and provide our consideration.

2 Discussion

Based on RAN1 agreement, it was agreed that an NR-U cell can be:

- Type 1: a DL in unlicensed band and a UL in unlicensed band (optional);

- Type 2: a DL in unlicensed band and a UL licensed band.

In Rel-15, to improve coverage, SUL operation is supported. A NR cell can be configured with 2 ULs: one is normal UL and the other is SUL. The next question arises whether SUL operation can be applied to NR-U, e.g. an unlicensed SUL can be configured for type 2 NR-U cell to achieve load balance between unlicensed band and licensed band. In that case a NR-U cell (type 3) can be defined which includes a DL in unlicensed band, a UL in unlicensed band and a UL in licensed band. One example is showed in figure 1.

Figure 1: type of NR-U cell

Proposal 1: RAN2 to study the scenario on NR-U cell with one unlicensed DL, one unlicensed UL and one licensed UL.

3 Conclusion

Based on the discussions above, we have the following proposals:

Proposal 1: RAN2 to study the scenario on NR-U cell with one unlicensed DL, one unlicensed UL and one licensed UL.

4 References