3GPP TSG-RAN2 Meeting #104	R2-1816627

# Spokane, USA, November 12th – 16th 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1203 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core, LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2018-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 7.2, EDT is considered as a procedure in RRC IDLE state, which is not fully correct. There is no need to list EDT in the description of the state. In 7.3b.2, the CP-EDT is described as there is no transition to RRC CONNECTED. This is not correct since a UE initiating CP-EDT may transition to RRC CONNECTED state. In the note at the end fo 7.3b.2, “msg5” is not defined. In 7.3b.3, the UP-EDT is described as there is no transition to RRC CONNECTED. This is not correct since a UE initiating UP-EDT may transition to RRC CONNECTED state. In the note at the end fo 7.3b.3, the fallback case to legacy connection establishment is not mentioned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 7.2, remove EDT as a procedure in RRC IDLE state In 7.3b.2, replace “there is no transition to RRC CONNECTED” with “The UE initiates EDT in RRC IDLE and may be in RRC IDLE after the completion of EDT”. In 7.3b.2, change “msg5” to RRCConnectionSetupComplete. In 7.3b.3, replace “there is no transition to RRC CONNECTED” with “The UE initiates EDT in RRC IDLE and may be in RRC IDLE after the completion of EDT”. In 7.3.b.3, add the fallback case to legacy connection establishment in the note for completeness. Editorial change in 10.1.5.1. Impact Analysis: Impacted functionality: Early data transmission Inter-operability: 1. if the Network supports the changes and the UE does not: No issue with inter-operability. 2. if the UE supports the change and the network does not: No issue with inter-operability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the UE does not implement the change, confusion and ambiguity about EDT feature exist in this specification and among relevant specifications, e.g., TS36.331, TS36.321. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.3b.2, 7.3b.3, 10.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

## 7.2	RRC protocol states & state transitions

RRC uses the following states:

-	RRC_IDLE:

-	PLMN selection;

-	DRX configured by NAS;

-	Broadcast of system information;

-	Paging;

-	Cell re-selection mobility;

-	The UE shall have been allocated an id which uniquely identifies the UE in a tracking area;

-	No RRC context stored in the eNB (except for a UE that supports User Plane CIoT EPS optimizations [20] where a context may be stored for the resume procedure);

-	Sidelink communication transmission and reception;

-	Sidelink discovery announcement and monitoring;

-	V2X sidelink communication transmission and reception.

-	RRC_CONNECTED:

-	UE has an E-UTRAN-RRC connection;

-	UE has context in E-UTRAN;

-	E-UTRAN knows the cell which the UE belongs to;

-	Network can transmit and/or receive data to/from UE;

-	Network controlled mobility (handover and inter-RAT cell change order to GERAN with NACC);

-	Neighbour cell measurements;

-	Sidelink communication transmission and reception;

-	Sidelink discovery announcement and monitoring;

-	V2X sidelink communication transmission and reception;

-	At PDCP/RLC/MAC level:

-	UE can transmit and/or receive data to/from network;

-	UE monitors control signalling channel for shared data channel to see if any transmission over the shared data channel has been allocated to the UE;

-	UE also reports channel quality information and feedback information to eNB;

-	DRX period can be configured according to UE activity level for UE power saving and efficient resource utilization. This is under control of the eNB.

E-UTRA connected to 5GC additionally supports RRC_INACTIVE state which can be characterised as follows:

-	RRC_INACTIVE:

-	PLMN selection;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Monitors a Paging channel for CN paging and RAN paging;

-	RAN-based notification area (RNA) is configured by NG-RAN;

-	DRX for RAN paging configured by NG-RAN;

-	5GC-NG-RAN connection (both C/U-planes) is established for UE;

-	The UE AS context is stored in NG-RAN and the UE;

-	NG-RAN knows the RNA which the UE belongs to.

Next change

## 7.3b	EDT

### 7.3b.1	General

EDT allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure.

EDT is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data (i.e., not signalling or SMS) and the uplink data size is less than or equal to a TB size indicated in the system information. EDT is not used for data over the control plane when using the User Plane CIoT EPS optimizations.

EDT is only applicable to BL UEs, UEs in Enhanced Coverage and NB-IoT UEs.

### 7.3b.2	EDT for Control Plane CIoT EPS optimizations

EDT for Control Plane CIoT EPS optimizations, as defined in TS 24.301 [20], is characterized as below:

-	Uplink user data are transmitted in a NAS message concatenated in UL RRCEarlyDataRequest message on CCCH;

-	Downlink user data are optionally transmitted in a NAS message concatenated in DL RRCEarlyDataComplete message on CCCH;

-		The UE initiates EDT in RRC IDLE and may be in RRC IDLE after the completion of EDT.

The EDT procedure for Control Plane CIoT EPS optimizations is illustrated in Figure 7.3b-1.

Figure 7.3b-1: EDT for Control Plane CIoT EPS Optimizations

0.	Upon connection establishment request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	UE sends RRCEarlyDataRequest message concatenating the user data on CCCH.

2.	The eNB initiates the S1-AP Initial UE message procedure to forward the NAS message and establish the S1 connection. The eNB may indicate in this procedure that this connection is triggered for EDT.

3.	The MME requests the S-GW to re-activate the EPS bearers for the UE.

4.	The MME sends the uplink data to the S-GW.

5.	If downlink data are available, the S-GW sends the downlink data to the MME.

6.	If downlink data are received from the S-GW, the MME forwards the data to the eNB via DL NAS Transport procedure and may also indicate whether further data are expected. Otherwise, the MME may trigger Connection Establishment Indication procedure and also indicate whether further data are expected.

7.	If no further data are expected, the eNB can send the RRCEarlyDataComplete message on CCCH to keep the UE in RRC_IDLE. If downlink data were received in step 6, they are concatenated in RRCEarlyDataComplete message.

8.	The S1 connection is released and the EPS bearers are deactivated.

NOTE:	If the MME or the eNB decides to move the UE in RRC_CONNECTED mode, RRCConnectionSetup message is sent in step 7 to fall back to the legacy RRC Connection establishment procedure; the eNB will discard the zero-length NAS PDU received inRRCConnectionSetupComplete message.

### 7.3b.3	EDT for User Plane CIoT EPS optimizations

EDT for User Plane CIoT EPS optimizations, as defined in TS 24.301 [20], is characterized as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

-	Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	 The UE initiates EDT in RRC IDLE and may be in RRC IDLE after the completion of EDT.

The EDT procedure for User Plane CIoT EPS optimizations is illustrated in Figure 7.3b-2.

Figure 7.3b-2: EDT for User Plane CIoT EPS Optimizations

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected from the S-GW, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH.

NOTE:	If the MME or eNB decides the UE to move in RRC_CONNECTED mode, RRCConnectionResume message is sent in step 7 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 7 to fall back to the RRC Connection establishment procedure.

Next change

#### 10.1.5.1	Contention based random access procedure

The contention based random access procedure is outlined on Figure 10.1.5.1-1 below:

Figure 10.1.5.1-1: Contention based Random Access Procedure

The four steps of the contention based random access procedures are:

1)	Random Access Preamble on RACH in uplink:

-	There are two possible groups defined and one is optional. If both groups are configured the size of message 3 and the pathloss are used to determine which group a preamble is selected from. The group to which a preamble belongs provides an indication of the size of the message 3 and the radio conditions at the UE. The preamble group information along with the necessary thresholds are broadcast on system information.

2)	Random Access Response generated by MAC on DL-SCH:

-	Semi-synchronous (within a flexible window of which the size is one or more TTI) with message 1;

-	No HARQ;

-	Addressed to RA-RNTI on PDCCH;

-	Conveys at least RA-preamble identifier, Timing Alignment information for the pTAG, initial UL grant and assignment of Temporary C-RNTI (which may or may not be made permanent upon Contention Resolution);

-	Intended for a variable number of UEs in one DL-SCH message.

3)	First scheduled UL transmission on UL-SCH:

-	Uses HARQ;

-	Size of the transport blocks depends on the UL grant conveyed in step 2.

-	For initial access:

-	Conveys the RRC Connection Request generated by the RRC layer and transmitted via CCCH;

-	Conveys at least NAS UE identifier but no NAS message;

-	RLC TM: no segmentation.

-	For RRC Connection Re-establishment procedure:

-	Conveys the RRC Connection Re-establishment Request generated by the RRC layer and transmitted via CCCH;

-	RLC TM: no segmentation;

-	Does not contain any NAS message.

-	After handover, in the target cell:

-	Conveys the ciphered and integrity protected RRC Handover Confirm generated by the RRC layer and transmitted via DCCH;

-	Conveys the C-RNTI of the UE (which was allocated via the Handover Command);

-	Includes an uplink Buffer Status Report when possible.

-	For other events:

-	Conveys at least the C-RNTI of the UE;

-	In the procedure to resume the RRC connection or in the EDT procedure for User Plane CIoT EPS Optimizations:

-	Conveys the RRC Connection Resume Request generated by the RRC layer and transmitted via CCCH;

-	Conveys a Resume ID to resume the RRC connection;

-	For NB-IoT:

-	In the procedure to setup the RRC connection:

-	An indication of the amount of data for subsequent transmission(s) on SRB or DRB can be indicated.

-	For EDT for Control Plane CIoT EPS Optimizations:

-	Conveys the RRC Early Data Request generated by the RRC layer and transmitted via CCCH;

-	Conveys NAS UE identifier and user data concatenated in a NAS message.

-	Conveys ciphered user data transmitted via DTCH.

4)	Contention Resolution on DL:

-	Early contention resolution shall be used i.e. eNB does not wait for NAS reply before resolving contention;

-	For NB-IoT, for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure, eNB may transmit MAC PDU containing the UE contention resolution identity MAC control element without RRC response message;

NOTE:	In Release 13, NB-IoT UEs do not support the MAC PDU containing the UE contention resolution identity MAC control element without RRC response message for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure.

-	Not synchronised with message 3;

-	HARQ is supported;

-	Addressed to:

-	The Temporary C-RNTI on PDCCH for initial access and after radio link failure;

-	The C-RNTI on PDCCH for UE in RRC_CONNECTED.

-	HARQ feedback is transmitted only by the UE which detects its own UE identity, as provided in message 3, echoed in the Contention Resolution message;

-	For initial access, RRC Connection Re-establishment procedure and EDT for Control Plane CIoT EPS Optimizations, no segmentation is used (RLC-TM).

The Temporary C-RNTI is promoted to C-RNTI for a UE which detects RA success and does not already have a C-RNTI; it is dropped by others. A UE which detects RA success and already has a C-RNTI, resumes using its C-RNTI.

When CA is configured, the first three steps of the contention based random access procedures occur on the PCell while contention resolution (step 4) can be cross-scheduled by the PCell.

When DC is configured, the first three steps of the contention based random access procedures occur on the PCell in MCG and PSCell in SCG. When CA is configured in SCG, the first three steps of the contention based random access procedures occur on the PSCell while contention resolution (step 4) can be cross-scheduled by the PSCell.

End of changes