3GPP TSG-RAN WG2 Meeting #104 	                           R2-1816671

Spokane, USA, 12th – 16th Nov 2018

Source: 			ZTE Corporation, Sanechips

Title: 	Bandwidth of initial BWP in Option2 operation

Agenda item:		10.4.1.3.1.1

Document for: 	Discussion and Decision

- Introduction

During last RAN2 meeting, the following agreement were made for Option2 operation.

Agreements

1	Capture in the field description of the initialDownlinkBWP in DownlinkConfigCommonSIB that “The network configures the  locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.”

2	Capture in the field description of the commonControlResourceSet that “The network configures the commonControlResourceSet in SIB1 so that it is confined in the bandwidth of CORESET#0”.

3	Capture in the field description of initialDownlinkBWP that “The  locationAndBandwidth is applicable after reception of Msg4.”

4	The rules how the UE determines the DCI sizes based on CORESET#0 are not captured in the RRC specification since this is clearly the scope of the L1 specs.

5	The problem that the UE is not configured with CORESET#0 should be resolved by RAN1 and captured in their specs, e.g. UE determines DCI 1-0 size by CORESET#0 if configured and by the width of the initial DL BWP otherwise.

For BWP Option2 operation, the larger bandwidth of initial BWP will be indicated in SIB1, and it is applicable right after reception of Msg4. In this contribution, we further discussed the remaining issues and shared our views.

- Discussion

For 6-1 feature UEs, if network wants to configure a larger operating bandwidth towards UE at the beginning, and Option2 operation is selected in this case, network can indicate this large bandwidth in locationAndBandwith of initialDownlinkBWP in SIB1.

Upon the on-going email discussion[1], companies are invited to discuss whether UE should use the SIB1 broadcast “locationAndBandwidth” for cell accessibility check, and assume the cell as “bar” when this broadcast bandwidth is beyond UE’s capability. Considering this field is included in SIB1 and is applicable to all UEs, from network perspective, the key issue is how to determine the maximum bandwidth that supported by all UEs.

In the latest TS38.101-1, RAN4 has defined a Table 5.3.5-1 to represent the supported channel bandwidths upon each SCS and operation bands(as shown in Annex1). After RAN plenary meeting, it is confirmed that the combinations(blocks) marked as “Yes” in the table should be mandatory supported by all single connection UEs that can operate on that band. This implies that, with this pre-defined table, for a specific band and SCS, network is able to know the maximum channel bandwidth supported by all UEs. For example, band77 and 60kHz capable UEs must support 100MHz channel bandwidth. Therefore, for Option2 operation, smart network will not configure the locationAndBandwidth exceed UE capability in SIB1.

Observation1: For a given band and SCS, the UE mandatory supported channel bandwidths are pre-defined in RAN4 specification. Network can based on these values to determine the maximum bandwidth of initial DL/UL BWPs in SIB1.

However, after checking TS38.101-1 Table5.5A.1-1 and Table5.5A.1-2(as shown in Annex2), the UE may not support some certain value of channel bandwidths when operating in intra-band CA. For instance, an band77 capable UE may support maximum 100MHz channel bandwidth in NR SA, but if UE only support intra-band CA with CA-bandclassNR set to “g”, the maximum bandwidth of one component carrier becomes 50MHz. Which means if UE operates on an 100MHz PCell when enters RRC_Connected, network may have to reduce the bandwidth of PCell when adding SCells.

Observation2: In case of CA, network may be required to reconfigure the bandwidth of PCell due the change of the maximum supported bandwidth.

In addition, in real deployment, different UEs may have different service requirement and data throughput, then for option2 cells, since the same bandwidth will be used for all UEs, it is hard to fulfill the requirement of all UEs. So at least we think it should be allowed for network to re-configure the locationAndBandwidth of initial BWP during RRC connection period. On the other hand, this can only be done by reconfigurationWithSync due to the locationAndBandwidth of initial BWP is only included in ServingCellConfigCommon.

Proposal 1: 	At least in BWP Option2 operation, network is allowed to re-configure the locationAndBandwidth of initial DL/UL BWP within in same cell during RRC connection period.

Comparing with BWP Option1 operation(per-UE execution), the limitation and drawback of Option2 is obvious. If proposal1 can be agreed, the follow up issue would be the mismatch between the common field in RRC dedicated signaling and the common field in system information. In any case, it should be clarified in specification that, when UE receives the common configurations from SIB1, it should not override the common field from dedicated RRC signalling during RRC connection period.

Proposal 2: 	In BWP Option2 operation(i.e. the locationAndBandwidth of initial BWP is different from CORESET#0), UE should not override the servingCellConfigCommon received from dedicated RRC signalling when receives the same field from SIB1 later.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation1: For a given band and SCS, the UE mandatory supported channel bandwidths are pre-defined in RAN4 specification. Network can based on these values to determine the maximum bandwidth of initial DL/UL BWPs in SIB1.

Observation2: In case of CA, network may be required to reconfigure the bandwidth of PCell due the change of the maximum supported bandwidth.

Proposal 1: 	At least in BWP Option2 operation, network is allowed to re-configure the locationAndBandwidth IE of initial DL/UL BWP within in same cell during RRC connection period.

Proposal 2: 	In BWP Option2 operation(i.e. the locationAndBandwidth of initial BWP is different from CORESET#0), UE should not override the servingCellConfigCommon received from dedicated RRC signalling when receives the same field from SIB1 later.

- Reference

- Email discussion [103bis#14][NR] Channel bandwidth signalling

- Annex1

TS38.101-1 f.3.0

    - 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1 Channel bandwidths for each NR band

|  | NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| n70 | 15 | Yes | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
| n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n75 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n76 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n86 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Annex2

TS38.101-1 f.3.0

  - 5.5A	Configurations for CA

    - 5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  | Aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |  |
| CA_n77C CA_n78C CA_n79C |  | 50 | 60 |  |  |  |  | 110 | 0 |
|  |  | 60 | 60 |  |  |  |  | 120 |  |
|  |  | 50 | 80 |  |  |  |  | 130 |  |
|  |  | 60 | 80 |  |  |  |  | 140 |  |
|  |  | 50 | 100 |  |  |  |  | 150 |  |
|  |  | 60 | 100 |  |  |  |  | 160 |  |
|  |  | 80 | 80 |  |  |  |  |  |  |
|  |  | 80 | 100 |  |  |  |  | 180 |  |
|  |  | 100 | 100 |  |  |  |  | 200 |  |
| CA_n77D, CA_n78D, CA_n79D |  | 50 | 60 | 100 |  |  |  | 210 |  |
|  |  | 60 | 60 | 100 |  |  |  | 220 |  |
|  |  | 50 | 80 | 100 |  |  |  | 230 |  |
|  |  | 60 | 80 | 100 |  |  |  | 240 |  |
|  |  | 50 | 100 | 100 |  |  |  | 250 |  |
|  |  | 80 | 80 | 100 |  |  |  | 260 |  |
|  |  | 80 | 90 | 100 |  |  |  | 270 |  |
|  |  | 80 | 100 | 100 |  |  |  | 280 |  |
|  |  | 90 | 100 | 100 |  |  |  | 290 |  |
|  |  | 100 | 100 | 100 |  |  |  | 300 |  |
| CA_n77E, CA_n78E, CA_n79E |  | 50 | 60 | 100 | 100 |  |  | 310 |  |
|  |  | 60 | 60 | 100 | 100 |  |  | 320 |  |
|  |  | 50 | 80 | 100 | 100 |  |  | 330 |  |
|  |  | 60 | 80 | 100 | 100 |  |  | 340 |  |
|  |  | 50 | 100 | 100 | 100 |  |  | 350 |  |
|  |  | 80 | 80 | 100 | 100 |  |  | 360 |  |
|  |  | 80 | 90 | 100 | 100 |  |  | 370 |  |
|  |  | 80 | 100 | 100 | 100 |  |  | 380 |  |
|  |  | 90 | 100 | 100 | 100 |  |  | 390 |  |
|  |  | 100 | 100 | 100 | 100 |  |  | 400 |  |

Table 5.5A.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA for fallback group 2

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  | aggregated bandwidth (MHz]) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for the other carrier (MHz |  |  |
| CA_n77F, CA_n78F, CA_n79F |  | 40 | 20 |  | 60 | 0 |
|  |  | 50 | 20 |  | 70 |  |
|  |  | 40 | 40 |  | 80 |  |
|  |  | 40 | 50 |  | 90 |  |
|  |  | 50 | 50 |  | 100 |  |
| CA_n77G, CA_n78G, CA_n79G |  | 40 | 20 | 50 | 110 |  |
|  |  | 50 | 20 | 50 | 120 |  |
|  |  | 40 | 40 | 50 | 130 |  |
|  |  | 40 | 50 | 50 | 140 |  |
|  |  | 50 | 50 | 50 | 150 |  |
| CA_n77H, CA_n78H, CA_n79H |  | 40 | 20 | 50x2 | 160 |  |
|  |  | 50 | 20 | 50x2 | 170 |  |
|  |  | 40 | 40 | 50x2 | 180 |  |
|  |  | 40 | 50 | 50x2 | 190 |  |
|  |  | 50 | 50 | 50x2 | 200 |  |
| CA_n77I, CA_n78I, CA_n79I |  | 40 | 20 | 50x3 | 210 |  |
|  |  | 50 | 20 | 50x3 | 220 |  |
|  |  | 40 | 40 | 50x3 | 230 |  |
|  |  | 40 | 50 | 50x3 | 240 |  |
|  |  | 50 | 50 | 50x3 | 250 |  |
| CA_n77J, CA_n78J, CA_n79J |  | 40 | 20 | 50x4 | 260 |  |
|  |  | 50 | 20 | 50x4 | 270 |  |
|  |  | 40 | 40 | 50x4 | 280 |  |
|  |  | 40 | 50 | 50x4 | 290 |  |
|  |  | 50 | 50 | 50x4 | 300 |  |
| CA_n77K, CA_n78K, CA_n79K |  | 40 | 20 | 50x5 | 310 |  |
|  |  | 50 | 20 | 50x5 | 320 |  |
|  |  | 40 | 40 | 50x5 | 330 |  |
|  |  | 40 | 50 | 50x5 | 340 |  |
|  |  | 50 | 50 | 50x5 | 350 |  |
| CA_n77L, CA_n78L, CA_n79L |  | 40 | 20 | 50x6 | 360 |  |
|  |  | 50 | 20 | 50x6 | 370 |  |
|  |  | 40 | 40 | 50x6 | 380 |  |
|  |  | 40 | 50 | 50x6 | 390 |  |
|  |  | 50 | 50 | 50x6 | 400 |  |