3GPP TSG-RAN WG2 Meeting #103bis	R2-1816773

Chengdu, China, 8-12 October 2018

Agenda Item:	11.2.2.2

Source:	Charter Communications Inc.

Title:	Considerations on PCI Confusion Issue

Document for: Discussion and Agreement

# Introduction

This contribution revises TP agreed at RAN2#103bis [4] wrt PCI Confusion issue, and also aims to shed some light on how to possibly tackle the issue of PCI confusion when multiple PLMNs from different operators can share the same channel and when coordination between different operators may not happen.

# Discussion

The following agreement [1] regarding PCI confusion was captured at RAN2#103bis.

In unlicensed spectrum, multiple PLMNs from different operators can share the same channel and coordination between different operators may not happen. This may cause PCI conflict and confusion for measurements

The following agreement [1] regarding PCI confusion was captured at RAN2#103bis. Although the highlighted part was added, in general an RRC_IDLE mode UE needs to read PCI to perform operations other than measurements e.g. cell (re)selection. Therefore, it is proposed to revise the TP by removing the highlighted.

Proposal 1: Change TP to “In unlicensed spectrum, multiple PLMNs from different operators can share the same channel and coordination between different operators may not happen. This may cause PCI conflict and confusion. for measurements.”

When in RRC_CONNECTED mode, UE is typically requested to perform Measurement reports. Usually, only RSRQ/RSRP/SINR for PCI-based reporting is activated. From [3]:

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-Info                                                                    OPTIONAL

]]

}

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

UE may be requested to activate CGI reporting. If done, then CGI reporting is done based on PCI(s). But, in LTE, CGI reporting isn’t typically activated by eNBs. This is because

- The same carrier is not used by more than one operator (except in MOCN cases – where deployment is coordinated)

- An eNB learns of its neighbor eNBs (and cells) via manual configuration (usually at eNB startup) and then refines it via ANR. The operation of learning new neighbours is performed less frequently once steady system state is reached.

NR is expected to follow similar reasoning during deployment phase.

Per TP above, in an unlicensed spectrum deployment, it is possible that for the same PCI there’d be multiple PLMNs which don’t coordinate apriori. Mobility-related decisions are made based on such received measurements. If the UE sends RSRP/RSRQ/SINR information for neighbour cells’ PCI, without qualifying which of the PLMN(s) the PCI belong to, then the gNB is may perform handover operation (e.g. initiate handover to a cell of a different PLMN) based on inaccurate information.

Observation 1: In case where multiple PLMNs from different operators share the same channel and apriori coordination amongst operator doesn’t occur, a gNB may make connected mode mobility decisions based on UE-reported PCI measurements belonging to a different PLMN.”

Based on Observation 1, it is proposed to capture the following TP in the TR 38.889:

Proposal 2: Capture TP: “In such a case, for a UE in RRC_CONNECTED state, it is possible for the serving gNB to make connected mode mobility decisions, for a given UE, based on the UE-reported PCI measurements belonging to a different PLMN. It is recommended to find a solution which allows a gNB to perform mobility decisions based on UE-reported measurement information belonging to PCIs of PLMN(s) or EPLMN(s) to which it belongs.”

Option 1

One approach to solving the issue is to allow the UE to report PLMN-ID corresponding to the scanned PCI. To remove any ambiguity, reporting NCGI for the given PCI may also be recommended. Armed with this knowledge, gNB can then provide the UE with an updated white/black list of PCIs belonging to PLMNs for which it doesn’t wish to receive the measurement reports. As the number of PLMNs using the same PCI, in a given area, can’t be predicted in advance (but it is assumed to be a small number), it may be beneficial for the gNB to request UE to only report measurements for a limited number of PCI(s). The high-level sequence of events would be as follows:

- (Existing behavior per NR spec) UE in RRC_CONNECTED mode is tasked to report NCGIs of PLMNs it sees for a given PCI by setting reportType to reportCGI in MeasurementReport message

- (New proposal using existing RRC IEs) Based on the received NCGIs for the given PCI(s), gNB can then update white/Black cell list for that UE such that the UE only subsequently reports RSRP/RSRQ/SINR for PCI(s) which this gNB is interested in (e.g. belonging to gNB’s PLMN, and possibly EPLMNs associated w/ serving PLMN)

- Based on the above reported information, a more informed (current or future) HO decision can be taken by the gNB

Option 2

Another approach to solving this issue could be as follows:

- UE reports PCIs in measurement report along with RSRP (as part of normal measurement configuration)

- If the network is unsure if the strongest reported PCI belongs to its PLMN, it can tell the UE to report CGI for that PCI (this is done by RRC signaling, i.e. reconfiguration of measurement with reportCGI and the UE responds with one-shot measurement report)

- UE reports CGI. Network issues handover command to the cell if the CGI is good. Otherwise, take another PCI and ask again the UE to report CGI.

There may be other approaches as well. However, the key is to have a robust solution which increases the probability of a successful (intra- or inter- frequency) handover to neighbor cell. Hence, we’re recommending investigations of the different options during WI phase.

# Conclusion

The following proposals are provided in this contribution.

Proposal 1: Change TP to “In unlicensed spectrum, multiple PLMNs from different operators can share the same channel and coordination between different operators may not happen. This may cause PCI conflict and confusion. for measurements.”

Proposal 2: Capture TP: “In such a case, for a UE in RRC_CONNECTED state, it is possible for the serving gNB to make connected mode mobility decisions, for a given UE, based on the UE-reported PCI measurements belonging to a different PLMN. It is recommended to find a solution which allows a gNB to perform mobility decisions based on UE-reported measurement information belonging to PCIs of PLMN(s) or EPLMN(s) to which it belongs.”

Corresponding text proposal is provided in Section 5.

# References

- R2-1815891, “TP for Mobility and RLM in NR-U”, Qualcomm

- R2-1813739, “The impact of multiple operators on cell selection for NR-U:, ZTE

- TS 38.331 v15.3.0, “NR Radio Resource Control (RRC) protocol specification”

- R2-186060, “Report from Session on NR Unlicensed”

# Text Proposal

### 2.3.1 RLM/RLF and Mobility

For RRM, RLM, and mobility procedures, NR licensed specification in Rel-15 are considered as a baseline for NR-U. However, changes to these due to new physical layer design and LBT for the unlicensed operation can be introduced. These will support both synchronous and, except for LAA case, asynchronous deployments.

The RRM and RLM framework for NR-U will also support multiple beam operation.

For UE measurements, it is assumed that recurring transmissions of SSB/PBCH and RMSI will be available with possibly reduced opportunities due to LBT. The NR licensed measurement framework (cell and beam quality derivation for RSRP, RSRQ, and SINR, filtering and combining multiple beams) is used as a baseline. The handling of missing measurements due to LBT are expected to be captured at physical layer specifications.

In addition to the existing measurement quantities, channel occupancy and RSSI, similar to adopted for LTE LAA, are considered useful.

In unlicensed spectrum, multiple PLMNs from different operators can share the same channel and coordination between different operators may not happen. This may cause PCI conflict and confusion. In such a case, for a UE in RRC_CONNECTED state, it is possible for the serving gNB to make connected mode mobility decisions, for a given UE, based on the UE-reported PCI measurements belonging to a different PLMN. Therefore, it is recommended to find a solution which allows a gNB to perform mobility decisions based on UE-reported measurement information belonging to PCIs of PLMN(s) or EPLMN(s) to which it belongs.