3GPP TSG-RAN WG2 Meeting #104	R2-1816813

# Spokane, Washington, USA, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0578 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of missing fields in SIB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Aligned with a previous RAN2 agreement, most fields in SIB1 are OPTIONAL, Need R, to allow for the case of an EN-DC deployment supporting ANR. However, if certain fields are omitted in a standalone deployment, the UE will be unable to use the cell; this applies to cellSelectionInfo, connEstFailureControl, servingCellConfigCommon, and ue-TimersAndConstants. If these fields are missing, the UE should have some clear behaviour to avoid using the cell, e.g. treating it as barred. - Field description for uac-AccessCategory1-SelectionAssistanceInfo is out of alphabetical order in the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The fields cellSelectionInfo, connEstFailureControl, servingCellConfigCommon, and ue-TimersAndConstants are added to the field description table, with guidance that if any of these fields is missing, the UE treats the cell as barred. The fields are changed to Need S. - Reordered uac-AccessCategory1-SelectionAssistanceInfo in the field description table. Impact analysis Impacted functionality: Reception of SIB1 Inter-operability: - if the network is implemented according to the CR and the UE is not: there is no inter-operability issue. - if the UE is implemented according to the CR and the network is not: there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE handling of a cell with missing fields from SIB1 is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                                      OPTIONAL,   -- Need R

q-RxLevMinSUL                       Q-RxLevMin                                                          OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                           OPTIONAL,   -- Need R

q-QualMinOffset                     INTEGER (1..8)                                                      OPTIONAL    -- Need R

}                                                                                                           OPTIONAL,   -- Need S

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                                   OPTIONAL,   -- Need S

si-SchedulingInfo                   SI-SchedulingInfo                                                       OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                              OPTIONAL,   -- Need S

ims-EmergencySupport                ENUMERATED {true}                                                       OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                                       OPTIONAL,   -- Cond Absent

ue-TimersAndConstants               UE-TimersAndConstants                                                   OPTIONAL,   -- Need S

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                               OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                             OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                                       OPTIONAL

}                                                                                                           OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                                       OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=	ENUMERATED {a, b, c}

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

cellSelectionInfo Parameters governing cell selection related to the serving cell. If the field is not present, the UE shall treat the cell as barred in accordance with TS 38.304 [20].

connEstFailureControl Parameters governing failure handling in connection establishment. If the field is not present, the UE shall treat the cell as barred in accordance with TS 38.304 [20].

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "QrxlevminSUL" in TS 38.304 [4], applicable for serving cell

servingCellConfigCommon Configuration of the serving cell. If the field is not present, the UE shall treat the cell as barred in accordance with TS 38.304 [20].

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in [25]. A UE compliant with this version of the specification shall ignore this field.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs, unless overwritten by the PLMN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in section 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. If the field is not present, the UE shall treat the cell as barred in accordance with TS 38.304 [20].

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses full I-RNTI and RRCResumeRequest1 if the field is present, or short I-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| Absent | The field is not used in this version of the specification, if received the UE shall ignore. |