3GPP TSG-RAN WG2 Meeting #104	R2-1817191

Spokane, USA, 12-16 November 2018	(Revision of R2-1814294)

Agenda item:	11.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RLM/RLF measurement on NR-U

WID/SID:	FS_NR_unlic – Release 16

Document for:	Discussion and Decision

# 1	Introduction

During RAN plenary #78, the release 15 NR specifications supporting licensed band operation were approved. Before that a NR Study Item [1] dealing with NR-based access to unlicensed spectrum has been approved in RAN plenary #75. One objective is to:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands both below and above 6GHz, up to 52.6GHz

    - Consider unlicensed bands above 52.6GHz to the extent that waveform design principles remain unchanged with respect to below 52.6GHz bands

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

In RAN1 #93 meeting, RLM topic was discussed and made the following agreement [2]:

Agreement:

- Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

In RAN1 #94bis meeting, the open issue for RLM and RRM was discussed and made the following agreement [3]:

Agreement:

- It is considered beneficial to configure DMTC(s) (DRS Measurement Time Configuration) in which UEs can perform measurements.

- DRS-based RRM measurements are performed inside the DMTC(s)

  - FFS: Similarity with Rel-15 SMTC

  - CSI-RS-based measurements may be performed outside the DMTC(s)

- DRS-based RLM for unlicensed SpCell is performed inside the DMTC(s)

  - RLM DMTC may coincide with DRS transmission window

  - CSI-RS-based RLM may be performed outside of DMTC(s)

- FFS: Explicit indication is provided by gNB to indicate whether or not DRS and/or CSI-RS transmissions occurred

- FFS: If DMTCs for RRM measurements and RLM are the same or can be different

In this contribution, we will discuss radio-link monitoring/failure (RLM/RLF) framework in NR-U, identify the potential impact on the RLM/RLF measurement due to the reduced transmission opportunities for RLM-RS signal due to LBT failure in NR-U, and also give some possible solutions to enhance the RLM measurement by increasing the RLM-RS transmission opportunities.

# 2	Discussion

For the purpose of monitoring downlink radio link quality to indicate out-of-sync/in-sync status indications to higher layers in LTE, the UE shall measure the reference signal (RS) and compare it to the thresholds Qout and Qin [4]. When the estimated downlink radio link quality becomes worse than threshold Qout, the physical layer in the UE shall send an out-of-sync (OOS) indication to higher layers. When the estimated downlink radio link quality becomes better than threshold Qin, the physical layer in the UE shall send an in-sync (IS) indication to higher layers. After reception of enough consecutive OOS indications, a timer is started. If no enough consecutive IS indications are received before the timer expiry, the radio link failure (RLF) is declared.

In NR licensed spectrum, both Synchronization Signal Block (SSB) and Channel Status Information Reference Signal (CSI-RS) can be used as RLM reference signals [5]. Both RLM-RS resources can be transmitted periodically at specific time instants. The UE shall monitor the downlink quality based on the SSB and/or CSI-RS, and assess the downlink radio link quality per every evaluation period. However, in NR unlicensed spectrum, the periodical transmission of RLM-RS may be interrupted due to LBT failure as shown in Fig.1. Once the gNB misses the RS transmission due to the failed LBT, it must wait until the next transmission opportunity. More seriously, the gNB may fail to access the channel for several consecutive periods. In this case, the UE may not exactly know the radio link status because the UE cannot distinguish whether the radio quality is poor or the reference signals are not transmitting due to LBT failure. In this case, if the UE always reports OOS indication to higher layer even if the channel condition is good, the inaccurate measurement result may lead to unnecessary RLF. On the other hand, if the UE always doesn’t report OOS indications when it cannot detect any of its configured RLM-RS resources, this will result in unacceptable delays in declaring RLF. Therefore, the RLM/RLF mechanism may be impacted by the possible lack of reference signals due to LBT.

- Observation 1: the RLM/RLF measurement and report may be impacted by the reduced transmission opportunity of the RLM reference signals due to LBT failure.

Figure1: Legacy periodical RLM-RS transmission in NR-U

- In RAN1#94bis [3], it was agreed that DRS-based RLM for unlicensed SpCell is performed inside the DMTC(s) (DRS Measurement Time Configuration). The presence of DRS in unlicensed spectrum can be expected because the gNB would always attempt to transmit the SSB with multiple candidate positions in the DMTC as greed in RAN1#94bis [3]. Thus, the measurements on DRS signals within the DMTC window are more reliable than the periodical RLM-RS (i.e. CSI-RS) outside of the DMTC window. Therefore, L1 samples within the DMTC window should primarily be used for both out-of-sync and in-sync evaluations.

Proposal 1: L1 samples based on DRS transmission within the DMTC window are used for both in-sync and out-of-sync evaluations.

In case the UE cannot detect any DRS transmission during the DMTC window, it cannot distinguish whether this is due to poor link quality or because the DRS transmission is blocked by LBT. Therefore, the event of the UE not detecting any DRS transmissions within the DMTC window should also be counted in the out-of-sync evaluations. How to count the missing DRS samples within the DMTC window in the out-of-sync evaluations is for RAN4 to be discussed.

Proposal 2: The event of the UE not detecting a DRS transmission within the DMTC window is also counted in the out-of-sync evaluations. How to count the missing DRS samples within the DMTC window in the out-of-sync evaluations is for RAN4 to be discussed.

In RAN1#94bis [3], it was agreed that CSI-RS-based RLM may be performed outside of DMTC(s). As by proposal 2, LBT blocking within the DMTC window may result in unnecessary declaration of RLF. To allow for faster RLM recovery in case DRS transmission within the DMTC is blocked, we propose that detected L1 samples outside the DMTC in correspondence of configured CSI-RS resources can be used in the in-sync evaluations.

Proposal 3: L1 samples outside the DMTC window are used for in-sync evaluations (upon detection of RLM-RS transmission from the gNB).

As by proposal 3, outside of the DMTC window the samples based on the detection of periodical RLM-RS (e.g. CSI-RS) can be used for the in-sync evaluations. It seems that the number of detected RLM-RS signals, which relies on the RLM-RS periodicity and the success probability of LBT, may impact the in-sync evaluation performance. The RLM-RS signals used for in-sync evaluations might not have sufficient density when the periodical RLM-RS signal could not always be transmitted periodically at the configured time resource due to the unsuccessful channel access.

Observation 2: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are always blocked by failed LBT

Further enhancements for RLM-RS signal outside of the DMTC window need to be investigated, especially for long periodicity of RLM-RS resources. For example, when the gNB could not send the RLM-RS signal for pre-defined time duration (e.g., K=2 consecutive periods), an additional opportunity window could be triggered as shown in Fig.2, where some additional occasions of RLM-RS signal transmission should be introduced. Besides the legacy periodical RLM-RS signal (i.e., CSI-RS), the opportunistic RLM-RS signal can also be extended to include additional reference signals such as DMRS (i.e., Group common DMRS or UE-Specific DMRS) in PDCCH or UE-Specific DMRS in PDSCH if they happen to be transmitted in the opportunity window.

Proposal 4: For outside the DMTC window, the RLM measurement should consider the additional opportunistic RLM-RS transmission in one triggered window if the legacy periodical RLM-RS transmission is not detected.

Figure 2: Opportunistic RLM-RS transmission in one triggered window in NR-U

In NR, the UE shall be able to evaluate the downlink quality on the configured RLM-RS resources over a certain evaluation period, which is based on RLM-RS period and the number of RLM-RS samples. However, in NR-U, considering that the DRS signal may be introduced for RLM as proposed above, the evaluation period may be redefined in terms of DMTC period and the number of DRS samples. Lower number of DRS samples will impact the accuracy of RLM. However, for a given DMTC period, the evaluation period increases as the number of DRS samples increases. The larger evaluation period may result in slow response times in the L1 sync evaluation. Therefore, the detailed number of DRS samples under various DMTC periods needs to be specified after evaluation.

Proposal 5: The definition of evaluation period should depend on the DMTC period and the number of DRS samples. The specific number of DRS samples under various DMTC periods needs to be evaluated.

It was agreed in RAN2 #102 meeting [6], multiple beam operation and related procedures should be studied. The RLM framework for NR-U should also support multiple beam operation. In NR licensed spectrum, in-sync (IS) can be indicated to higher layers if at least one beam provides sufficient link quality, i.e., above Qin threshold. On the other hand, out-of-sync (OOS) can be indicated to higher layers if all configured beams (or the number of beams within its capability) could not provide sufficient link quality, i.e., below Qout threshold. The LBT impacts have been considered in the L1 in-sync and out-of-sync evaluations for each beam. Therefore, for NR-U RLM, RAN2 could consider the multi-beam framework of NR licensed spectrum as baseline.

Proposal 6: For NR-U RLM, RAN2 could use the multi-beam framework of NR licensed spectrum as baseline.

# 3	Conclusion

The proposals and observations made in this contribution are summarized below.

- Observation 1: the RLM/RLF measurement and report may be impacted by the reduced transmission opportunity of the RLM reference signals due to LBT failure.

- Proposal 1: L1 samples based on DRS transmission within the DMTC window are used for both in-sync and out-of-sync evaluations.

Proposal 2: The event of the UE not detecting a DRS transmission within the DMTC window is also counted in the out-of-sync evaluations. How to count the missing DRS samples within the DMTC window in the out-of-sync evaluations is for RAN4 to be discussed.

Proposal 3: L1 samples outside the DMTC window are also used for in-sync evaluations (upon detection of RLM-RS transmission from the gNB).

Observation 2: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are always blocked by failed LBT

Proposal 4: For outside the DMTC window, the RLM measurement should consider the additional opportunistic RLM-RS transmission in one triggered window if the legacy periodical RLM-RS transmission is not detected.

Proposal 5: The definition of evaluation period should depend on the DMTC period and the number of DRS samples. The specific number of DRS samples under various DMTC periods needs to be evaluated.

Proposal 6: For NR-U RLM, RAN2 could use the multi-beam framework of NR licensed spectrum as baseline.

# References

- RP-172021, “New SID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- Final Report of 3GPP TSG RAN WG1 #93

- Final Report of 3GPP TSG RAN WG1 #94bis

- 3GPP TS 36.133 V15.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management”

- Final Report of 3GPP TSG RAN WG1 #91

- Final Report of 3GPP TSG RAN WG2 #102