3GPP TSG-RAN WG2#104	R2-1817215

Spokane, Washington, USA, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0612 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pending RNAU in any cell selection state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-1 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#103bis the following has been agreed: Agreements 1 If the UE enters "Any cell selection" the UE remains in RRC_INACTIVE 2 If the UE camps on an acceptable cell (i.e. “Camped on any cell” state) the UE remains in RRC_INACTIVE and then goes to IDLE when RNAU update timer expires or until the UE NAS initiates signalling (RRC also to be updated to ensure that RNAU is not triggered on acceptable cell) The agreement does not cover the case where RNA update is triggered due to barring alleviation e.g. in case there is a pending RNA update. Impact analysis: UE action is unclear if network does not implement that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We handle the case where RNAU is triggered when the UE camps on an acceptable cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour does not cover all cases where RNAU may be triggered e.g. when barring is alleviated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | User name “Ericsson-RAN2-104” is used for changes on top of the changes from IPA CR from RAN2#103bis. The text is also highlihgted to facilitate the identification. |  |  |  |  |  |  |  |  |

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, except when the UE is in “any cell selection state”, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2

2>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set to 'rna-Update';

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to 'TRUE':

3>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set to 'rna-Update'.

Upon entering “any cell selection state” while in RRC_INACTIVE state, the UE shall:

1>	if T380 expires while the UE is in “any cell selection state”; or

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4, while the UE is in “any cell selection state”;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.