3GPP TSG-RAN WG2 Meeting #104	R2-1817243

Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 36.306 | CR | 1665 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative signalling option for SupportedBandListNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specification the field en-DC-r15 indicates support of EN-DC while the supportedBandListNR-r15 is indicating vaguely the UE support for NR (whether this is standalone or including EN-DC is not clear at all). In the tradition of the TS 36.306, a 1 bit field should be matched by a corresponding list (e.g. utraFDD matched with supportedBandListUTRA-FDD, utraTDD128 matched with supportedBandListUTRA-TDD128 and many other such examples). This CR adds separate indication and list for UE supporting EN-DC and NR SA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main changes are as follows: - Rename supportedBandListNR-r15 to supportedBandListEN-DC-r15 matching en-DC-r15. - Add separate bit nr-SA-r15 and list supportedBandListNR-SA-r15 corresponding to this. Impact analysis Impacted functionality: Signalling support of NR SA. Inter-operability: - If the network is implemented according to the CR and the UE is not, the network is uncertain if the supported list of bands indicated for NR by the UE are corresponding to the support for EN-DC or for the support of NR SA. - If the UE is implemented according to the CR and the network is not, the network cannot know if the UE supports either EN-DC and/or NR SA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network remains unclear UE supports either EN-DC and/or NR SA and the supported bands corresponding to those. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 4.3.34	Inter-RAT Parameters NR

#### 4.3.34.1	en-DC-r15

This field indicates whether UE supports E-UTRA NR Dual Connectivity as specified in TS 36.331 [5] and TS 38.331 [32].

#### 4.3.34.2	supportedBandListEN-DC-r15

Only applicable if the UE supports E-UTRA NR Dual Connectivity. This field includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34].

#### 4.3.34.3	nr-SA-r15

This field indicates whether UE supports standalone NR as specified in TS 38.331 [32].

#### 4.3.34.4	supportedBandListNR-SA-r15

Only applicable if the UE supports standalone NR. This field includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34].

### 4.3.35	FeCoMP Parameters

#### 4.3.35.1	qcl-CRI-BasedCSI-Reporting-r15

This field indicates whether the UE supports CRI based CSI feedback for the FeCoMP feature as specified in TS 36.213 [23], clause 7.1.10.

#### 4.3.35.2	qcl-TypeC-Operation-r15

This field indicates the support of the following three UE features: QCL Type-C operation for FeCoMP, the capability to support separate PDSCH RE mapping for different PDSCH CWs in non-coherent joint transmission and the capability to support handling new DMRS port to MIMO layer mapping for the CWs, as specified in TS 36.213 [23], clause 7.1.10. The UE includes this field only when all three features are supported by the UE.

Next Modified Subclause