# 3GPP TSG-RAN WG2 #103bis	R2-1817289

# Chengdu, China, 8th – 12th October 2018

Title:	Draft LS updates on resume failure in RRC_INACTIVE

Release:	Release 15

Work Item:	5GS_Ph1/ NR_newRAT

Source:	Ericsson (To be RAN WG2)

To:	TSG CT1

Cc:	RAN WG3, SA WG3  TSG RAN

Contact Person:

#### Name:	Wahaj Arshad

E-mail Address:	wahaj.arshad@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In RAN2#103bis in Chengdu, RAN2 has sent LS to CT1 describing:

************************************************************************************************************************

- When the UE fails to receive a response to RRCResumeRequest during the configured timer or the UE performs cell re-selection before receiving a response from the network

- When the UE receives the RRCReject in response to RRCResumeRequest triggered by the upper layer

For those cases, RAN2 agreed the following:

- For both cases, AS layer indicates the failure to resume the RRC connection to upper layers. For case 2), notification of access barring will be provided to the upper layer in addition to the failure to resume.

- For case 1), the UE then transits to 5GMM-IDLE mode and performs the NAS signalling connection recovery and

- For case 2), the UE stays in 5GMM-CONNECTED mode with RRC Inactive indication without performing the NAS signalling connection recovery.

RAN2 identified that case 2) is currently not supported in TS24.501, but RAN2 expect the upper layer can distinguish case 2) from case 1) based on the additional notification of access barring.

************************************************************************************************************************

In RAN2#104 in Spokane the discussions were updated and RAN2 has concluded that a simpler solution would be that upon the reception of RRCReject for a request from upper layers, regardless if the UE is in RRC_INACTIVE or RRC_IDLE, AS informs NAS only that barring is applicable to all access categories except ‘0’ and ‘2’ i.e. AS does not inform NAs of a resume failure or connection establishment failure as the intended behaviour agreed in RAN2 is that the UE shall not perform state transition in these cases.

2. Actions:

To CT1 group.

ACTION: 	RAN 2 respectfully asks CT1 to confirm that the updated solution is aligned with CT1.

3. Date of Next RAN WG2 Meetings:

RAN-WG2 Meeting#105	RAN2#105, Athens, 25 Feb – 1 Mar 2019